| Signal | Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| CAL_MBUS_EN_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| CAL_MBUS_EN_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| CAL_MBUS_EN_P3V3 | Rise Time (ns) | 8.41 | ≤ 4ns | FAIL |
| CAL_MBUS_EN_P3V3 | Fall Time (ns) | 7.57 | ≤ 4ns | FAIL |
| CAL_MBUS_EN_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| CAL_MBUS_EN_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| CAL_MBUS_EN_P3V3 | Overshoot/Undershoot (V) | 0.664V (20.1%) | ≤ ±10% of 3.3V amplitude | FAIL |
| CAL_MBUS_RX_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| CAL_MBUS_RX_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| CAL_MBUS_RX_P3V3 | Rise Time (ns) | 9.96 | ≤ 4ns | FAIL |
| CAL_MBUS_RX_P3V3 | Fall Time (ns) | 8.91 | ≤ 4ns | FAIL |
| CAL_MBUS_RX_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| CAL_MBUS_RX_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| CAL_MBUS_RX_P3V3 | Overshoot/Undershoot (V) | 0.131V (4.0%) | ≤ ±10% of 3.3V amplitude | PASS |
| CAL_MBUS_RX_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| CAL_MBUS_RX_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| CAL_MBUS_RX_P3V3 | Rise Time (ns) | 9.96 | ≤ 4ns | FAIL |
| CAL_MBUS_RX_P3V3 | Fall Time (ns) | 8.91 | ≤ 4ns | FAIL |
| CAL_MBUS_RX_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| CAL_MBUS_RX_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| CAL_MBUS_RX_P3V3 | Overshoot/Undershoot (V) | 0.131V (4.0%) | ≤ ±10% of 3.3V amplitude | PASS |
| CAL_MBUS_TX_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| CAL_MBUS_TX_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| CAL_MBUS_TX_P3V3 | Rise Time (ns) | 7.23 | ≤ 4ns | FAIL |
| CAL_MBUS_TX_P3V3 | Fall Time (ns) | 6.51 | ≤ 4ns | FAIL |
| CAL_MBUS_TX_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| CAL_MBUS_TX_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| CAL_MBUS_TX_P3V3 | Overshoot/Undershoot (V) | 1.308V (39.6%) | ≤ ±10% of 3.3V amplitude | FAIL |
| CBUS_EN_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| CBUS_EN_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| CBUS_EN_P3V3 | Rise Time (ns) | 9.58 | ≤ 4ns | FAIL |
| CBUS_EN_P3V3 | Fall Time (ns) | 8.51 | ≤ 4ns | FAIL |
| CBUS_EN_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| CBUS_EN_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| CBUS_EN_P3V3 | Overshoot/Undershoot (V) | 0.217V (6.6%) | ≤ ±10% of 3.3V amplitude | PASS |
| CBUS_RX_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| CBUS_RX_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| CBUS_RX_P3V3 | Rise Time (ns) | 9.65 | ≤ 4ns | FAIL |
| CBUS_RX_P3V3 | Fall Time (ns) | 8.56 | ≤ 4ns | FAIL |
| CBUS_RX_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| CBUS_RX_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| CBUS_RX_P3V3 | Overshoot/Undershoot (V) | 0.208V (6.3%) | ≤ ±10% of 3.3V amplitude | PASS |
| CBUS_TX_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| CBUS_TX_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| CBUS_TX_P3V3 | Rise Time (ns) | 9.62 | ≤ 4ns | FAIL |
| CBUS_TX_P3V3 | Fall Time (ns) | 8.53 | ≤ 4ns | FAIL |
| CBUS_TX_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| CBUS_TX_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| CBUS_TX_P3V3 | Overshoot/Undershoot (V) | 0.217V (6.6%) | ≤ ±10% of 3.3V amplitude | PASS |
| DIR_AB_0_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| DIR_AB_0_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| DIR_AB_0_P3V3 | Rise Time (ns) | 9.16 | ≤ 4ns | FAIL |
| DIR_AB_0_P3V3 | Fall Time (ns) | 7.84 | ≤ 4ns | FAIL |
| DIR_AB_0_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| DIR_AB_0_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| DIR_AB_0_P3V3 | Overshoot/Undershoot (V) | 0.315V (9.5%) | ≤ ±10% of 3.3V amplitude | PASS |
| DIR_AB_1_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| DIR_AB_1_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| DIR_AB_1_P3V3 | Rise Time (ns) | 9.63 | ≤ 4ns | FAIL |
| DIR_AB_1_P3V3 | Fall Time (ns) | 8.54 | ≤ 4ns | FAIL |
| DIR_AB_1_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| DIR_AB_1_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| DIR_AB_1_P3V3 | Overshoot/Undershoot (V) | 0.213V (6.5%) | ≤ ±10% of 3.3V amplitude | PASS |
| DIR_AB_2_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| DIR_AB_2_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| DIR_AB_2_P3V3 | Rise Time (ns) | 9.42 | ≤ 4ns | FAIL |
| DIR_AB_2_P3V3 | Fall Time (ns) | 8.45 | ≤ 4ns | FAIL |
| DIR_AB_2_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| DIR_AB_2_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| DIR_AB_2_P3V3 | Overshoot/Undershoot (V) | 0.303V (9.2%) | ≤ ±10% of 3.3V amplitude | PASS |
| DIR_AB_3_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| DIR_AB_3_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| DIR_AB_3_P3V3 | Rise Time (ns) | 9.45 | ≤ 4ns | FAIL |
| DIR_AB_3_P3V3 | Fall Time (ns) | 8.49 | ≤ 4ns | FAIL |
| DIR_AB_3_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| DIR_AB_3_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| DIR_AB_3_P3V3 | Overshoot/Undershoot (V) | 0.280V (8.5%) | ≤ ±10% of 3.3V amplitude | PASS |
| DIR_AB_4_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| DIR_AB_4_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| DIR_AB_4_P3V3 | Rise Time (ns) | 9.42 | ≤ 4ns | FAIL |
| DIR_AB_4_P3V3 | Fall Time (ns) | 8.46 | ≤ 4ns | FAIL |
| DIR_AB_4_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| DIR_AB_4_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| DIR_AB_4_P3V3 | Overshoot/Undershoot (V) | 0.302V (9.2%) | ≤ ±10% of 3.3V amplitude | PASS |
| DIR_AB_5_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| DIR_AB_5_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| DIR_AB_5_P3V3 | Rise Time (ns) | 10.03 | ≤ 4ns | FAIL |
| DIR_AB_5_P3V3 | Fall Time (ns) | 9.00 | ≤ 4ns | FAIL |
| DIR_AB_5_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| DIR_AB_5_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| DIR_AB_5_P3V3 | Overshoot/Undershoot (V) | 0.114V (3.5%) | ≤ ±10% of 3.3V amplitude | PASS |
| DIR_AB_6_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| DIR_AB_6_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| DIR_AB_6_P3V3 | Rise Time (ns) | 9.58 | ≤ 4ns | FAIL |
| DIR_AB_6_P3V3 | Fall Time (ns) | 8.50 | ≤ 4ns | FAIL |
| DIR_AB_6_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| DIR_AB_6_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| DIR_AB_6_P3V3 | Overshoot/Undershoot (V) | 0.227V (6.9%) | ≤ ±10% of 3.3V amplitude | PASS |
| DIR_AB_7_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| DIR_AB_7_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| DIR_AB_7_P3V3 | Rise Time (ns) | 9.85 | ≤ 4ns | FAIL |
| DIR_AB_7_P3V3 | Fall Time (ns) | 8.83 | ≤ 4ns | FAIL |
| DIR_AB_7_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| DIR_AB_7_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| DIR_AB_7_P3V3 | Overshoot/Undershoot (V) | 0.167V (5.1%) | ≤ ±10% of 3.3V amplitude | PASS |
| EXT_FAN_EN_P3V3 | VOH (V) | N/A | ≥ 2.31V | N/A |
| EXT_FAN_EN_P3V3 | VOL (V) | N/A | ≤ 0.99V | N/A |
| EXT_FAN_EN_P3V3 | Rise Time (ns) | N/A | ≤ 4ns | N/A |
| EXT_FAN_EN_P3V3 | Fall Time (ns) | N/A | ≤ 4ns | N/A |
| EXT_FAN_EN_P3V3 | Noise Margin High (V) | N/A | ≥ 0.533 V | N/A |
| EXT_FAN_EN_P3V3 | Noise Margin Low (V) | N/A | ≥ 0.533 V | N/A |
| EXT_FAN_EN_P3V3 | Overshoot/Undershoot (V) | N/A | ≤ ±10% of 3.3V amplitude | N/A |
| FAN_EN_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| FAN_EN_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| FAN_EN_P3V3 | Rise Time (ns) | 9.45 | ≤ 4ns | FAIL |
| FAN_EN_P3V3 | Fall Time (ns) | 8.39 | ≤ 4ns | FAIL |
| FAN_EN_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| FAN_EN_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| FAN_EN_P3V3 | Overshoot/Undershoot (V) | 0.276V (8.4%) | ≤ ±10% of 3.3V amplitude | PASS |
| LMK_PLL_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| LMK_PLL_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| LMK_PLL_SYNC_P3V3 | Rise Time (ns) | 9.46 | ≤ 4ns | FAIL |
| LMK_PLL_SYNC_P3V3 | Fall Time (ns) | 8.45 | ≤ 4ns | FAIL |
| LMK_PLL_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| LMK_PLL_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| LMK_PLL_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.264V (8.0%) | ≤ ±10% of 3.3V amplitude | PASS |
| LMK_RST_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| LMK_RST_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| LMK_RST_P3V3 | Rise Time (ns) | 9.53 | ≤ 4ns | FAIL |
| LMK_RST_P3V3 | Fall Time (ns) | 8.45 | ≤ 4ns | FAIL |
| LMK_RST_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| LMK_RST_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| LMK_RST_P3V3 | Overshoot/Undershoot (V) | 0.245V (7.4%) | ≤ ±10% of 3.3V amplitude | PASS |
| LMK_STATUS_LD1_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| LMK_STATUS_LD1_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| LMK_STATUS_LD1_P3V3 | Rise Time (ns) | 9.76 | ≤ 4ns | FAIL |
| LMK_STATUS_LD1_P3V3 | Fall Time (ns) | 8.71 | ≤ 4ns | FAIL |
| LMK_STATUS_LD1_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| LMK_STATUS_LD1_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| LMK_STATUS_LD1_P3V3 | Overshoot/Undershoot (V) | 0.195V (5.9%) | ≤ ±10% of 3.3V amplitude | PASS |
| LMK_STATUS_LD2_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| LMK_STATUS_LD2_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| LMK_STATUS_LD2_P3V3 | Rise Time (ns) | 9.75 | ≤ 4ns | FAIL |
| LMK_STATUS_LD2_P3V3 | Fall Time (ns) | 8.69 | ≤ 4ns | FAIL |
| LMK_STATUS_LD2_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| LMK_STATUS_LD2_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| LMK_STATUS_LD2_P3V3 | Overshoot/Undershoot (V) | 0.196V (5.9%) | ≤ ±10% of 3.3V amplitude | PASS |
| MBUS_EN_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| MBUS_EN_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| MBUS_EN_P3V3 | Rise Time (ns) | 9.58 | ≤ 4ns | FAIL |
| MBUS_EN_P3V3 | Fall Time (ns) | 8.51 | ≤ 4ns | FAIL |
| MBUS_EN_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| MBUS_EN_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| MBUS_EN_P3V3 | Overshoot/Undershoot (V) | 0.218V (6.6%) | ≤ ±10% of 3.3V amplitude | PASS |
| MBUS_RX_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| MBUS_RX_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| MBUS_RX_P3V3 | Rise Time (ns) | 9.62 | ≤ 4ns | FAIL |
| MBUS_RX_P3V3 | Fall Time (ns) | 8.53 | ≤ 4ns | FAIL |
| MBUS_RX_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| MBUS_RX_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| MBUS_RX_P3V3 | Overshoot/Undershoot (V) | 0.217V (6.6%) | ≤ ±10% of 3.3V amplitude | PASS |
| MBUS_TX_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| MBUS_TX_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| MBUS_TX_P3V3 | Rise Time (ns) | 9.61 | ≤ 4ns | FAIL |
| MBUS_TX_P3V3 | Fall Time (ns) | 8.53 | ≤ 4ns | FAIL |
| MBUS_TX_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| MBUS_TX_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| MBUS_TX_P3V3 | Overshoot/Undershoot (V) | 0.220V (6.7%) | ≤ ±10% of 3.3V amplitude | PASS |
| P3V3_P1V8_MGTV_RUN | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| P3V3_P1V8_MGTV_RUN | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| P3V3_P1V8_MGTV_RUN | Rise Time (ns) | 9.47 | ≤ 4ns | FAIL |
| P3V3_P1V8_MGTV_RUN | Fall Time (ns) | 8.50 | ≤ 4ns | FAIL |
| P3V3_P1V8_MGTV_RUN | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| P3V3_P1V8_MGTV_RUN | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| P3V3_P1V8_MGTV_RUN | Overshoot/Undershoot (V) | 0.243V (7.4%) | ≤ ±10% of 3.3V amplitude | PASS |
| P48V_EN_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| P48V_EN_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| P48V_EN_P3V3 | Rise Time (ns) | 9.41 | ≤ 4ns | FAIL |
| P48V_EN_P3V3 | Fall Time (ns) | 8.27 | ≤ 4ns | FAIL |
| P48V_EN_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| P48V_EN_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| P48V_EN_P3V3 | Overshoot/Undershoot (V) | 0.300V (9.1%) | ≤ ±10% of 3.3V amplitude | PASS |
| RXLO_MBUS_EN_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| RXLO_MBUS_EN_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| RXLO_MBUS_EN_P3V3 | Rise Time (ns) | 7.79 | ≤ 4ns | FAIL |
| RXLO_MBUS_EN_P3V3 | Fall Time (ns) | 7.02 | ≤ 4ns | FAIL |
| RXLO_MBUS_EN_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| RXLO_MBUS_EN_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| RXLO_MBUS_EN_P3V3 | Overshoot/Undershoot (V) | 0.980V (29.7%) | ≤ ±10% of 3.3V amplitude | FAIL |
| RXLO_MBUS_RX_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| RXLO_MBUS_RX_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| RXLO_MBUS_RX_P3V3 | Rise Time (ns) | 9.46 | ≤ 4ns | FAIL |
| RXLO_MBUS_RX_P3V3 | Fall Time (ns) | 8.54 | ≤ 4ns | FAIL |
| RXLO_MBUS_RX_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| RXLO_MBUS_RX_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| RXLO_MBUS_RX_P3V3 | Overshoot/Undershoot (V) | 0.262V (8.0%) | ≤ ±10% of 3.3V amplitude | PASS |
| RXLO_MBUS_TX_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| RXLO_MBUS_TX_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| RXLO_MBUS_TX_P3V3 | Rise Time (ns) | 8.33 | ≤ 4ns | FAIL |
| RXLO_MBUS_TX_P3V3 | Fall Time (ns) | 7.51 | ≤ 4ns | FAIL |
| RXLO_MBUS_TX_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| RXLO_MBUS_TX_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| RXLO_MBUS_TX_P3V3 | Overshoot/Undershoot (V) | 0.703V (21.3%) | ≤ ±10% of 3.3V amplitude | FAIL |
| SYNC_EN0_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| SYNC_EN0_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| SYNC_EN0_P3V3 | Rise Time (ns) | 8.69 | ≤ 4ns | FAIL |
| SYNC_EN0_P3V3 | Fall Time (ns) | 7.61 | ≤ 4ns | FAIL |
| SYNC_EN0_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| SYNC_EN0_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| SYNC_EN0_P3V3 | Overshoot/Undershoot (V) | 0.401V (12.1%) | ≤ ±10% of 3.3V amplitude | FAIL |
| SYNC_EN1_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| SYNC_EN1_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| SYNC_EN1_P3V3 | Rise Time (ns) | 8.67 | ≤ 4ns | FAIL |
| SYNC_EN1_P3V3 | Fall Time (ns) | 7.77 | ≤ 4ns | FAIL |
| SYNC_EN1_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| SYNC_EN1_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| SYNC_EN1_P3V3 | Overshoot/Undershoot (V) | 0.516V (15.6%) | ≤ ±10% of 3.3V amplitude | FAIL |
| SYNC_EN2_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| SYNC_EN2_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| SYNC_EN2_P3V3 | Rise Time (ns) | 8.49 | ≤ 4ns | FAIL |
| SYNC_EN2_P3V3 | Fall Time (ns) | 7.38 | ≤ 4ns | FAIL |
| SYNC_EN2_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| SYNC_EN2_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| SYNC_EN2_P3V3 | Overshoot/Undershoot (V) | 0.423V (12.8%) | ≤ ±10% of 3.3V amplitude | FAIL |
| SYNC_EN3_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| SYNC_EN3_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| SYNC_EN3_P3V3 | Rise Time (ns) | 8.65 | ≤ 4ns | FAIL |
| SYNC_EN3_P3V3 | Fall Time (ns) | 7.75 | ≤ 4ns | FAIL |
| SYNC_EN3_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| SYNC_EN3_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| SYNC_EN3_P3V3 | Overshoot/Undershoot (V) | 0.529V (16.0%) | ≤ ±10% of 3.3V amplitude | FAIL |
| SYNC_EN4_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| SYNC_EN4_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| SYNC_EN4_P3V3 | Rise Time (ns) | 8.71 | ≤ 4ns | FAIL |
| SYNC_EN4_P3V3 | Fall Time (ns) | 7.64 | ≤ 4ns | FAIL |
| SYNC_EN4_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| SYNC_EN4_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| SYNC_EN4_P3V3 | Overshoot/Undershoot (V) | 0.407V (12.3%) | ≤ ±10% of 3.3V amplitude | FAIL |
| SYNC_EN5_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| SYNC_EN5_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| SYNC_EN5_P3V3 | Rise Time (ns) | 7.62 | ≤ 4ns | FAIL |
| SYNC_EN5_P3V3 | Fall Time (ns) | 6.89 | ≤ 4ns | FAIL |
| SYNC_EN5_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| SYNC_EN5_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| SYNC_EN5_P3V3 | Overshoot/Undershoot (V) | 1.150V (34.9%) | ≤ ±10% of 3.3V amplitude | FAIL |
| SYNC_EN6_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| SYNC_EN6_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| SYNC_EN6_P3V3 | Rise Time (ns) | 8.18 | ≤ 4ns | FAIL |
| SYNC_EN6_P3V3 | Fall Time (ns) | 7.37 | ≤ 4ns | FAIL |
| SYNC_EN6_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| SYNC_EN6_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| SYNC_EN6_P3V3 | Overshoot/Undershoot (V) | 0.772V (23.4%) | ≤ ±10% of 3.3V amplitude | FAIL |
| SYNC_EN7_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| SYNC_EN7_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| SYNC_EN7_P3V3 | Rise Time (ns) | 7.96 | ≤ 4ns | FAIL |
| SYNC_EN7_P3V3 | Fall Time (ns) | 7.18 | ≤ 4ns | FAIL |
| SYNC_EN7_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| SYNC_EN7_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| SYNC_EN7_P3V3 | Overshoot/Undershoot (V) | 0.915V (27.7%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TE_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TE_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TE_EXT_SYNC_P3V3 | Rise Time (ns) | 9.46 | ≤ 4ns | FAIL |
| TE_EXT_SYNC_P3V3 | Fall Time (ns) | 8.50 | ≤ 4ns | FAIL |
| TE_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TE_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TE_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.282V (8.5%) | ≤ ±10% of 3.3V amplitude | PASS |
| TR0_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR0_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR0_EXT_SYNC_P3V3 | Rise Time (ns) | 9.25 | ≤ 4ns | FAIL |
| TR0_EXT_SYNC_P3V3 | Fall Time (ns) | 8.34 | ≤ 4ns | FAIL |
| TR0_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR0_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR0_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.355V (10.8%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR1_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR1_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR1_EXT_SYNC_P3V3 | Rise Time (ns) | 8.98 | ≤ 4ns | FAIL |
| TR1_EXT_SYNC_P3V3 | Fall Time (ns) | 8.09 | ≤ 4ns | FAIL |
| TR1_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR1_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR1_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.447V (13.6%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR10_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR10_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR10_EXT_SYNC_P3V3 | Rise Time (ns) | 8.64 | ≤ 4ns | FAIL |
| TR10_EXT_SYNC_P3V3 | Fall Time (ns) | 7.78 | ≤ 4ns | FAIL |
| TR10_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR10_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR10_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.575V (17.4%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR11_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR11_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR11_EXT_SYNC_P3V3 | Rise Time (ns) | 7.00 | ≤ 4ns | FAIL |
| TR11_EXT_SYNC_P3V3 | Fall Time (ns) | 6.30 | ≤ 4ns | FAIL |
| TR11_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR11_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR11_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 1.462V (44.3%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR12_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR12_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR12_EXT_SYNC_P3V3 | Rise Time (ns) | 7.58 | ≤ 4ns | FAIL |
| TR12_EXT_SYNC_P3V3 | Fall Time (ns) | 6.82 | ≤ 4ns | FAIL |
| TR12_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR12_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR12_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 1.099V (33.3%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR13_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR13_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR13_EXT_SYNC_P3V3 | Rise Time (ns) | 8.12 | ≤ 4ns | FAIL |
| TR13_EXT_SYNC_P3V3 | Fall Time (ns) | 7.31 | ≤ 4ns | FAIL |
| TR13_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR13_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR13_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.807V (24.5%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR14_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR14_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR14_EXT_SYNC_P3V3 | Rise Time (ns) | 9.20 | ≤ 4ns | FAIL |
| TR14_EXT_SYNC_P3V3 | Fall Time (ns) | 8.29 | ≤ 4ns | FAIL |
| TR14_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR14_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR14_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.369V (11.2%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR15_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR15_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR15_EXT_SYNC_P3V3 | Rise Time (ns) | 9.49 | ≤ 4ns | FAIL |
| TR15_EXT_SYNC_P3V3 | Fall Time (ns) | 8.54 | ≤ 4ns | FAIL |
| TR15_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR15_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR15_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.279V (8.5%) | ≤ ±10% of 3.3V amplitude | PASS |
| TR16_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR16_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR16_EXT_SYNC_P3V3 | Rise Time (ns) | 9.66 | ≤ 4ns | FAIL |
| TR16_EXT_SYNC_P3V3 | Fall Time (ns) | 8.67 | ≤ 4ns | FAIL |
| TR16_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR16_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR16_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.225V (6.8%) | ≤ ±10% of 3.3V amplitude | PASS |
| TR17_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR17_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR17_EXT_SYNC_P3V3 | Rise Time (ns) | 9.56 | ≤ 4ns | FAIL |
| TR17_EXT_SYNC_P3V3 | Fall Time (ns) | 8.59 | ≤ 4ns | FAIL |
| TR17_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR17_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR17_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.258V (7.8%) | ≤ ±10% of 3.3V amplitude | PASS |
| TR18_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR18_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR18_EXT_SYNC_P3V3 | Rise Time (ns) | 9.37 | ≤ 4ns | FAIL |
| TR18_EXT_SYNC_P3V3 | Fall Time (ns) | 8.44 | ≤ 4ns | FAIL |
| TR18_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR18_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR18_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.317V (9.6%) | ≤ ±10% of 3.3V amplitude | PASS |
| TR19_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR19_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR19_EXT_SYNC_P3V3 | Rise Time (ns) | 8.93 | ≤ 4ns | FAIL |
| TR19_EXT_SYNC_P3V3 | Fall Time (ns) | 8.05 | ≤ 4ns | FAIL |
| TR19_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR19_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR19_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.460V (13.9%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR2_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR2_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR2_EXT_SYNC_P3V3 | Rise Time (ns) | 8.66 | ≤ 4ns | FAIL |
| TR2_EXT_SYNC_P3V3 | Fall Time (ns) | 7.81 | ≤ 4ns | FAIL |
| TR2_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR2_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR2_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.566V (17.1%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR20_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR20_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR20_EXT_SYNC_P3V3 | Rise Time (ns) | 9.03 | ≤ 4ns | FAIL |
| TR20_EXT_SYNC_P3V3 | Fall Time (ns) | 8.14 | ≤ 4ns | FAIL |
| TR20_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR20_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR20_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.425V (12.9%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR21_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR21_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR21_EXT_SYNC_P3V3 | Rise Time (ns) | 9.35 | ≤ 4ns | FAIL |
| TR21_EXT_SYNC_P3V3 | Fall Time (ns) | 8.42 | ≤ 4ns | FAIL |
| TR21_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR21_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR21_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.325V (9.8%) | ≤ ±10% of 3.3V amplitude | PASS |
| TR22_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR22_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR22_EXT_SYNC_P3V3 | Rise Time (ns) | 9.60 | ≤ 4ns | FAIL |
| TR22_EXT_SYNC_P3V3 | Fall Time (ns) | 8.62 | ≤ 4ns | FAIL |
| TR22_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR22_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR22_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.245V (7.4%) | ≤ ±10% of 3.3V amplitude | PASS |
| TR23_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR23_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR23_EXT_SYNC_P3V3 | Rise Time (ns) | 9.73 | ≤ 4ns | FAIL |
| TR23_EXT_SYNC_P3V3 | Fall Time (ns) | 8.73 | ≤ 4ns | FAIL |
| TR23_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR23_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR23_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.201V (6.1%) | ≤ ±10% of 3.3V amplitude | PASS |
| TR24_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR24_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR24_EXT_SYNC_P3V3 | Rise Time (ns) | 9.60 | ≤ 4ns | FAIL |
| TR24_EXT_SYNC_P3V3 | Fall Time (ns) | 8.62 | ≤ 4ns | FAIL |
| TR24_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR24_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR24_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.245V (7.4%) | ≤ ±10% of 3.3V amplitude | PASS |
| TR25_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR25_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR25_EXT_SYNC_P3V3 | Rise Time (ns) | 9.30 | ≤ 4ns | FAIL |
| TR25_EXT_SYNC_P3V3 | Fall Time (ns) | 8.38 | ≤ 4ns | FAIL |
| TR25_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR25_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR25_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.337V (10.2%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR26_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR26_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR26_EXT_SYNC_P3V3 | Rise Time (ns) | 8.83 | ≤ 4ns | FAIL |
| TR26_EXT_SYNC_P3V3 | Fall Time (ns) | 7.96 | ≤ 4ns | FAIL |
| TR26_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR26_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR26_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.500V (15.2%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR27_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR27_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR27_EXT_SYNC_P3V3 | Rise Time (ns) | 7.58 | ≤ 4ns | FAIL |
| TR27_EXT_SYNC_P3V3 | Fall Time (ns) | 6.82 | ≤ 4ns | FAIL |
| TR27_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR27_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR27_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 1.099V (33.3%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR28_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR28_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR28_EXT_SYNC_P3V3 | Rise Time (ns) | 7.78 | ≤ 4ns | FAIL |
| TR28_EXT_SYNC_P3V3 | Fall Time (ns) | 7.01 | ≤ 4ns | FAIL |
| TR28_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR28_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR28_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.984V (29.8%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR29_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR29_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR29_EXT_SYNC_P3V3 | Rise Time (ns) | 8.52 | ≤ 4ns | FAIL |
| TR29_EXT_SYNC_P3V3 | Fall Time (ns) | 7.68 | ≤ 4ns | FAIL |
| TR29_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR29_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR29_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.621V (18.8%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR3_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR3_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR3_EXT_SYNC_P3V3 | Rise Time (ns) | 7.44 | ≤ 4ns | FAIL |
| TR3_EXT_SYNC_P3V3 | Fall Time (ns) | 6.70 | ≤ 4ns | FAIL |
| TR3_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR3_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR3_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 1.183V (35.8%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR30_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR30_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR30_EXT_SYNC_P3V3 | Rise Time (ns) | 8.98 | ≤ 4ns | FAIL |
| TR30_EXT_SYNC_P3V3 | Fall Time (ns) | 8.10 | ≤ 4ns | FAIL |
| TR30_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR30_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR30_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.443V (13.4%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR31_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR31_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR31_EXT_SYNC_P3V3 | Rise Time (ns) | 9.33 | ≤ 4ns | FAIL |
| TR31_EXT_SYNC_P3V3 | Fall Time (ns) | 8.40 | ≤ 4ns | FAIL |
| TR31_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR31_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR31_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.328V (9.9%) | ≤ ±10% of 3.3V amplitude | PASS |
| TR32_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR32_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR32_EXT_SYNC_P3V3 | Rise Time (ns) | 9.11 | ≤ 4ns | FAIL |
| TR32_EXT_SYNC_P3V3 | Fall Time (ns) | 8.21 | ≤ 4ns | FAIL |
| TR32_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR32_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR32_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.401V (12.1%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR33_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR33_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR33_EXT_SYNC_P3V3 | Rise Time (ns) | 8.69 | ≤ 4ns | FAIL |
| TR33_EXT_SYNC_P3V3 | Fall Time (ns) | 7.83 | ≤ 4ns | FAIL |
| TR33_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR33_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR33_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.555V (16.8%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR34_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR34_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR34_EXT_SYNC_P3V3 | Rise Time (ns) | 8.29 | ≤ 4ns | FAIL |
| TR34_EXT_SYNC_P3V3 | Fall Time (ns) | 7.47 | ≤ 4ns | FAIL |
| TR34_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR34_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR34_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.725V (22.0%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR35_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR35_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR35_EXT_SYNC_P3V3 | Rise Time (ns) | 6.89 | ≤ 4ns | FAIL |
| TR35_EXT_SYNC_P3V3 | Fall Time (ns) | 6.21 | ≤ 4ns | FAIL |
| TR35_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR35_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR35_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 1.539V (46.6%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR36_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR36_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR36_EXT_SYNC_P3V3 | Rise Time (ns) | 7.38 | ≤ 4ns | FAIL |
| TR36_EXT_SYNC_P3V3 | Fall Time (ns) | 6.65 | ≤ 4ns | FAIL |
| TR36_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR36_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR36_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 1.213V (36.8%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR37_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR37_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR37_EXT_SYNC_P3V3 | Rise Time (ns) | 8.32 | ≤ 4ns | FAIL |
| TR37_EXT_SYNC_P3V3 | Fall Time (ns) | 7.50 | ≤ 4ns | FAIL |
| TR37_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR37_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR37_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.712V (21.6%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR38_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR38_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR38_EXT_SYNC_P3V3 | Rise Time (ns) | 8.89 | ≤ 4ns | FAIL |
| TR38_EXT_SYNC_P3V3 | Fall Time (ns) | 8.01 | ≤ 4ns | FAIL |
| TR38_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR38_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR38_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.477V (14.4%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR39_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR39_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR39_EXT_SYNC_P3V3 | Rise Time (ns) | 9.37 | ≤ 4ns | FAIL |
| TR39_EXT_SYNC_P3V3 | Fall Time (ns) | 8.44 | ≤ 4ns | FAIL |
| TR39_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR39_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR39_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.317V (9.6%) | ≤ ±10% of 3.3V amplitude | PASS |
| TR4_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR4_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR4_EXT_SYNC_P3V3 | Rise Time (ns) | 7.80 | ≤ 4ns | FAIL |
| TR4_EXT_SYNC_P3V3 | Fall Time (ns) | 7.02 | ≤ 4ns | FAIL |
| TR4_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR4_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR4_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.974V (29.5%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR40_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR40_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR40_EXT_SYNC_P3V3 | Rise Time (ns) | 7.23 | ≤ 4ns | FAIL |
| TR40_EXT_SYNC_P3V3 | Fall Time (ns) | 6.52 | ≤ 4ns | FAIL |
| TR40_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR40_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR40_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 1.307V (39.6%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR41_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR41_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR41_EXT_SYNC_P3V3 | Rise Time (ns) | 7.08 | ≤ 4ns | FAIL |
| TR41_EXT_SYNC_P3V3 | Fall Time (ns) | 6.38 | ≤ 4ns | FAIL |
| TR41_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR41_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR41_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 1.407V (42.6%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR42_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR42_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR42_EXT_SYNC_P3V3 | Rise Time (ns) | 7.27 | ≤ 4ns | FAIL |
| TR42_EXT_SYNC_P3V3 | Fall Time (ns) | 6.55 | ≤ 4ns | FAIL |
| TR42_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR42_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR42_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 1.283V (38.9%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR43_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR43_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR43_EXT_SYNC_P3V3 | Rise Time (ns) | 7.68 | ≤ 4ns | FAIL |
| TR43_EXT_SYNC_P3V3 | Fall Time (ns) | 6.92 | ≤ 4ns | FAIL |
| TR43_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR43_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR43_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 1.038V (31.5%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR44_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR44_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR44_EXT_SYNC_P3V3 | Rise Time (ns) | 7.23 | ≤ 4ns | FAIL |
| TR44_EXT_SYNC_P3V3 | Fall Time (ns) | 6.51 | ≤ 4ns | FAIL |
| TR44_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR44_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR44_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 1.311V (39.7%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR45_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR45_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR45_EXT_SYNC_P3V3 | Rise Time (ns) | 7.11 | ≤ 4ns | FAIL |
| TR45_EXT_SYNC_P3V3 | Fall Time (ns) | 6.41 | ≤ 4ns | FAIL |
| TR45_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR45_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR45_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 1.387V (42.0%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR46_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR46_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR46_EXT_SYNC_P3V3 | Rise Time (ns) | 7.21 | ≤ 4ns | FAIL |
| TR46_EXT_SYNC_P3V3 | Fall Time (ns) | 6.49 | ≤ 4ns | FAIL |
| TR46_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR46_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR46_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 1.326V (40.2%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR47_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR47_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR47_EXT_SYNC_P3V3 | Rise Time (ns) | 7.62 | ≤ 4ns | FAIL |
| TR47_EXT_SYNC_P3V3 | Fall Time (ns) | 6.86 | ≤ 4ns | FAIL |
| TR47_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR47_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR47_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 1.075V (32.6%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR48_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR48_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR48_EXT_SYNC_P3V3 | Rise Time (ns) | 8.84 | ≤ 4ns | FAIL |
| TR48_EXT_SYNC_P3V3 | Fall Time (ns) | 7.97 | ≤ 4ns | FAIL |
| TR48_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR48_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR48_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.498V (15.1%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR49_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR49_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR49_EXT_SYNC_P3V3 | Rise Time (ns) | 9.03 | ≤ 4ns | FAIL |
| TR49_EXT_SYNC_P3V3 | Fall Time (ns) | 8.14 | ≤ 4ns | FAIL |
| TR49_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR49_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR49_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.429V (13.0%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR5_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR5_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR5_EXT_SYNC_P3V3 | Rise Time (ns) | 8.53 | ≤ 4ns | FAIL |
| TR5_EXT_SYNC_P3V3 | Fall Time (ns) | 7.69 | ≤ 4ns | FAIL |
| TR5_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR5_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR5_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.621V (18.8%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR50_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR50_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR50_EXT_SYNC_P3V3 | Rise Time (ns) | 9.22 | ≤ 4ns | FAIL |
| TR50_EXT_SYNC_P3V3 | Fall Time (ns) | 8.31 | ≤ 4ns | FAIL |
| TR50_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR50_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR50_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.366V (11.1%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR51_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR51_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR51_EXT_SYNC_P3V3 | Rise Time (ns) | 9.38 | ≤ 4ns | FAIL |
| TR51_EXT_SYNC_P3V3 | Fall Time (ns) | 8.45 | ≤ 4ns | FAIL |
| TR51_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR51_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR51_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.315V (9.5%) | ≤ ±10% of 3.3V amplitude | PASS |
| TR52_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR52_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR52_EXT_SYNC_P3V3 | Rise Time (ns) | 9.36 | ≤ 4ns | FAIL |
| TR52_EXT_SYNC_P3V3 | Fall Time (ns) | 8.43 | ≤ 4ns | FAIL |
| TR52_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR52_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR52_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.322V (9.8%) | ≤ ±10% of 3.3V amplitude | PASS |
| TR53_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR53_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR53_EXT_SYNC_P3V3 | Rise Time (ns) | 9.06 | ≤ 4ns | FAIL |
| TR53_EXT_SYNC_P3V3 | Fall Time (ns) | 8.16 | ≤ 4ns | FAIL |
| TR53_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR53_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR53_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.420V (12.7%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR54_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR54_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR54_EXT_SYNC_P3V3 | Rise Time (ns) | 8.72 | ≤ 4ns | FAIL |
| TR54_EXT_SYNC_P3V3 | Fall Time (ns) | 7.86 | ≤ 4ns | FAIL |
| TR54_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR54_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR54_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.543V (16.4%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR55_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR55_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR55_EXT_SYNC_P3V3 | Rise Time (ns) | 8.34 | ≤ 4ns | FAIL |
| TR55_EXT_SYNC_P3V3 | Fall Time (ns) | 7.51 | ≤ 4ns | FAIL |
| TR55_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR55_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR55_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.704V (21.3%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR56_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR56_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR56_EXT_SYNC_P3V3 | Rise Time (ns) | 8.91 | ≤ 4ns | FAIL |
| TR56_EXT_SYNC_P3V3 | Fall Time (ns) | 8.03 | ≤ 4ns | FAIL |
| TR56_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR56_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR56_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.471V (14.3%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR57_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR57_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR57_EXT_SYNC_P3V3 | Rise Time (ns) | 8.64 | ≤ 4ns | FAIL |
| TR57_EXT_SYNC_P3V3 | Fall Time (ns) | 7.79 | ≤ 4ns | FAIL |
| TR57_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR57_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR57_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.574V (17.4%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR58_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR58_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR58_EXT_SYNC_P3V3 | Rise Time (ns) | 8.37 | ≤ 4ns | FAIL |
| TR58_EXT_SYNC_P3V3 | Fall Time (ns) | 7.54 | ≤ 4ns | FAIL |
| TR58_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR58_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR58_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.692V (21.0%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR59_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR59_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR59_EXT_SYNC_P3V3 | Rise Time (ns) | 8.06 | ≤ 4ns | FAIL |
| TR59_EXT_SYNC_P3V3 | Fall Time (ns) | 7.26 | ≤ 4ns | FAIL |
| TR59_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR59_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR59_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.834V (25.3%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR6_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR6_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR6_EXT_SYNC_P3V3 | Rise Time (ns) | 8.99 | ≤ 4ns | FAIL |
| TR6_EXT_SYNC_P3V3 | Fall Time (ns) | 8.11 | ≤ 4ns | FAIL |
| TR6_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR6_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR6_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.441V (13.4%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR60_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR60_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR60_EXT_SYNC_P3V3 | Rise Time (ns) | 8.45 | ≤ 4ns | FAIL |
| TR60_EXT_SYNC_P3V3 | Fall Time (ns) | 7.62 | ≤ 4ns | FAIL |
| TR60_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR60_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR60_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.653V (19.8%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR61_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR61_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR61_EXT_SYNC_P3V3 | Rise Time (ns) | 8.95 | ≤ 4ns | FAIL |
| TR61_EXT_SYNC_P3V3 | Fall Time (ns) | 8.06 | ≤ 4ns | FAIL |
| TR61_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR61_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR61_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.458V (13.9%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR62_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR62_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR62_EXT_SYNC_P3V3 | Rise Time (ns) | 9.28 | ≤ 4ns | FAIL |
| TR62_EXT_SYNC_P3V3 | Fall Time (ns) | 8.36 | ≤ 4ns | FAIL |
| TR62_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR62_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR62_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.346V (10.5%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR63_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR63_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR63_EXT_SYNC_P3V3 | Rise Time (ns) | 9.52 | ≤ 4ns | FAIL |
| TR63_EXT_SYNC_P3V3 | Fall Time (ns) | 8.55 | ≤ 4ns | FAIL |
| TR63_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR63_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR63_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.274V (8.3%) | ≤ ±10% of 3.3V amplitude | PASS |
| TR7_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR7_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR7_EXT_SYNC_P3V3 | Rise Time (ns) | 9.32 | ≤ 4ns | FAIL |
| TR7_EXT_SYNC_P3V3 | Fall Time (ns) | 8.40 | ≤ 4ns | FAIL |
| TR7_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR7_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR7_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.334V (10.1%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR8_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR8_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR8_EXT_SYNC_P3V3 | Rise Time (ns) | 9.17 | ≤ 4ns | FAIL |
| TR8_EXT_SYNC_P3V3 | Fall Time (ns) | 8.27 | ≤ 4ns | FAIL |
| TR8_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR8_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR8_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.379V (11.5%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TR9_EXT_SYNC_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TR9_EXT_SYNC_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TR9_EXT_SYNC_P3V3 | Rise Time (ns) | 8.80 | ≤ 4ns | FAIL |
| TR9_EXT_SYNC_P3V3 | Fall Time (ns) | 7.93 | ≤ 4ns | FAIL |
| TR9_EXT_SYNC_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TR9_EXT_SYNC_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TR9_EXT_SYNC_P3V3 | Overshoot/Undershoot (V) | 0.512V (15.5%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TXLO_MBUS_EN_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TXLO_MBUS_EN_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TXLO_MBUS_EN_P3V3 | Rise Time (ns) | 8.26 | ≤ 4ns | FAIL |
| TXLO_MBUS_EN_P3V3 | Fall Time (ns) | 7.44 | ≤ 4ns | FAIL |
| TXLO_MBUS_EN_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TXLO_MBUS_EN_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TXLO_MBUS_EN_P3V3 | Overshoot/Undershoot (V) | 0.742V (22.5%) | ≤ ±10% of 3.3V amplitude | FAIL |
| TXLO_MBUS_RX_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TXLO_MBUS_RX_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TXLO_MBUS_RX_P3V3 | Rise Time (ns) | 9.59 | ≤ 4ns | FAIL |
| TXLO_MBUS_RX_P3V3 | Fall Time (ns) | 8.66 | ≤ 4ns | FAIL |
| TXLO_MBUS_RX_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TXLO_MBUS_RX_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TXLO_MBUS_RX_P3V3 | Overshoot/Undershoot (V) | 0.224V (6.8%) | ≤ ±10% of 3.3V amplitude | PASS |
| TXLO_MBUS_TX_P3V3 | VOH (V) | 3.30 | ≥ 2.31V | PASS |
| TXLO_MBUS_TX_P3V3 | VOL (V) | 0.00 | ≤ 0.99V | PASS |
| TXLO_MBUS_TX_P3V3 | Rise Time (ns) | 8.81 | ≤ 4ns | FAIL |
| TXLO_MBUS_TX_P3V3 | Fall Time (ns) | 7.94 | ≤ 4ns | FAIL |
| TXLO_MBUS_TX_P3V3 | Noise Margin High (V) | 1.30 | ≥ 0.533 V | PASS |
| TXLO_MBUS_TX_P3V3 | Noise Margin Low (V) | 0.80 | ≥ 0.533 V | PASS |
| TXLO_MBUS_TX_P3V3 | Overshoot/Undershoot (V) | 0.496V (15.0%) | ≤ ±10% of 3.3V amplitude | FAIL |