## Required Equipment/Tools

Table 5.  Test Equipment/Hardware List

| Item # | Equipment/Tools/Parts | Part Number | Serial Number | Description |
| --- | --- | --- | --- | --- |
| T1 | Digital Multimeter |  |  | 1000V AC/DC; 10A AC/DC; resistance to 50 MΩ |
| T2 | Oscilliscope |  |  | Quad Channel O-Scope |
| T3 | Power Supply |  |  | Power Supply, capable of sourcing 100 A |
| T4 | J24 Test Harness / BOB |  |  | Test Harness providing connections and test points between the Power Supply and UUT Input Connector (J24). |
| T5 | Output Test Harness |  |  | Test Harness providing connections and test points between UUT Output Connectors (J2 – J5) and the Electronic Load. |
| T6 | UUT |  |  | Control Assy Power Module PWB |
| T7 | Electronic Load |  |  | Capable of syncing 20 A |
| T8 | Oscilliscope Probe |  |  | 100 MHz probe |

### Test Setup

## General Requirements

### Test Configuration/Flow Requirements

This design verification requires the following assembly levels and/or configurations of the UUT:

- Master Array Processor PWA

Associated Drawing(s): 13791459

Special Hardware: N/A

Description of Configuration: UUT at CCA Assembly Level

Serial Number: TBD

Test requirements shall be performed in the sequence they are called out in section 3.6.

### Input Power Characteristics

Summarized in Table 6 below are the input power interface characteristics required for the power supplies of the UUT tests outlined in this document.

Table 6.  Input Power Characteristics

| Voltage Net Name | Nom Voltage (V) | Min/Max Voltage (V) | Reference Net Name | Max Required Current (A) | Min/Max Ramp Time (V/ms) | Assembly Level/Usage Description |
| --- | --- | --- | --- | --- | --- | --- |
| P28V | 28 | 27/29 | P28V_RTN | 25 |  | Powers the UUT |

#### Grounding

The UUT has the following isolated ground domain(s):

Table 7. Grounding Table

| Signal | DC Resistance @ => | Withstanding Voltage | Max Capacitance |
| --- | --- | --- | --- |
| 28V_RTN to GNDD | open | 100V | - |
| P48V_RTN to GNDD | open | 100V | - |
| 28V_RTN to P48V_RTN | open | 100V | - |

#### Application of Power

The CCA Assembly Level test requirements outlined in this document must have power applied to the UUT in the following sequence:

- P28V

#### Power Cycling

The UUT can be power cycled by powering off the input power supply for approximately 5 seconds and then enabling power.

### Environmental Requirements

Unless otherwise specified, all tests shall be performed under Room Ambient Condition (RAC) with temperatures in the range of 15°C to 35°C and relative humidity less than 80%.

## Test Procedures

The requirements listed in the Requirements Verification Matrix (Section 3.3) are as specified in 325-RIG-001 and proceed to applicable tests described in Section 3.6. Test requirements in this document may be identified as Self-Imposed Requirements (SIR) or Good Engineering Practice (GEP). The SIR test requirements map directly to DVM items in the UUT DRS.  The following section outlines the test procedures to be used to verify these requirements.

### Safe-To-Turn-On Measurements

#### Visual Inspection [GEP]

Verify correct orientation of active components, diodes, tantalum capacitors, and connectors.

□ Completed

Comments: No observed orientation discrepancies.

Inspect observable solder joints for any breaks, cracks, and correct adhesion to component pins.

□ Completed

Comments: No observed damage to the PWA.

Identify any tombstone or billboard effects on passive components.

□ Completed

Comments: No tombstone/billboard effects observed.

Check for FOD or any other debris that may damage circuits upon power-up.

□ Completed

Comments: No FOD observed.

Verify all Do Not Populate (DNP) components are not installed per UUT CCA Drawing listed in Section 2.

□ Completed

Comments: Most DNP parts are hidden underneath shields for the RX amps. Visible components were confirmed.

#### Power to Ground Isolation [GEP]

Perform the power to ground isolation resistance measurements using a DMM and document results in Table 8.

Table 8. Power to Ground Isolation

| Measure: |  | Voltage Net | Ground Net | Measurement Requirement (Ω) | Value Measured (Ω) | Result |
| --- | --- | --- | --- | --- | --- | --- |
| From UUT Location | To UUT Location |  |  |  |  |  |
| J24-9 | J24-15 | P28V | P28V_RTN | > 20k | 1.51M | ( P / F ) |
| J24-6 | J24-3 | P48V | P48V_RTN | > 20k | OL | ( P / F ) |

#### Power to Power Isolation [GEP]

Perform the power to power isolation resistance measurements using a DMM and document results in Table 9.

Table 9. Power to Power Isolation

| Measure: |  | Voltage Net | Voltage Net | Measurement Requirement (Ω) | Value Measured (Ω) | Result |
| --- | --- | --- | --- | --- | --- | --- |
| From UUT Location | To UUT Location |  |  |  |  |  |
| J24-9 | J24-6 | P28V | P48V | > 20k | OL | ( P / F ) |

#### Ground to Ground Isolation / Continuity

Perform the ground to ground isolation resistance measurements using a DMM and document results in Table 10.

Table 10. Ground to Ground Isolation / Continuity

| Measure: |  | Ground Net | Ground Net | Measurement Requirement (Ω) | Value Measured (Ω) | Result |
| --- | --- | --- | --- | --- | --- | --- |
| From UUT Location | To UUT Location |  |  |  |  |  |
| J24-15 | J24-64 | P28V_RTN | GNDD | < 1 | 0.29 | ( P / F ) |
| J24-3 | J24-64 | P48V_RTN | GNDD | > 20k | 4.3M | ( P / F ) |
| J24-25 | J24-3 | P28V_RTN | P48V_RTN | > 20k | OL | ( P / F ) |

#### Local Supply Isolation / Continuity

Table 10a. Local Supply Isolation / Continuity

| Measure: |  | Power Net | Ground Net | Measurement Requirement (Ω) | Value Measured (Ω) | Result |
| --- | --- | --- | --- | --- | --- | --- |
| From UUT Location | To UUT Location |  |  |  |  |  |
| C440-1 | C440-2 | P12V_SRS2 | GNDD | > 10 | 5.2k | ( P / F ) |
| C200-1 | C200-2 | P12V_SRS1 | GNDD | > 10 | 4.2k | ( P / F ) |
| C649-1 | C649-2 | P12V_PRS | GNDD | > 10 | 4.07k | ( P / F ) |
| C206-1 | C206-2 | P12V_SRS0 | GNDD | > 10 | 4.3k | ( P / F ) |
| C1221-1 | C1221-2 | P6V | GNDD | > 10 | 10.4k | ( P / F ) |
| C226-2 | C226-1 | VCC_CORE | GNDD | > 10 | 2.22k | ( P / F ) |
| C351-2 | C351-1 | P3V3 | GNDD | > 10 | 41.1 | ( P / F ) |
| C700-2 | C700-1 | P2V5 | GNDD | > 10 | 1.22k | ( P / F ) |
| C98-2 | C98-1 | PS_VPLL | GNDD | > 10 | 1.7k | ( P / F ) |
| C183-1 | C183-2 | PS_VTT | GNDD | > 10 | 1.15k | ( P / F ) |
| C196-1 | C196-2 | PS_VREF | GNDD | > 10 | 322k | ( P / F ) |
| C419-2 | C419-1 | PL_VPP | GNDD | > 10 | 2.31k | ( P / F ) |
| C448-1 | C448-2 | PL_VREF | GNDD | > 10 | 1.8k | ( P / F ) |
| C455-1 | C455-2 | PL_VTT | GNDD | > 10 | 2.3k | ( P / F ) |
| C33-2 | C33-1 | PS_VCCINT | GNDD | > 1 | 3.4 | ( P / F ) |
| C896-2 | C896-1 | PS_VCCINT | GNDD | > 1 | 3.3 | ( P / F ) |
| C144-2 | C144-1 | PS_VPP | GNDD | > 10 | 2.22k | ( P / F ) |
| C143-1 | C143-2 | P1V2 | GNDD | > 10 | 133 | ( P / F ) |
| C316-1 | C316-2 | P1V8 | GNDD | > 10 | 165 | ( P / F ) |
| C675-1 | C675-2 | MGTYAVTT_PFLT | GNDD | > 10 | 33 | ( P / F ) |
| C362-1 | C362-2 | MGTYAVCC_PFLT | GNDD | > 1 | 7.2 | ( P / F ) |
| C640-1 | C640-2 | MGTYVCCAUX_PFLT | GNDD | > 10 | 3.23k | ( P / F ) |
| C480-1 | C480-2 | PL_VCCINT | GNDD | > 1 | .44 | ( P / F ) |
| C540-1 | C540-1 | PL_VCCINT | GNDD | > 1 | .53 | ( P / F ) |
| C1331-1 | C1331-1 | PL_VCCINT | GNDD | > 1 | .45 | ( P / F ) |
| C1310-1 | C1310-1 | PL_VCCINT | GNDD | > 1 | .56 | ( P / F ) |

#### Resistor Impedance Check

Perform the ground to ground isolation resistance measurements using a DMM and document results in Table 10b.

Table 10b. Resistor Impedance Check

| Measure: |  | Positive Net | Negative Net | Measurement Requirement (Ω) | Value Measured (Ω) |
| --- | --- | --- | --- | --- | --- |
| From UUT Location | To UUT Location |  |  |  |  |
| R271-2 | R271-1 | $74N35255 | GNDD | 183.1 | R271-2 |
| R452-2 | R452-1 | VFB | GNDD | 4.04k | R452-2 |
| R156-2 | R156-1 | _77N26838 | GNDD | 4.03k | R156-2 |
| R149-2 | R149-1 | _1N25279 | GNDD | 4.04k | R149-2 |
| R318-2 | R318-1 | _1N26838 | GNDD | 4.05k | R318-2 |
| R100-2 | R100-1 | _79N45151 | _79N45152 | 5.92k | R100-2 |
| R101-2 | R101-1 | P1V2 | _79N45151 | 5.91k | R101-2 |
| R258-2 | R258-1 | _79N49051 | _79N49052 | 7.23k | R258-2 |
| R257-2 | R257-1 | P1V8 | _79N49051 | 7.44k | R257-2 |
| R464-2 | R464-1 | _79N46370 | _79N46371 | 4.45k | R464-2 |
| R465-2 | R465-1 | MGTYAVTT_PFLT | _79N46370 | 4.45k | R465-2 |
| R208-2 | R208-1 | P3V3 | _80N51621 | 2.22k | R208-2 |
| R209-1 | R209-2 | _80N51621 | _80N51622 | 2.12k | R209-1 |
| R321-2 | R321-1 | MGTYAVCC_PFLT | __79N33501 | 7.34k | R321-2 |
| R320-2 | R320-1 | _79N33501 | _79N33502 | 7.54k | R320-2 |
| R406-2 | R406-1 | _80N52834 | _80N52835 | 8.3k | R406-2 |
| R407-2 | R407-1 | MGTYVCCAUX_PFLT | _80N52834 | 13.4k | R407-2 |
| R343-2 | R343-1 | PL_VPP | $25N4624 | 2.13k | R343-2 |
| R351-2 | R351-1 | $25N4624 | GNDD | 193 | R351-2 |
| R102-2 | R102-1 | PS_VPP | $50N6351 | 2.14k | R102-2 |
| R97-2 | R97-1 | $50N6351 | GNDD | 193 | R97-2 |
| R455-2 | R455-1 | P2V5 | $21N11248 | 1.4k | R455-2 |
| R443-2 | R443-1 | $21N11248 | GNDD | 190 | R443-2 |
| R444-2 | R444-1 | P1V1 | $21N11669 | 434 | R444-2 |
| R435-2 | R435-1 | $21N11669 | GNDD | 189 | R435-2 |
| R94-1 | R94-2 | PS_VPLL | $47N12503 | 1.01k | R94-1 |
| R93-2 | R93-1 | $47N12503 | GNDD | 187 | R93-2 |

#### Reserved

### Power-On

#### Power-On Test Procedure

- Connect as shown in Figure 1.

Figure 1. Input Power Test

- Ensure Power Supply output is disabled and configure the power supply with the following nominal settings:

| Output Voltage: | 27VDC + 0.5% |
| --- | --- |
| Voltage Ramp: | 0.14V/ms (if settable) |
| Current Limit: | 25A |

- Use a DMM to verify the voltage and current limit settings of the power supply. Reconnect the Input Power Test Cable to the Power Supply once verified.

- Connect the Power Supply to the UUT per Figure 2

- Turn on Power Supply

- Monitor 28V (configured @ 27V) Power supply current output and record in Table 11 .

Figure 2. Power Supply to UUT

Table 11. P28V Input Current

| Voltage Net | Expected Current (A) | Observed Current (A) | Result |
| --- | --- | --- | --- |
| P28V | < 5A | 0.74 | ( P / F) |

- While powered on, make the following voltage measurements at the UUT using a DMM and document results in Table 12.

Table 12. 28V Input Test Results

| Voltage Net | Measure |  | Required Voltage (V) | Measured Voltage | Result |
| --- | --- | --- | --- | --- | --- |
|  | From UUT Location | To UUT Location |  |  |  |
| P28V | J0-F8 | J0-D1 | 26 - 28 | 27.91 | ( P / F ) |
| P28V | J1-F8 | J0-D1 | 26 - 28 | 27.91 | ( P / F ) |
| P28V | J2-F8 | J0-D1 | 26 - 28 | 27.91 | ( P / F ) |
| P28V | J3-F8 | J0-D1 | 26 - 28 | 27.91 | ( P / F ) |
| P28V | J4-F8 | J0-D1 | 26 - 28 | 27.91 | ( P / F ) |
| P28V | J5-F8 | J0-D1 | 26 - 28 | 27.91 | ( P / F ) |
| P28V | J6-F8 | J0-D1 | 26 - 28 | 27.91 | ( P / F ) |
| P28V | J7-F8 | J0-D1 | 26 - 28 | 27.91 | ( P / F ) |
| P28V | J25-17 | J25-18 | 26 - 28 | 27.91 | ( P / F ) |
| P28V | J26-10 | J26-9 | 26 - 28 | 27.91 | ( P / F ) |
| P28V | J27-16 | J27-17 | 26 - 28 | 27.91 | ( P / F ) |
| P6V | C306-1 | C306-2 | 5.85-6.25 | 6.077 | ( P / F ) |
| P12V_PRS | J10-F11 | J10-A2 | 11.4 – 12.6 | 12.01 | ( P / F ) |
| P12V_SRS0 | J14-F11 | J14-A2 | 11.4 – 12.6 | 12.002 | ( P / F ) |
| P12V_SRS1 | J18-F11 | J18-A2 | 11.4 – 12.6 | 12.03 | ( P / F ) |
| P12V_SRS2 | J22-F11 | J22-A2 | 11.4 – 12.6 | 12.09 | ( P / F ) |
| P1V8 | J33-2 | J33-1 | 1.71 – 1.89 | 1.802 | ( P / F ) |
| P1V8 | J34-2 | J34-1 | 1.71 – 1.89 | 1.802 | ( P / F ) |
| P1V8 | J39-3 | J39-2 | 1.71 – 1.89 | 1.803 | ( P / F ) |
| P3V3 | J29-2 | J29-1 | 3.14 – 3.46 | 3.322 | ( P / F ) |
| P3V3 | J30-2 | J30-1 | 3.14 – 3.46 | 3.331 | ( P / F ) |
| P3V3 | J31-2 | J31-1 | 3.14 – 3.46 | 3.323 | ( P / F ) |
| P3V3 | J32-2 | J32-1 | 3.14 – 3.46 | 3.323 | ( P / F ) |
| P28V | J0-F8 | J0-D1 | 26 - 28 | 27.91 | ( P / F ) |
| P28V | J1-F8 | J0-D1 | 26 - 28 | 27.91 | ( P / F ) |
| P28V | J2-F8 | J0-D1 | 26 - 28 | 27.91 | ( P / F ) |
| P28V | J3-F8 | J0-D1 | 26 - 28 | 27.91 | ( P / F ) |
| P28V | J4-F8 | J0-D1 | 26 - 28 | 27.91 | ( P / F ) |
| P28V | J5-F8 | J0-D1 | 26 - 28 | 27.91 | ( P / F ) |
| P28V | J6-F8 | J0-D1 | 26 - 28 | 27.91 | ( P / F ) |
| P28V | J7-F8 | J0-D1 | 26 - 28 | 27.91 | ( P / F ) |

- Make the following voltage ripple measurements using an Oscilliscope and probes rated for 100 MHz. Record results in Table 13.

Table 13. Voltage Ripple

| Voltage Net | Measure |  |  | Required Voltage Ripple (V, pk - pk) | Measured Voltage Ripple (V, pk – pk) | Result |
| --- | --- | --- | --- | --- | --- | --- |
|  | From UUT Location | To UUT Location |  |  |  |  |
| P28V | J0-F8 | J0-D1 | < 0.3 |  | .00411 | ( P / F ) |
| P28V | J1-F8 | J0-D1 | < 0.3 |  | .00412 | ( P / F ) |
| P28V | J2-F8 | J0-D1 | < 0.3 |  | .00412 | ( P / F ) |
| P28V | J3-F8 | J0-D1 | < 0.3 |  | .00414 | ( P / F ) |
| P28V | J4-F8 | J0-D1 | < 0.3 |  | .00412 | ( P / F ) |
| P28V | J5-F8 | J0-D1 | < 0.3 |  | .00413 | ( P / F ) |
| P28V | J6-F8 | J0-D1 | < 0.3 |  | .00414 | ( P / F ) |
| P28V | J7-F8 | J0-D1 | < 0.3 |  | .00413 | ( P / F ) |
| P28V | J25-17 | J25-18 | < 0.3 |  | .00412 | ( P / F ) |
| P28V | J26-10 | J26-9 | < 0.3 |  | .00413 | ( P / F ) |
| P28V | J27-16 | J27-17 | < 0.3 |  | .00411 | ( P / F ) |
| P12V_PRS | J10-F11 | J10-A2 | < 0.128 |  | .0081 | ( P / F ) |
| P12V_SRS0 | J14-F11 | J14-A2 | < 0.128 |  | .0097 | ( P / F ) |
| P12V_SRS1 | J18-F11 | J18-A2 | < 0.128 |  | .0092 | ( P / F ) |
| P12V_SRS2 | J22-F11 | J22-A2 | < 0.128 |  | .0091 | ( P / F ) |
| P1V8 | J33-2 | J33-1 | < 0.019 |  | .00062 | ( P / F ) |
| P1V8 | J34-2 | J34-1 | < 0.019 |  | .00061 | ( P / F ) |
| P1V8 | J39-3 | J39-2 | < 0.019 |  | .00063 | ( P / F ) |
| P3V3 | J29-2 | J29-1 | < 0.035 |  | 0.0015 | ( P / F ) |
| P3V3 | J30-2 | J30-1 | < 0.035 |  | 0.0011 | ( P / F ) |
| P3V3 | J31-2 | J31-1 | < 0.035 |  | 0.0012 | ( P / F ) |
| P3V3 | J32-2 | J32-1 | < 0.035 |  | 0.0013 | ( P / F ) |

### Input Power Characteristics

#### Input Power Characteristics Test Setup

The previous section tested output power at 27V. This section will test output voltages when input is at 29V. For Test Setup, see Figure 2.

#### 29V Test Procedure

- Verify connections are made Figure 2. Power Supply to UUT

- With Power Supply disconnected, configure voltage to 29V.

- Turn On Power Supply.

- Check Voltages with DMM and record in Table 14.

#### Test Data

Table 14. 27V Input Test Results

| Voltage Net | Measure |  | Required Voltage (V) | Measured Voltage | Required Voltage Ripple (V) | Measured Voltage Ripple (V, pk – pk)) | Result |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | From UUT Location | To UUT Location |  |  |  |  |  |
| P28V | J0-F8 | J0-D1 | 28 - 30 | 27.87 | < 0.3 | 0.0039 | ( P / F ) |
| P28V | J1-F8 | J0-D1 | 28 - 30 | 27.87 | < 0.3 | 0.0039 | ( P / F ) |
| P28V | J2-F8 | J0-D1 | 28 - 30 | 27.87 | < 0.3 | 0.0039 | ( P / F ) |
| P28V | J3-F8 | J0-D1 | 28 - 30 | 27.87 | < 0.3 | 0.0039 | ( P / F ) |
| P28V | J4-F8 | J0-D1 | 28 - 30 | 27.87 | < 0.3 | 0.0039 | ( P / F ) |
| P28V | J5-F8 | J0-D1 | 28 - 30 | 27.87 | < 0.3 | 0.00391 | ( P / F ) |
| P28V | J6-F8 | J0-D1 | 28 - 30 | 27.87 | < 0.3 | 0.00391 | ( P / F ) |
| P28V | J7-F8 | J0-D1 | 28 - 30 | 27.87 | < 0.3 | 0.00394 | ( P / F ) |
| P28V | J25-17 | J25-18 | 28 - 30 | 27.87 | < 0.3 | 0.00393 | ( P / F ) |
| P28V | J26-10 | J26-9 | 28 - 30 | 27.87 | < 0.3 | 0.00391 | ( P / F ) |
| P28V | J27-16 | J27-17 | 28 - 30 | 27.87 | < 0.3 | 0.00391 | ( P / F ) |
| P12V_PRS | J10-F11 | J10-A2 | 11.4 – 12.6 | 11.99 | < 0.128 | .0097 | ( P / F ) |
| P12V_SRS0 | J14-F11 | J14-A2 | 11.4 – 12.6 | 12.017 | < 0.128 | .0096 | ( P / F ) |
| P12V_SRS1 | J18-F11 | J18-A2 | 11.4 – 12.6 | 12.013 | < 0.128 | .0094 | ( P / F ) |
| P12V_SRS2 | J22-F11 | J22-A2 | 11.4 – 12.6 | 12.014 | < 0.128 | .0092 | ( P / F ) |
| P1V8 | J33-2 | J33-1 | 1.71 – 1.89 | 1.799 | < 0.019 | 0.00018 | ( P / F ) |
| P1V8 | J34-2 | J34-1 | 1.71 – 1.89 | 1.799 | < 0.019 | 0.00013 | ( P / F ) |
| P1V8 | J39-3 | J39-2 | 1.71 – 1.89 | 1.799 | < 0.019 | 0.00016 | ( P / F ) |
| P3V3 | J29-2 | J29-1 | 3.14 – 3.46 | 3.31 | < 0.035 | 0.00014 | ( P / F ) |
| P3V3 | J30-2 | J30-1 | 3.14 – 3.46 | 3.31 | < 0.035 | 0.00011 | ( P / F ) |
| P3V3 | J31-2 | J31-1 | 3.14 – 3.46 | 3.31 | < 0.035 | 0.00011 | ( P / F ) |
| P3V3 | J32-2 | J32-1 | 3.14 – 3.46 | 3.31 | < 0.035 | 0.00011 | ( P / F ) |

### Weight

#### Weight Test Setup

Weigh with scale.

### EIA-422 Signal Test

#### Objective:

Verify that signals labeled EIA-422 meet the specifications for overshoot/undershoot, rise time, fall time, and voltage logic levels per the Electronics Industries Association (EIA) RS-422 standard.

#### Setup:

- Connect the signal under test to an oscilloscope using a high-impedance probe (10MΩ or higher) to minimize loading effects.

- Configure the oscilloscope with the following settings:

- Vertical Scale: 1V/div (or appropriate based on signal voltage levels).

- Horizontal Scale: 200ns/div (or appropriate to observe rise and fall times).

- Trigger: Set to the rising edge of the signal under test.

#### Test Steps:

##### Overshoot/Undershoot Verification:

- Measure the signal overshoot (peak voltage above the logic high level) and undershoot (voltage dip below the logic low level).

- Record the measured overshoot and undershoot in Table 15.

- Compare to the RS-422 specification limits (<10% of signal amplitude).

##### Rise Time Verification:

- Measure the time taken for the signal to transition from 10% to 90% of its full amplitude on the rising edge.

- Record the measured rise time in Table 15.

- Verify the rise time does not exceed the RS-422 specification of 50ns (as defined for typical applications).

##### Fall Time Verification:

- Measure the time taken for the signal to transition from 90% to 10% of its full amplitude on the falling edge.

- Record the measured fall time in Table 15.

- Verify the fall time does not exceed the RS-422 specification of 50ns.

##### Voltage Logic Level Verification:

- Measure the voltage levels of the differential pair (A and B) for logic high and logic low states.

- Record the logic high (VH) and logic low (VL) levels in Table 15.

- Verify that VH - VL ≥ 2.0V and the signal operates within the voltage swing specified by RS-422.

##### Noise Margin Verification:

Calculate the noise margin as the minimum of |VOD_high| and |VOD_low| minus the receiver input sensitivity threshold (200mV per TIA/EIA-422-B).

Record the noise margin in Table 15.

Verify that the noise margin is ≥ 0.2V.

Table 15. EIA-422 Signal Test Results

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) |  |  | ≤ 50ns |  |
| Fall Time (ns) |  |  | ≤ 50ns |  |
| Peak Voltage (V) |  |  | ≤ 8V |  |
| Trough Voltage (V) |  |  | ≥ -4V |  |
| Overshoot (%) |  |  | < 10% of amplitude |  |
| Undershoot (%) |  |  | < 10% of amplitude |  |
| VOD (VP – VN) (V) |  |  | ≥ 2.0V |  |
| Noise Margin (V) |  |  | ≥ 0.2V |  |

### P3V3 POWER Signal Test

#### Objective:

- Verify that the P3V3 POWER signal operates within the defined voltage, current, regulation, and ripple specifications.

#### Setup:

- Connect the power signal (P3V3 POWER) to a programmable electronic load (configured to simulate the maximum load condition).

- Use a digital multimeter (DMM) for static voltage measurements and an oscilloscope to measure ripple and regulation.

- Ensure a stable and controlled environment to minimize external noise.

#### Test Steps:

##### Voltage Verification:

- Connect the DMM probes across the P3V3 POWER signal and GND.

- Measure the DC voltage under no-load and full-load conditions.

- Record the measured voltage in Table 16.

- Verify the voltage is within the 3.3V ± 100mV range.

##### Current Verification:

- Configure the electronic load to draw 3000mA.

- Observe the power supply's behavior and measure the current using the load or the DMM.

- Record the current in Table 16Error! Reference source not found.

- Ensure the current supply meets the maximum 3000mA requirement.

##### Regulation Verification:

- Measure the voltage at no-load and full-load conditions using the DMM.

- Calculate the difference between no-load and full-load voltages.

- Record the regulation in Table 16.

- Verify the voltage regulation is within ± 100mV.

##### Ripple Voltage Verification:

- Connect the oscilloscope probes to the P3V3 POWER signal with AC coupling enabled.

- Configure the oscilloscope with the following settings:

  - Vertical Scale: 20mV/div

  - Horizontal Scale: 10µs/div

- Measure the peak-to-peak ripple voltage.

- Record the ripple voltage in Table 16Error! Reference source not found.

- Verify the ripple does not exceed 60mVpp.

Table 16. P3V3 POWER Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Voltage (No Load, V) |  | 3.3V ± 100mV |  |
| Voltage (Full Load, V) |  | 3.3V ± 100mV |  |
| Current (mA) |  | ≤ 3000mA |  |
| Regulation (V) |  | ± 100mV |  |
| Ripple (Vpp) |  | ≤ 60mVpp |  |

### EIA-485 Signal Test

#### Objective:

- Verify that signals labeled EIA-485 comply with the RS-485 specification, including differential voltage levels, rise and fall times, and proper noise margins.

#### Setup:

- Connect the EIA-485 signal under test to an oscilloscope using differential probes (one probe on each signal line, A and B).

- Configure the oscilloscope with the following settings:

  - Vertical Scale: 2V/div (or appropriate for the signal voltage range).

  - Horizontal Scale: 200ns/div (or suitable for the data rate).

  - Trigger: Set to the differential signal’s rising edge.

- Ensure the RS-485 bus is properly terminated with 120Ω resistors on both ends.

#### Test Steps:

##### Differential Voltage Verification:

- Measure the differential voltage (VAB = VA - VB) for both logic high and logic low states using the oscilloscope.

- Record the results in Table 17.

- Verify that the differential voltage is ≥ 1.5V for logic high and ≤ -1.5V for logic low per RS-485 specifications.

##### Rise and Fall Time Verification:

- Measure the rise time (10% to 90% of full amplitude) and fall time (90% to 10% of full amplitude) of the differential signal using the oscilloscope.

- Record the rise and fall times in Table 17.

- Verify that the rise and fall times are appropriate for the data rate of the system (e.g., ≤ 30ns for 10 Mbps).

##### Noise Margin Verification:

Calculate the noise margin as the minimum of |VOD_high| and |VOD_low| minus the receiver input sensitivity threshold (200mV per TIA/EIA-485).

Record the noise margin in Table 17.

Verify that the noise margin is ≥ 0.2V.

Table 17. EIA-485 Signal Test Results

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) |  |  | ≤ 30ns |  |
| Fall Time (ns) |  |  | ≤ 30ns |  |
| Peak Voltage (V) |  |  | ≤ 8V |  |
| Trough Voltage (V) |  |  | ≥ -4V |  |
| Overshoot (%) |  |  | < 10% of amplitude |  |
| Undershoot (%) |  |  | < 10% of amplitude |  |
| VOD (VP – VN) (V) |  |  | ≥ 1.5V |  |
| Noise Margin (V) |  |  | ≥ 0.2V |  |

### 1.8V LVCMOS Signal Test

#### Objective:

- Verify that signals labeled 1.8V LVCMOS comply with the JEDEC JESD8-7A specification, including voltage levels, rise and fall times, and noise margin.

#### Setup:

- Connect the LVCMOS signal under test to an oscilloscope using a high-impedance probe (10MΩ or higher).

- Configure the oscilloscope with the following settings:

  - Vertical Scale: 500mV/div (or appropriate for 1.8V signaling).

  - Horizontal Scale: 50ns/div (or appropriate for the signal frequency).

  - Trigger: Set to the signal's rising edge.

#### Test Steps:

##### Voltage Level Verification:

- Measure the voltage levels for logic high (VOH) and logic low (VOL) at the signal source using the oscilloscope or a DMM.

- Record the results in Table 18.

- Verify that VOH is ≥ 1.35V (75% of VDD) and VOL is ≤ 0.45V (25% of VDD) as per JESD8-7A.

##### Rise and Fall Time Verification:

- Measure the rise time (10% to 90% of full amplitude) and fall time (90% to 10% of full amplitude) using the oscilloscope.

- Record the rise and fall times in Table 18.

- Verify that the rise and fall times are ≤ 10ns.

##### Noise Margin Verification:

- Measure the signal's noise margins for both logic high and logic low states.

- Record the results in Table 18.

- Verify that the noise margin meets the requirement of at least 0.18V for VOH and 0.18V for VOL.

##### Signal Integrity Verification:

- Observe the signal waveform for overshoot, undershoot, and ringing.

- Record any deviations exceeding ± 10% of the 1.8V amplitude.

- Ensure the signal waveform remains stable without excessive transient behavior.

Table 18. 1.8V LVCMOS Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) |  | ≥ 1.35V |  |
| VOL (V) |  | ≤ 0.45V |  |
| Rise Time (ns) |  | ≤ 10ns |  |
| Fall Time (ns) |  | ≤ 10ns |  |
| NMH (V) |  | ≥ 0.18V |  |
| NML (V) |  | ≥ 0.18V |  |
| Overshoot (V) |  | Peak ≤ 2.1V |  |
| Undershoot (V) |  | Trough ≥ −0.3V |  |

### P1V8 POWER Signal Test

#### Objective:

- Verify that the P1V8 POWER signal operates within the specified voltage, current, regulation, and ripple parameters.

#### Setup:

- Connect the P1V8 POWER signal under test to a programmable electronic load capable of drawing up to 2A.

- Use a digital multimeter (DMM) for voltage measurements and an oscilloscope for ripple measurement.

- Ensure a stable and noise-free environment to ensure accurate results.

#### Test Steps:

##### Voltage Verification:

- Connect the DMM probes across the P1V8 POWER signal and GND.

- Measure the DC voltage under no-load and full-load (2A) conditions.

- Record the results in Table 19.

- Verify that the voltage is within 1.8V ± 100mV.

##### Current Capability Verification:

- Configure the electronic load to draw 2000mA.

- Observe the power supply output and measure the current using the electronic load or DMM.

- Record the measured current in Table 19.

- Ensure the signal can deliver the specified current without excessive voltage drop.

##### Regulation Verification:

- Measure the output voltage at no-load and full-load conditions using the DMM.

- Calculate the voltage difference between these conditions.

- Record the regulation in Table 19.

- Verify that the voltage variation is within ± 100mV.

##### Ripple Voltage Verification:

- Connect the oscilloscope to the P1V8 POWER signal with AC coupling enabled.

- Set the oscilloscope to the following settings:

- Vertical Scale: 10mV/div

- Horizontal Scale: 10µs/div

- Measure the peak-to-peak ripple voltage.

- Record the ripple voltage in Table 19.

- Verify that the ripple voltage does not exceed 50mVpp.

Table 19. P1V8 POWER Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Voltage (No Load, V) |  | 1.8V ± 100mV |  |
| Voltage (Full Load, V) |  | 1.8V ± 100mV |  |
| Current (mA) |  | ≤ 2000mA |  |
| Regulation (V) |  | ± 100mV |  |
| Ripple (Vpp) |  | ≤ 50mVpp |  |

### LVDS Signal Test

#### Objective:

- Verify that signals labeled LVDS comply with the EIA RS-644 specification, including differential voltage levels, rise and fall times, and signal integrity.

#### Setup:

- Connect the LVDS signal under test to a differential oscilloscope probe (one probe on each line, A and B).

- Configure the oscilloscope with:

- Vertical Scale: 200mV/div (or appropriate for the LVDS signal voltage range).

- Horizontal Scale: 10ns/div (or suitable for the signal frequency).

- Trigger: Set to the differential signal’s rising edge.

- Ensure the LVDS bus is properly terminated with 100Ω between the A and B lines.

#### Test Steps:

##### Differential Voltage Verification:

- Measure the differential voltage (VOD = VA - VB).

- Record the results in Table 20.

- Verify that the differential voltage is within 250mV to 450mV per RS-644.

##### Rise and Fall Time Verification:

- Measure the rise and fall times of the differential signal.

- Record the results in Table 20.

- Verify that the times are ≤ 260ps for data rates ≤ 1.5Gbps.

##### Signal Integrity Verification:

- Observe the signal waveform for overshoot, undershoot, and ringing.

- Record any deviations exceeding ±10% of the differential amplitude.

Table 20. LVDS Signal Test Results

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) |  |  | ≤ 2.08ns |  |
| Fall Time (ns) |  |  | ≤ 2.08ns |  |
| Peak Voltage (V) |  |  | ≤ 2.4V |  |
| Trough Voltage (V) |  |  | ≥ 0.0V |  |
| Overshoot (%) |  |  | < 10% |  |
| Undershoot (%) |  |  | < 10% |  |
| VOD (VP – VN) (mV) |  |  | 250–450mV |  |
| VOS (V) |  |  | 1.125–1.375V |  |
| Noise Margin (V) |  |  | ≥ 0.1V |  |

### 3.3V LVCMOS Signal Test

#### Objective:

- Verify that signals labeled 3.3V LVCMOS comply with JEDEC Section 1 specifications, including voltage levels, rise and fall times, and noise margins.

#### Setup:

- Connect the LVCMOS signal under test to an oscilloscope using a high-impedance probe (10MΩ or higher).

- Configure the oscilloscope with:

- Vertical Scale: 1V/div (or appropriate for 3.3V signaling).

- Horizontal Scale: 50ns/div.

#### Test Steps:

##### Voltage Level Verification:

- Measure logic high (VOH) and logic low (VOL) levels.

- Record the results in Table 21.

- Verify that VOH is ≥ 2.4V (73% of VDD) and VOL is ≤ 0.4V (12% of VDD).

##### Rise and Fall Time Verification:

- Measure the rise time (10% to 90%) and fall time (90% to 10%).

- Record the results in Table 21.

- Verify the rise and fall times are ≤ 10ns.

##### Noise Margin Verification:

- Measure the signal's noise margins for both logic high and logic low states.

- Record the results in Table 21.

- Verify that the noise margin meets the requirement of at least 0.4V for VOH and 0.4V for VOL.

##### Signal Integrity Verification:

- Observe the signal waveform for overshoot, undershoot, and ringing.

- Record any deviations exceeding ± 10% of the 3.3V amplitude.

- Ensure the signal waveform remains stable without excessive transient behavior.

Table 21. 3.3V LVCMOS Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) |  | ≥ 2.4V |  |
| VOL (V) |  | ≤ 0.4V |  |
| Rise Time (ns) |  | ≤ 10ns |  |
| Fall Time (ns) |  | ≤ 10ns |  |
| NMH (V) |  | ≥ 0.4V |  |
| NML (V) |  | ≥ 0.4V |  |
| Overshoot (V) |  | Peak ≤ 3.6V |  |
| Undershoot (V) |  | Trough ≥ −0.3V |  |

### P12V POWER Signal Test

#### Objective:

- Verify that the P12V POWER signal operates within the specified voltage, current, regulation, and ripple parameters.

#### Setup:

- Connect the P12V POWER signal under test to a programmable electronic load capable of drawing up to 10A.

- Use a DMM for static voltage measurements and an oscilloscope for ripple measurement.

#### Test Steps:

##### Voltage Verification:

- Measure the DC voltage under no-load and full-load (10A) conditions using the DMM.

- Record the results in Table 22.

- Verify the voltage is within 12V ± 600mV.

##### Current Capability Verification:

- Configure the electronic load to draw 10,000mA.

- Measure the current using the electronic load or DMM.

- Record the measured current in Table 22.

##### Regulation Verification:

- Measure the voltage at no-load and full-load conditions.

- Calculate and record the regulation in Table 22.

- Verify the voltage variation is ≤ 600mV.

##### Ripple Voltage Verification:

- Connect the oscilloscope to the P12V POWER signal with AC coupling.

- Measure the peak-to-peak ripple voltage.

- Record the ripple in Table 22.

- Verify that the ripple is ≤ 50mVpp.

Table 22. P12V POWER Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Voltage (No Load, V) |  | 12V ± 600mV |  |
| Voltage (Full Load, V) |  | 12V ± 600mV |  |
| Current (mA) |  | ≤ 10,000mA |  |
| Regulation (V) |  | ≤ 600mV |  |
| Ripple (Vpp) |  | ≤ 50mVpp |  |

### P28V POWER Signal Test

#### Objective:

- Verify that the P28V POWER signal operates within the specified voltage, current, regulation, and ripple parameters.

#### Setup:

- Connect the P28V POWER signal under test to a programmable electronic load capable of drawing up to 16A.

- Use a DMM for voltage measurements and an oscilloscope for ripple measurement.

#### Test Steps:

##### Voltage Verification:

- Measure the DC voltage under no-load and full-load (16A) conditions using the DMM.

- Record the results in Table 23.

- Verify the voltage is within 28V ± 340mV.

##### Current Capability Verification:

- Configure the electronic load to draw 16,000mA.

- Measure the current using the electronic load or DMM.

- Record the measured current in Table 23.

##### Regulation Verification:

- Measure the voltage at no-load and full-load conditions.

- Calculate and record the regulation in Table 23.

- Verify the voltage variation is ≤ 340mV.

##### Ripple Voltage Verification:

- Connect the oscilloscope to the P28V POWER signal with AC coupling.

- Measure the peak-to-peak ripple voltage.

- Record the ripple in Table 23.

- Verify that the ripple is ≤ 150mVpp.

Table 23. P28V POWER Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Voltage (No Load, V) |  | 28V ± 340mV |  |
| Voltage (Full Load, V) |  | 28V ± 340mV |  |
| Current (mA) |  | ≤ 16,000mA |  |
| Regulation (V) |  | ≤ 340mV |  |
| Ripple (Vpp) |  | ≤ 150mVpp |  |

### P48V POWER Signal Test

#### Objective:

- Verify that the P48V POWER signal operates within the specified voltage, current, regulation, and ripple parameters.

#### Setup:

- Connect the P48V POWER signal under test to a programmable electronic load capable of drawing up to 13A.

- Use a DMM for voltage measurements and an oscilloscope for ripple measurement.

#### Test Steps:

##### Voltage Verification:

- Measure the DC voltage under no-load and full-load (13A) conditions using the DMM.

- Record the results in Table 24.

- Verify the voltage is within 48V ± 460mV.

##### Current Capability Verification:

- Configure the electronic load to draw 13,000mA.

- Measure the current using the electronic load or DMM.

- Record the measured current in Table 24.

##### Regulation Verification:

- Measure the voltage at no-load and full-load conditions.

- Calculate and record the regulation in Table 24.

- Verify the voltage variation is ≤ 460mV.

##### Ripple Voltage Verification:

- Connect the oscilloscope to the P48V POWER signal with AC coupling.

- Measure the peak-to-peak ripple voltage.

- Record the ripple in Table 24.

- Verify that the ripple is ≤ 275mVpp.

Table 24. P48V POWER Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Voltage (No Load, V) |  | 48V ± 460mV |  |
| Voltage (Full Load, V) |  | 48V ± 460mV |  |
| Current (mA) |  | ≤ 13,000mA |  |
| Regulation (V) |  | ≤ 460mV |  |
| Ripple (Vpp) |  | ≤ 275mVpp |  |

### IF_ADC Signal Test

#### Objective:

- Verify that the IF_ADC signals meet the required electrical characteristics for maximum input power, analog input bandwidth, and optimized common mode voltage range.

#### Setup:

##### Equipment:

- Signal generator capable of producing differential signals with precise power control up to 14.6 dBm.

- Spectrum analyzer for bandwidth and return loss measurements.

- Differential probe for common mode voltage and return loss verification.

##### Connection:

- Connect the differential input signal to the RF-ADC through a 100Ω termination.

- Ensure proper AC or DC coupling based on the test requirements.

#### Test Steps:

##### Maximum Input Power Verification:

- Set the signal generator to a differential signal with a frequency within the ADC bandwidth (e.g., 1 GHz).

- Gradually increase the input signal power while monitoring the ADC output.

- Record the input power when the ADC output reaches full scale.

- Verify the maximum input power does not exceed 1 VPPD (1 dBm) at 0 dB attenuation or 4.8 VPPD (14.6 dBm) at ≥ 15 dB attenuation.

##### Analog Input Bandwidth Verification:

- Sweep the input signal frequency from DC to 6 GHz while maintaining a constant input power level (e.g., -1 dBFS).

- Use the spectrum analyzer to measure the output amplitude.

- Record the -3 dB bandwidth and verify it meets the 6 GHz specification.

##### Common Mode Voltage Range Verification:

- Measure the common mode voltage (VCM) at the differential input using a DMM or oscilloscope with a differential probe.

- Record the VCM value and verify it lies within the optimized range of 0.68–0.72 V or the maximum range of 0.4–1 V.

##### Crosstalk Isolation Verification:

- Inject a test tone (e.g., 1 GHz at -1 dBFS) on one channel while measuring the power level on an adjacent channel.

- Record the crosstalk isolation and verify it meets the specification of ≤ -75 dBc for FIN = 0–4 GHz and ≤ -70 dBc for FIN = 0–6 GHz.

##### Return Loss Verification:

- Measure the return loss of the differential input using a vector network analyzer (VNA).

- Record the return loss at various frequencies and verify it meets the following specifications:

  - RL ≤ -12 dB up to 4 GHz

  - RL ≤ -10 dB up to 6 GHz

Table 25. IF_ADC Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Maximum Input Power (dBm) |  | ≤ 1 dBm @ 0 dB attenuation; ≤ 14.6 dBm @ ≥ 15 dB attenuation |  |
| Bandwidth (-3 dB, GHz) |  | 6 GHz |  |
| Common Mode Voltage (V) |  | 0.68–0.72 V (optimized) or 0.4–1 V (maximum) |  |
| Crosstalk Isolation (dBc) |  | ≤ -75 dBc @ 0–4 GHz; ≤ -70 dBc @ 0–6 GHz |  |
| Return Loss (dB) |  | ≤ -12 dB @ ≤ 4 GHz; ≤ -10 dB @ ≤ 6 GHz |  |

### IF_DAC Signal Test

#### Objective:

- Verify that the IF_DAC signals meet the required electrical characteristics for resolution, sample rate, output power, output current range, bandwidth, and other relevant parameters.

#### Setup:

##### Equipment:

- Signal generator to provide a clock input to the RF-DAC.

- Spectrum analyzer for bandwidth, output power, and return loss measurements.

- Oscilloscope with differential probes for signal integrity and output waveform analysis.

- High-precision current measurement tool (e.g., a current probe or meter).

##### Connection:

- Connect the differential outputs of the DAC to a 100Ω termination.

- Use AC or DC coupling depending on the parameter under test.

- Ensure proper VDAC_AVTT biasing (nominally 3.0V).

#### Test Steps:

##### Resolution Verification:

- Configure the RF-DAC to output a waveform at full-scale resolution.

- Verify that the output resolution is 14 bits.

##### Sample Rate Verification:

- Configure the RF-DAC at its maximum sample rate for the selected datapath mode:

  - Mode 2/3/4: Verify sample rates up to 9.85 GS/s for -2E speed grade or 8.92 GS/s for -1E speed grade.

  - Mode 1: Verify sample rates up to 7.0 GS/s.

- Record the sample rate and ensure it matches the expected performance.

##### Maximum Output Power Verification:

- Set the RF-DAC to output a sine wave at a frequency below 200 MHz.

- Measure the output power using the spectrum analyzer.

- Verify that the power falls within -18.5 to 6.5 dBm.

##### Output Current Range Verification:

- Measure the differential output current at the termination resistor for both AC and DC coupling:

  - AC Coupling: Verify a range of 2.25–40.5 mA.

  - DC Coupling: Verify a range of 6.4–32 mA.

- Record the output current.

##### Variable Output Power and Bandwidth Verification:

- Measure the DAC output power dynamic range across specified frequencies:

- At 240 MHz: ≥ 24 dB

- At 3500 MHz: ≥ 20 dB

- At 4900 MHz: ≥ 18 dB

- At 5900 MHz: ≥ 17 dB

- Sweep the output frequency from DC to 6 GHz to measure the -3 dB bandwidth. Verify the bandwidth is 6 GHz.

##### Return Loss Verification:

- Measure the return loss at the DAC outputs using a vector network analyzer (VNA):

- Up to 4 GHz: RL ≤ -12 dB

- Up to 6 GHz: RL ≤ -10 dB

##### Crosstalk Isolation Verification:

- Inject a test tone at the output of one channel and measure the isolation on adjacent channels:

  - 0–4 GHz: ≤ -75 dBc

  - 0–6 GHz: ≤ -70 dBc

Table 26. IF_DAC Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Resolution (bits) |  | 14 |  |
| Sample Rate (GS/s) |  | ≤ 9.85 (speed grade -2E) |  |
| Output Power (dBm) |  | -18.5 to 6.5 |  |
| Output Current (mA, AC) |  | 2.25–40.5 |  |
| Output Current (mA, DC) |  | 6.4–32 |  |
| Bandwidth (-3 dB, GHz) |  | 6 |  |
| Return Loss (dB @ ≤ 4 GHz) |  | ≤ -12 |  |
| Return Loss (dB @ ≤ 6 GHz) |  | ≤ -10 |  |
| Crosstalk Isolation (dBc) |  | ≤ -75 (0–4 GHz), ≤ -70 (0–6 GHz) |  |

### AXI CHIP2CHIP Signal Test

#### Objective:

- Verify that AXI CHIP2CHIP signals conform to the Aurora 64B/66B protocol requirements as outlined in the Xilinx SP011 specification, focusing on link integrity, data transfer performance, and error detection.

#### Setup:

##### Equipment:

- FPGA evaluation board with Aurora 64B/66B core implemented.

- Logic analyzer or protocol analyzer capable of monitoring AXI transactions.

- High-speed oscilloscope for signal integrity measurements.

- Test pattern generator (optional for error injection).

##### Configuration:

- Implement AXI CHIP2CHIP links on the FPGA and ensure the configuration aligns with the Aurora 64B/66B core setup.

- Establish a link between two FPGAs or between an FPGA and a testbench emulator.

#### Test Steps:

##### Link Initialization Verification:

- Power on the system and monitor the Aurora 64B/66B link initialization process.

- Verify that the link transitions from RESET to READY state as per the specification.

- Record the initialization time and ensure it is within acceptable limits.

##### Data Transfer Verification:

- Transmit predefined test patterns (e.g., PRBS sequences or known data payloads) across the AXI CHIP2CHIP link.

- Monitor and log received data at the destination.

- Compare the transmitted and received data to verify the integrity of the transmission.

##### Throughput Measurement:

- Measure the throughput of the AXI CHIP2CHIP interface under different operating conditions.

- Ensure the achieved throughput meets or exceeds the design's expected rate, as specified in the protocol.

##### Error Detection and Recovery:

- Inject errors (e.g., bit flips or invalid sequences) into the link using a pattern generator or testbench.

- Verify that the Aurora core detects and reports errors via the protocol's error monitoring features.

- Confirm that the link automatically recovers and resumes data transmission after errors are corrected.

##### Signal Integrity Verification:

- Measure eye diagrams and jitter at the high-speed serial transceiver using a high-speed oscilloscope.

- Record the signal's margin and verify it meets the Aurora protocol's requirements for the operational data rate.

##### Protocol Compliance Verification:

- Verify protocol-specific parameters such as frame alignment, control symbol decoding, and flow control signals.

- Ensure the link adheres to the Aurora protocol's requirements for frame format and error handling.

Table 27. AXI CHIP2CHIP Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Link Initialization Time |  | Within protocol limits |  |
| Data Integrity (Error Rate) |  | Zero errors |  |
| Throughput (Gbps) |  | Expected rate |  |
| Error Detection Capability |  | Detected and reported |  |
| Eye Diagram Margin |  | Meets protocol specs |  |
| Frame Alignment Compliance |  | As per specification |  |

### PHY_ETH Signal Test

#### Objective:

- Verify that PHY_ETH signals meet the IEEE 802.3 requirements for 10BASE-Te, 100BASE-TX, and 1000BASE-T operation, including electrical characteristics, link integrity, data transfer performance, and compliance with Ethernet standards.

#### Setup:

##### Equipment:

- Ethernet traffic generator and analyzer (e.g., IXIA, Spirent, or equivalent).

- High-bandwidth oscilloscope for signal integrity testing.

- Ethernet PHY evaluation board or system under test (SUT) with appropriate PHY drivers.

- Link partner device or Ethernet switch capable of supporting all three modes (10BASE-Te, 100BASE-TX, 1000BASE-T).

##### Connections:

- Connect the PHY_ETH port of the SUT to the test equipment using standard Ethernet cabling (Cat 5e or higher).

- Ensure proper termination and grounding per the IEEE 802.3 standard.

#### Test Steps:

##### Link Initialization Verification:

- Connect the PHY_ETH port to the link partner and verify auto-negotiation for all supported speeds (10 Mbps, 100 Mbps, 1 Gbps).

- Confirm link establishment and record the time taken for each mode.

##### Signal Integrity Testing:

- Capture the transmitted signals using a high-bandwidth oscilloscope.

- Analyze eye diagrams, rise/fall times, and jitter at the PHY outputs for each speed mode.

- Verify compliance with IEEE 802.3 electrical specifications for signal quality.

##### Transmit Timing Parameters:

- Measure timing parameters such as clock accuracy, transmit timing jitter, and skew.

- Verify compliance with the specified limits in IEEE 802.3 for each mode.

##### Data Transfer Verification:

- Generate Ethernet frames at each supported speed:

  - 10 Mbps (10BASE-Te)

  - 100 Mbps (100BASE-TX)

  - 1 Gbps (1000BASE-T)

- Verify successful frame transmission and reception with zero errors.

##### Throughput and Latency Measurement:

- Measure throughput and latency at each speed using the traffic generator and analyzer.

- Verify that throughput meets the maximum rate for the mode and latency is within acceptable limits.

##### Return Loss and Crosstalk Testing:

- Measure the return loss at the PHY's output for all supported frequencies.

- Verify compliance with IEEE 802.3 specifications (e.g., return loss < -15 dB at relevant frequencies).

- Perform crosstalk testing to ensure isolation between transmit and receive pairs.

##### Power Consumption Verification:

- Measure power consumption during idle, transmit, and receive modes at all speeds.

- Verify that power consumption meets the design's specifications for the PHY.

Table 28. PHY_ETH Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Link Initialization Time |  | Within IEEE limits |  |
| Signal Integrity Compliance |  | Meets IEEE standards |  |
| Timing Parameters (Jitter) |  | As per IEEE specifications |  |
| Throughput (Gbps) |  | Max rate per mode |  |
| Latency (µs) |  | Within acceptable limits |  |
| Return Loss (dB) |  | < -15 dB |  |
| Crosstalk Isolation (dB) |  | Meets IEEE requirements |  |
| Power Consumption (W) |  | Within design spec |  |

### OC_LOW Signal Test

#### Objective:

- Verify that the OC_LOW signal meets the active low open collector specifications, including voltage levels, current sink capability, and capacitive load behavior.

#### Setup:

- Connect the OC_LOW signal under test to a DMM or oscilloscope for voltage measurement.

- Use a programmable electronic load or resistor network to simulate a 24mA load.

#### Test Steps:

##### Low State Driver Verification:

- Measure the low-state output voltage (VOL) while sinking 24mA.

- Record the results in Table 29.

- Verify that VOL < 0.6V at IOL = 24mA.

##### Receiver Input Level Verification:

- Measure the receiver low input level and high input level.

- Record the results in Table 29.

- Verify that the receiver low level is < 0.8V and the high level is > 2.0V.

##### Capacitive Load Verification:

- Connect a 20pF capacitor to the OC_LOW signal.

- Verify signal stability and compliance with specifications.

##### Source and Sink Current Verification:

- Measure the source current at 0.6V and the sink current at 2.4V.

- Record the results in Table 29.

- Verify that source current @ 0.6V < 600µA and sink current @ 2.4V < 50µA.

Table 29. OC_LOW Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Low State Voltage (V) |  | < 0.6V at 24mA |  |
| Receiver Low Input Level (V) |  | < 0.8V |  |
| Receiver High Input Level (V) |  | > 2.0V |  |
| Source Current (µA) @ 0.6V |  | < 600µA |  |
| Sink Current (µA) @ 2.4V |  | < 50µA |  |

### SGMII Signal Test

#### Objective:

- Verify that SGMII signals comply with the Cisco Serial-GMII specification, including link initialization, signaling levels, timing characteristics, and data transfer performance.

#### Setup:

##### Equipment:

- Gigabit Ethernet traffic generator and analyzer (e.g., IXIA, Spirent, or equivalent).

- High-bandwidth oscilloscope for signal integrity testing.

- SGMII-capable PHY evaluation board or device under test (DUT).

- Reference link partner device or Ethernet switch supporting SGMII.

##### Connections:

- Connect the DUT's SGMII port to the test equipment or link partner using standard Ethernet cabling.

- Ensure proper grounding and differential signal routing for optimal performance.

#### Test Steps:

##### Link Initialization Verification:

- Power on the system and establish an SGMII link with the link partner or test equipment.

- Verify the auto-negotiation process between the MAC and PHY, ensuring proper speed and duplex settings (10 Mbps, 100 Mbps, 1 Gbps).

- Record link initialization time and ensure it is within the specification limits.

##### Signal Integrity Testing:

- Measure the differential signaling levels using a high-bandwidth oscilloscope:

  - Verify voltage swing levels for high-speed serial transmission (e.g., 400–600 mVpp).

  - Ensure proper signal transitions with minimal overshoot/undershoot and ringing.

- Capture eye diagrams for signal quality analysis. Verify compliance with Serial-GMII signal quality requirements.

##### Timing and Clock Recovery Verification:

- Measure the transmit and receive clock jitter, timing skew, and signal-to-noise ratio (SNR).

- Verify that timing parameters comply with the Serial-GMII specification for robust clock recovery.

##### Data Integrity Testing:

- Generate Ethernet frames with the traffic generator at various speeds (10 Mbps, 100 Mbps, 1 Gbps).

- Monitor transmitted and received frames for errors.

- Record the error rate and verify zero errors during normal operation.

##### Throughput and Latency Measurement:

- Measure the SGMII throughput and latency at different operating speeds.

- Verify that the measured throughput meets the expected performance for the speed and duplex mode.

##### Power Consumption Verification:

Measure the PHY or MAC's power consumption during idle, transmit, and receive modes.

Ensure power usage falls within acceptable limits as defined in the system requirements.

##### Crosstalk and Return Loss Verification:

- Measure the crosstalk between SGMII differential pairs and adjacent signal lines.

- Verify that crosstalk isolation meets the design requirements for the interface.

- Measure return loss for the differential signal and ensure it is within the specification limits.

Table 30. SGMII Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Link Initialization Time |  | Within Serial-GMII limits |  |
| Signal Voltage Swing (mVpp) |  | 400–600 |  |
| Eye Diagram Compliance |  | Meets Serial-GMII requirements |  |
| Clock Jitter (ps) |  | As per Serial-GMII spec |  |
| Data Integrity (Error Rate) |  | Zero errors |  |
| Throughput (Gbps) |  | 1 Gbps (full duplex) |  |
| Latency (µs) |  | Within acceptable limits |  |
| Crosstalk Isolation (dB) |  | Meets design requirements |  |
| Return Loss (dB) |  | As per Serial-GMII spec |  |
| Power Consumption (W) |  | Within design limits |  |

### FAN_PS_SIG Signal Test

#### Objective:

- Verify that FAN_PS_SIG signals meet the specified voltage range, period, duty cycle, and current output requirements.

#### Setup:

##### Equipment:

- Oscilloscope to measure voltage, period, and duty cycle.

- Programmable electronic load or current meter to measure output current.

- Test setup capable of controlling or simulating fan RPM.

##### Connections:

- Connect the FAN_PS_SIG signal output to the oscilloscope and electronic load.

- Ensure proper grounding and signal routing for accurate measurements.

#### Test Steps:

##### Voltage Range Verification:

- Measure the signal's peak voltage (Vmax) and minimum voltage (Vmin) using the oscilloscope.

- Record the results in Table 31.

- Verify that the signal voltage remains within the range of 4V–6V.

##### Period and Frequency Verification:

- Set the fan to a known RPM value.

- Measure the signal's period (T) using the oscilloscope.

- Record the period and calculate the corresponding RPM using the formula: RPM=60*2*T

- Verify that the measured period corresponds to the expected RPM.

##### Duty Cycle Verification:

- Measure the high time THIGH and low TLOW of the signal using the oscilloscope.

- Calculate the duty cycle using the formula: Duty Cycle = THIGHTHIGH+TLOW

- Record the duty cycle and verify it is within the range of 40%–60%.

##### Output Current Verification:

- Set the electronic load to draw a current of 1 mA at 5V from the FAN_PS_SIG output.

- Measure the actual current using the current meter.

- Record the result in Table 31 and verify that the signal can source > 1 mA at 5V.

Table 31. FAN_PS_SIG Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Voltage Range (V) | Vmin: | 4V–6V |  |
|  | Vmax: |  |  |
| Period (T, s) |  | Calculated from RPM |  |
| Duty Cycle (%) |  | 40%–60% |  |
| Output Current (mA) |  | > 1 mA @ 5V |  |

### IF_ADC_SE Signal Test

#### Objective:

- Verify that the IF_ADC_SE signal operates within the specified frequency, power, and impedance parameters.

#### Setup:

##### Equipment:

- Signal generator capable of producing single-ended signals with a frequency range up to 3 GHz.

- Spectrum analyzer to measure signal power and frequency.

- High-precision impedance analyzer or vector network analyzer (VNA).

##### Connections:

- Connect the signal generator output to the IF_ADC_SE input through a 50Ω termination.

- Connect the spectrum analyzer or impedance analyzer to monitor the signal characteristics.

#### Test Steps:

##### Frequency Verification:

- Generate a test signal at various frequencies within the operating range (e.g., 1 GHz, 2 GHz, 2.9 GHz).

- Measure the frequency using the spectrum analyzer.

- Verify that the frequency is < 3 GHz.

##### Power Verification:

- Set the signal generator output power to 0 dBm.

- Measure the signal power at the IF_ADC_SE input using the spectrum analyzer.

- Record the measured power and verify it matches the 0 dBm requirement.

##### Impedance Verification:

- Use the impedance analyzer or VNA to measure the input impedance of the IF_ADC_SE.

- Verify that the input impedance is 50Ω.

Table 32. IF_ADC_SE Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Frequency (GHz) |  | < 3 GHz |  |
| Power (dBm) |  | 0 dBm |  |
| Impedance (Ω) |  | 50Ω |  |

### IF_DAC_SE Signal Test

#### Objective:

- Verify that the IF_DAC_SE signal operates within the specified frequency, power, and impedance parameters.

#### Setup:

##### Equipment:

- Spectrum analyzer to measure the output frequency and power.

- High-precision impedance analyzer or VNA.

##### Connections:

- Connect the IF_DAC_SE output to a spectrum analyzer through a 50Ω termination.

- Use an impedance analyzer to measure the output impedance.

#### Test Steps:

##### Frequency Verification:

- Activate the DAC to output signals at various frequencies (e.g., 1 GHz, 2 GHz, 2.9 GHz).

- Measure the output frequency using the spectrum analyzer.

- Verify that the frequency is < 3 GHz.

##### Power Verification:

- Measure the output power at the DAC using the spectrum analyzer.

- Record the measured power and verify it is 0 dBm.

##### Impedance Verification:

- Use the impedance analyzer to measure the output impedance of the DAC.

- Verify that the output impedance is 50Ω.

Table 33. IF_DAC_SE Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Frequency (GHz) |  | < 3 GHz |  |
| Power (dBm) |  | 0 dBm |  |
| Impedance (Ω) |  | 50Ω |  |

### USB1_0 Signal Test

#### Objective:

- Verify that USB1_0 signals comply with the USB Specification, Chapter 7.1, covering high-speed signal quality, timing, and electrical characteristics.

#### Setup:

##### Equipment:

- USB protocol analyzer capable of testing USB 2.0 signaling.

- High-bandwidth oscilloscope for signal integrity analysis.

- USB device and host for loopback or communication tests.

##### Connections:

- Connect the USB1_0 signal to a protocol analyzer or loopback test device.

- Ensure proper termination and cable quality per the USB specification.

#### Test Steps:

##### Signal Integrity Verification:

- Measure USB high-speed differential signaling using an oscilloscope.

- Capture eye diagrams for compliance with USB 2.0 high-speed signal quality requirements.

- Verify compliance with voltage swing, jitter, and rise/fall times.

##### Timing Analysis:

- Measure and verify the USB signaling timing parameters, such as EOP width, frame interval, and packet timing.

- Compare measured values to USB specification limits.

##### Protocol Compliance Testing:

- Use a USB protocol analyzer to monitor data exchange between the host and device.

- Verify error-free communication, proper enumeration, and data transfer.

Table 34. USB1_0 Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Eye Diagram Compliance |  | Meets USB 2.0 specification |  |
| Timing Parameters |  | As per USB specification |  |
| Data Integrity |  | Zero errors |  |

### USB_1VBUS Signal Test

#### Objective:

- Verify that USB_1VBUS signals meet the electrical characteristics defined in USB Specification, Chapter 7.2.

#### Setup:

##### Equipment:

- Multimeter or oscilloscope for voltage measurements.

- USB load device to simulate current draw.

##### Connections:

- Connect the USB_1VBUS signal to a test load.

- Ensure proper grounding for accurate measurements.

#### Test Steps:

##### Voltage Range Verification:

- Measure the USB_1VBUS voltage with a multimeter or oscilloscope under no-load and full-load conditions.

- Record the results and verify the voltage is within the USB specification range (typically 4.75V–5.25V).

##### Load Current Capability:

- Connect a load device to draw the maximum specified current.

- Verify the VBUS voltage does not drop below the acceptable range during maximum load conditions.

Table 35. USB_1VBUS Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Voltage (No Load) |  | 4.75V–5.25V |  |
| Voltage (Full Load) |  | 4.75V–5.25V |  |

1.30 VADCCM Signal Test

#### Objective:

- Verify that the VADCCM reference signal operates at 1.25V ± 20mV.

#### Setup:

##### Equipment:

- High-precision multimeter or oscilloscope for voltage measurement.

- Stable power source to minimize noise during testing.

##### Connections:

- Connect the VADCCM signal to the measurement device.

- Ensure proper calibration of the measurement equipment for accuracy.

#### Test Steps:

##### Voltage Measurement:

- Measure the VADCCM signal voltage using a high-precision multimeter or oscilloscope.

- Record the measured voltage.

- Verify the voltage is within 1.25V ± 20mV (1.23V–1.27V).

Table 36. VADCCM Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Voltage (V) |  | 1.25V ± 20mV |  |

### 2.5V LVCMOS Signal Test

#### Objective:

- Verify that the 2.5V LVCMOS signals comply with the JEDEC JESD80 specification, focusing on voltage levels, rise and fall times, and noise margins.

#### Setup:

##### Equipment:

- Digital multimeter (DMM) or oscilloscope for voltage measurements.

- Oscilloscope with a high-impedance probe (10MΩ or higher) for signal timing and integrity analysis.

- Stable signal generator for driving test patterns.

##### Connections:

- Connect the 2.5V LVCMOS signal under test to the measurement equipment.

- Ensure the measurement device is properly grounded for accurate readings.

#### Test Steps:

##### Voltage Level Verification:

- Measure the logic high (VOH) and logic low (VOL) voltage levels at the signal source.

- Record the results in Table 37.

- Verify that:

  - VOH ≥ 1.8V (73% of VDD)

  - VOL ≤ 0.75V (12% of VDD)

##### Rise and Fall Time Verification:

- Configure the oscilloscope to measure rise time (10% to 90% of the signal amplitude) and fall time (90% to 10%).

- Record the results in Table 37.

- Verify that the rise and fall times are ≤ 5ns.

##### Noise Margin Verification:

- Measure the noise margins for both logic high and logic low states.

- Record the results in Table 37.

- Verify that:

  - High-state noise margin ≥ 0.7V

  - Low-state noise margin ≥ 0.7V

##### Signal Integrity Verification:

- Observe the signal waveform on the oscilloscope for overshoot, undershoot, and ringing.

- Ensure deviations are within ±10% of the signal amplitude.

Table 37. 2.5V LVCMOS Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) |  | ≥ 1.8V |  |
| VOL (V) |  | ≤ 0.75V |  |
| Rise Time (ns) |  | ≤ 5ns |  |
| Fall Time (ns) |  | ≤ 5ns |  |
| Noise Margin High (V) |  | ≥ 0.7V |  |
| Noise Margin Low (V) |  | ≥ 0.7V |  |
| Overshoot/Undershoot (%) |  | ≤ ±10% of amplitude |  |

### P28V_DISCRETE Signal Test

#### Objective:

- Verify that the P28V_DISCRETE signal operates within the specified logic voltage levels, current sink/source capabilities, and signal integrity parameters.

#### Setup:

##### Equipment:

- Digital multimeter (DMM) or oscilloscope for voltage measurements.

- Programmable electronic load or resistor network to simulate a load.

- Signal generator or test controller to toggle the signal state.

##### Connections:

- Connect the P28V_DISCRETE signal to the measurement equipment and load.

- Ensure proper grounding and accurate probe placement for high-voltage signals.

#### Test Steps:

##### Voltage Logic Level Verification:

- Measure the high-state voltage (VOH) and low-state voltage (VOL) of the signal using a DMM or oscilloscope:

  - Toggle the signal to the high state and record VOH.

  - Toggle the signal to the low state and record VOL.

- Verify that:

  - VOH ≥ 24V and ≤ 28V.

  - VOL ≤ 0.5V.

##### Current Sourcing and Sinking Capability:

- Connect a 45mA load to the signal in both the high and low states:

- High State: Verify the signal maintains VOH ≥ 24V while sourcing 45mA.

- Low State: Verify the signal maintains VOL ≤ 0.5V while sinking 45mA.

- Record the results in Table 38.

##### Signal Integrity Verification:

- Observe the signal waveform using an oscilloscope while toggling between high and low states:

  - Check for overshoot, undershoot, or ringing.

  - Ensure any deviations do not exceed ±10% of the nominal voltage levels.

- Record any observed anomalies.

##### Timing Verification:

- Measure the rise time (10% to 90%) and fall time (90% to 10%) of the signal.

- Verify that the transition times meet system requirements or are within acceptable limits for the application.

Table 38. P28V_DISCRETE Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (High State, V) |  | 24–28V |  |
| VOL (Low State, V) |  | ≤ 0.5V |  |
| Current (High State, mA) |  | ≥ 45mA |  |
| Current (Low State, mA) |  | ≥ 45mA |  |
| Rise Time (ns) |  | Application-specific |  |
| Fall Time (ns) |  | Application-specific |  |
| Overshoot/Undershoot (%) |  | ≤ ±10% of levels |  |

### PL_MGT Signal Test

#### Objective:

- Verify that the PL_MGT signals comply with the DC characteristics of the GTY transceivers, including input/output voltage levels, resistance, and skew.

#### Setup:

##### Equipment:

- Differential signal generator capable of providing test signals at multiple data rates (e.g., >10.3125 Gb/s, 6.6–10.3125 Gb/s, ≤6.6 Gb/s).

- High-bandwidth oscilloscope for signal measurement and analysis.

- Differential probes for precise measurement.

- Precision impedance analyzer or vector network analyzer (VNA).

##### Connections:

- Connect the signal generator to the PL_MGT differential input pins.

- Connect the oscilloscope and VNA to the output pins for measurement.

- Ensure proper AC or DC coupling as required by the test case.

#### Test Steps:

##### Differential Input Voltage Verification:

###### Generate a differential signal at the specified data rates:

- 10.3125 Gb/s

- 6.6–10.3125 Gb/s

- ≤6.6 Gb/s

- Measure the differential peak-to-peak input voltage (DVPPIN) at the receiver pins.

###### Record the results and verify they meet the following specifications:

- 10.3125 Gb/s: 150–1250 mV

- 6.6–10.3125 Gb/s: 150–1250 mV

- ≤6.6 Gb/s: 150–2000 mV

##### Single-Ended Input Voltage Verification:

- Measure the single-ended input voltage (VIN) referenced to GND.

- Verify that the voltage falls within the range of -400 mV to VMGTAVTT (1.2V).

##### Common Mode Input Voltage Verification:

- Measure the common mode input voltage (VCMIN) at the receiver input.

- Verify that it is ≤ 2/3 VMGTAVTT (0.8V for VMGTAVTT = 1.2V).

##### Differential Output Voltage Verification:

- Configure the transmitter for maximum output swing.

- Measure the differential peak-to-peak output voltage (DVPPOUT).

- Verify that the output swing is ≥ 800 mV.

##### Common Mode Output Voltage Verification:

- For DC-coupled outputs: Measure the common mode output voltage (VCMOUTDC) under the following conditions:

  - Remote RX terminated to GND.

  - Remote RX floating.

  - Remote RX terminated to VRX_TERM.

- For AC-coupled outputs: Verify that VCMOUTAC follows the equation TBD

##### Resistance Verification:

- Measure the differential input resistance (RIN) at the receiver input. Verify that RIN = 100Ω.

- Measure the differential output resistance (ROUT) at the transmitter output. Verify that ROUT = 100Ω.

##### Output Skew Verification:

- Measure the intra-pair skew between TXP and TXN.

- Verify that the skew (TOSKEW) is ≤ 10 ps.

##### AC Coupling Capacitor Verification:

- Verify that the external AC coupling capacitor has a value of 100 nF.

Table 39. PL_MGT Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| DVPPIN (>10.3125 Gb/s) |  | 150–1250 mV |  |
| DVPPIN (6.6–10.3125 Gb/s) |  | 150–1250 mV |  |
| DVPPIN (≤6.6 Gb/s) |  | 150–2000 mV |  |
| VIN (Single-Ended, mV) |  | -400 to VMGTAVTT (1.2V) |  |
| VCMIN (mV) |  | ≤ 2/3 VMGTAVTT (0.8V) |  |
| DVPPOUT (mV) |  | ≥ 800 mV |  |
| VCMOUTDC |  | Per equation-based conditions |  |
| VCMOUTAC |  | Per equation-based conditions |  |
| RIN (Ω) |  | 100 |  |
| ROUT (Ω) |  | 100 |  |
| TOSKEW (ps) |  | ≤ 10 |  |
| AC Coupling Capacitor (nF) |  | 100 |  |

### PS_GTR Signal Test

#### Objective:

- Verify that the PS_GTR signals comply with the DC characteristics of the PS-GTR transceivers, including input/output voltage levels, resistance, and skew.

#### Setup:

##### Equipment:

- Differential signal generator for input testing.

- High-bandwidth oscilloscope for output signal measurement.

- Differential probes for precise voltage measurements.

- Impedance analyzer or vector network analyzer (VNA).

- Precision resistor measurement tool.

##### Connections:

- Connect the signal generator to the PS_GTR differential input pins with proper AC coupling.

- Connect the oscilloscope and impedance analyzer to the output pins for measurements.

#### Test Steps:

##### Differential Input Voltage Verification:

- Generate a differential signal with a peak-to-peak voltage (DVPPIN) ranging from 100 mV to 1200 mV.

- Measure the input voltage at the PS_GTR differential input pins.

- Record the results and verify the input voltage is within the range of 100–1200 mV.

##### Single-Ended Input Voltage Verification:

- Measure the single-ended input voltage (VIN) referenced to GND.

- Verify the voltage is within the range of 75 mV to VPS_MGTRAVCC.

##### Common Mode Input Voltage Verification:

- Measure the common mode input voltage (VCMIN).

- Verify that the value is 0 mV (as specified).

##### Differential Output Voltage Verification:

- Configure the transmitter for maximum output swing.

- Measure the differential peak-to-peak output voltage (DVPPOUT).

- Verify that the output swing is ≥ 800 mV.

##### Common Mode Output Voltage Verification:

- For AC-coupled outputs: Verify the common mode output voltage (VCMOUTAC) satisfies the equation: TBD

##### Resistance Verification:

- Measure the differential input resistance (RIN) and verify it is 100Ω.

- Measure the differential output resistance (ROUT) and verify it is 100Ω.

##### Calibration Resistor Verification:

- Measure the resistor value between the calibration resistor pin and GND (RMGTRREF).

- Verify the resistance is within 497.5Ω to 502.5Ω.

##### Output Skew Verification:

- Measure the intra-pair skew between TXP and TXN.

- Verify that the skew (TOSKEW) is ≤ 20 ps.

##### AC Coupling Capacitor Verification:

- Ensure the external AC coupling capacitor has a value of 100 nF.

Table 40. PS_GTR Signal Test Results

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| DVPPIN (mV) |  | 100–1200 |  |
| VIN (Single-Ended, mV) |  | 75–VPS_MGTRAVCC |  |
| VCMIN (mV) |  | 0 |  |
| DVPPOUT (mV) |  | ≥ 800 |  |
| VCMOUTAC |  | Equation-based |  |
| RIN (Ω) |  | 100 |  |
| ROUT (Ω) |  | 100 |  |
| RMGTRREF (Ω) |  | 497.5–502.5 |  |
| TOSKEW (ps) |  | ≤ 20 |  |
| AC Coupling Capacitor (nF) |  | 100 |  |

### Functional Test for P28V Voltage Drop Verification

#### Objective:

- Verify that the voltage drop of the P28V power input is less than 50mV between the input connector and each output connector to the Octo backplanes while operating per 3.2.1.1.22 P28V POWER Characteristics.

#### Setup:

##### Equipment:

- Digital multimeter (DMM) or high-accuracy voltage measurement device with mV resolution.

- Load device capable of simulating operating conditions as defined in 3.2.1.1.22 P28V POWER Characteristics.

##### Connections:

- Connect the P28V power input at J0-F8 (or equivalent input pin).

- Connect the P28V power return signal at J0-D1 (or equivalent reference return pin).

- Attach the load device to each respective P28V power output pin (e.g., J1-F8, J2-F8, etc.), ensuring the correct reference return pins are connected.

###### Load Configuration:

- Configure the load device to simulate the operating conditions as specified in 3.2.1.1.22.

#### Test Steps:

##### Baseline Measurement:

- With no load applied, measure the voltage at the input connector (J0-F8) and record the baseline voltage VINPUT

- Voltage Drop Measurement:

- Apply the specified load conditions to the system.

- For each output connector (J1-F8, J2-F8, ... J7-F8, J249, etc.):

  - Measure the voltage between the respective power output pin (e.g., J1-F8) and the reference return pin (e.g., J1-D1). Record the voltage VOUTPUT.

  - Calculate the voltage drop ΔV.

- Record all measurements in Table 41.

##### Compliance Verification:

Verify that the voltage drop ΔV for each output connector is less than 50mV.

##### Acceptance Criteria:

- The voltage drop ΔV for each output connector must not exceed 50mV under the specified operating conditions.

Table 41. P28V Voltage Drop Test Results

| Output Connector | Reference Return | VINPUT | VOUTPUT | Voltage Drop ΔV (mV) | Pass/Fail |
| --- | --- | --- | --- | --- | --- |
| J1-F8 | J1-D1 |  |  |  |  |
| J2-F8 | J2-D1 |  |  |  |  |
| J3-F8 | J3-D1 |  |  |  |  |
| J4-F8 | J4-D1 |  |  |  |  |
| J5-F8 | J5-D1 |  |  |  |  |
| J6-F8 | J6-D1 |  |  |  |  |
| J7-F8 | J7-D1 |  |  |  |  |
| J249 | J2415 |  |  |  |  |
| J2517 | J2518 |  |  |  |  |
| J2610 | J269 |  |  |  |  |
| J2716 | J2717 |  |  |  |  |
| J2814 | J2814 |  |  |  |  |

### Functional Test for P48V Voltage Drop Verification

#### Objective:

- Verify that the voltage drop of the P48V power input is less than 100mV between the input connector and each output connector to the Octo backplanes while operating per 3.2.1.1.23 P48V POWER Characteristics.

#### Setup:

##### Equipment:

- Digital multimeter (DMM) or high-accuracy voltage measurement device with mV resolution.

- Load device capable of simulating operating conditions as defined in 3.2.1.1.23 P48V POWER Characteristics.

##### Connections:

- Connect the P48V power input at J0-E17 (or equivalent input pin).

- Connect the P48V power return signal at J0-C13 (or equivalent reference return pin).

- Attach the load device to each respective P48V power output pin (e.g., J1-E17, J2-E17, etc.), ensuring the correct reference return pins are connected.

###### Load Configuration:

- Configure the load device to simulate the operating conditions as specified in 3.2.1.1.23.

#### Test Steps:

##### Baseline Measurement:

- With no load applied, measure the voltage at the input connector (J0-E17) and record the baseline voltage VINPUT.

##### Voltage Drop Measurement:

- Apply the specified load conditions to the system.

##### For each output connector (J1-E17, J2-E17, ... J7-E17, J246):

- Measure the voltage between the respective power output pin (e.g., J1-E17) and the reference return pin (e.g., J1-C13). Record the voltage VOUTPUT.

- Calculate the voltage drop VDELTA as: VDELTA=VINPUT-VOUTPUT

- Record all measurements in Table 42.

##### Compliance Verification:

- Verify that the voltage drop (VDELTA) for each output connector is less than 100mV.

##### Acceptance Criteria:

- The voltage drop (ΔV\Delta VΔV) for each output connector must not exceed 100mV under the specified operating conditions.

Table 42. P48V Voltage Drop Test Results

| Output Connector | Reference Return | VINPUT | VOUTPUT | Voltage Drop ΔV (mV) | Pass/Fail |
| --- | --- | --- | --- | --- | --- |
| J1-E17 | J1-C13 |  |  |  |  |
| J2-E17 | J2-C13 |  |  |  |  |
| J3-E17 | J3-C13 |  |  |  |  |
| J4-E17 | J4-C13 |  |  |  |  |
| J5-E17 | J5-C13 |  |  |  |  |
| J6-E17 | J6-C13 |  |  |  |  |
| J7-E17 | J7-C13 |  |  |  |  |
| J246 | J243 |  |  |  |  |

### Functional Test for TR_SPARES Electrical Routing Verification

#### Objective:

- Verify that the item provides proper electrical routing for 64 single-ended spare signals (TR_SPARES) to the Octo backplanes as specified, ensuring they are functional and meet the 1.8V LVCMOS standard for signal integrity.

#### Setup:

##### Equipment:

- Signal generator capable of producing 1.8V LVCMOS logic signals.

- Oscilloscope with high-impedance probes for measuring signal integrity, voltage levels, and timing.

- Test fixture to route signals to their respective pins on the Octo backplanes.

##### Connections:

- Connect the signal generator to each TR_SPARE signal input (e.g., J0-A2, J0-A4, etc.) for the specific TR_SPARE being tested.

- Connect the oscilloscope probe to the corresponding output pin for the Octo backplane (as routed).

- Ensure the ground reference is connected correctly to ensure accurate measurements.

#### Test Steps:

##### Signal Propagation Verification:

- Inject a square wave signal (1 MHz, 1.8V amplitude) from the signal generator into each TR_SPARE input pin (e.g., J0-A2, J0-A4, ..., J7-A16).

- Verify the signal is routed correctly to the corresponding Octo backplane output without distortion.

- Record the observed waveform in Table 43.

##### Voltage Level Verification:

- Measure the signal high level (VOH) and low level (VOL) at the Octo backplane output using the oscilloscope.

- Verify that:

  - VOH ≥ 1.35V (75% of VDD).

  - VOL ≤ 0.45V (25% of VDD).

##### Rise and Fall Time Verification:

- Measure the rise time (10% to 90%) and fall time (90% to 10%) of the signal at the Octo backplane output.

- Verify that the rise and fall times are ≤ 10ns.

##### Signal Integrity Verification:

- Observe the signal for overshoot, undershoot, ringing, or noise at the output.

- Verify that any deviations are within ±10% of the nominal 1.8V LVCMOS levels.

##### Functional Mapping Verification:

- Confirm that each TR_SPARE input pin maps correctly to its designated Octo backplane output.

- Record the functional mapping in Table 43.

##### Acceptance Criteria:

- All TR_SPARE signals must meet the 1.8V LVCMOS standard for voltage levels, timing, and signal integrity.

- Each signal must propagate to the correct Octo backplane output without distortion.

Table 43. TR_SPARES Electrical Routing Test Results

| Input Pin | Output Pin | VOH (V) | VOL (V) | Rise Time (ns) | Fall Time (ns) | Signal Integrity (Pass/Fail) | Functional Mapping (Pass/Fail) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| J0-A2 | Octo Backplane |  |  |  |  |  |  |
| J0-A4 | Octo Backplane |  |  |  |  |  |  |
| J0-A6 | Octo Backplane |  |  |  |  |  |  |
| J0-A8 | Octo Backplane |  |  |  |  |  |  |
| ... | ... |  |  |  |  |  |  |
| J7-A16 | Octo Backplane |  |  |  |  |  |  |

### Functional Test for TR_IF_TX Isolation Verification

#### Objective:

- Verify that the item isolates transmit traces (TR_IF_TX signals) from one another with a minimum isolation of 45 dB across the IF bandwidth in a 50-ohm system.

#### Setup:

##### Equipment:

- Vector network analyzer (VNA) or spectrum analyzer with a signal generator for S-parameter and isolation measurements.

- 50-ohm matched test cables and termination loads.

- Proper test fixture to access each TR_IF_TX signal.

##### Connections:

- Connect the VNA port 1 to the transmitting trace (e.g., J402 TR5_DAC_IF).

- Connect the VNA port 2 to an adjacent trace (e.g., J404 TR4_DAC_IF) or any other trace being tested for isolation.

- Ensure all unused traces are terminated with 50-ohm loads to prevent reflections and ensure accurate measurements.

#### Test Steps:

#### Isolation Measurement Setup:

- Configure the VNA to measure S21 (insertion loss between port 1 and port 2).

- Sweep the frequency across the entire IF bandwidth (e.g., 0.1 MHz to 3 GHz).

##### Measurement for Each Trace Pair:

- For each TR_IF_TX trace (e.g., J402, J404, etc.):

  - Connect port 1 of the VNA to the trace under test as the active transmitter.

  - Connect port 2 of the VNA to an adjacent trace as the passive receiver.

  - Sweep the frequency range and record the measured S21 (dB) values.

##### Repeat Measurements:

- Repeat the isolation measurement for all combinations of trace pairs.

- Ensure to measure isolation between all directly adjacent traces as well as non-adjacent traces.

##### Compliance Check:

- Verify that the measured S21 values are ≤ -45 dB across the entire IF bandwidth.

##### Acceptance Criteria:

- The isolation between any two TR_IF_TX traces must be greater than 45 dB across the IF bandwidth in a 50-ohm system.

Table 44. TR_IF_TX Isolation Test Results

| Active Trace | Passive Trace | Frequency Range (MHz) | Measured Isolation (dB) | Pass/Fail |
| --- | --- | --- | --- | --- |
| J402 (TR5_DAC_IF) | J404 (TR4_DAC_IF) | 0.1–3000 |  |  |
| J402 (TR5_DAC_IF) | J406 (TR3_DAC_IF) | 0.1–3000 |  |  |
| J404 (TR4_DAC_IF) | J408 (TR2_DAC_IF) | 0.1–3000 |  |  |
| ... | ... | ... |  |  |

Notes:

- Ensure that all unused TR_IF_TX traces are terminated with 50-ohm loads to maintain test accuracy.

- Use high-quality test cables and calibration to minimize test setup errors.

- Record any anomalies, and ensure the isolation meets or exceeds the 45 dB requirement.

### Functional Test for TR_IF_RX Isolation Verification

#### Objective:

- Verify that the item isolates receive traces (TR_IF_RX signals) from one another with a minimum isolation of 45 dB across the IF bandwidth in a 50-ohm system.

#### Setup:

##### Equipment:

- Vector network analyzer (VNA) or spectrum analyzer with a signal generator for S-parameter and isolation measurements.

- 50-ohm matched test cables and termination loads.

- Proper test fixture to access each TR_IF_RX signal.

##### Connections:

- Connect the VNA port 1 to the receiving trace (e.g., J401 TR5_ADC_IF).

- Connect the VNA port 2 to an adjacent trace (e.g., J403 TR4_ADC_IF) or any other trace being tested for isolation.

- Ensure all unused traces are terminated with 50-ohm loads to prevent reflections and ensure accurate measurements.

#### Test Steps:

#### Isolation Measurement Setup:

- Configure the VNA to measure S21 (insertion loss between port 1 and port 2).

- Sweep the frequency across the entire IF bandwidth (e.g., 0.1 MHz to 3 GHz).

##### Measurement for Each Trace Pair:

- For each TR_IF_RX trace (e.g., J401, J403, etc.):

  - Connect port 1 of the VNA to the trace under test as the active receiver.

  - Connect port 2 of the VNA to an adjacent trace as the passive trace.

  - Sweep the frequency range and record the measured S21 (dB) values.

##### Repeat Measurements:

- Repeat the isolation measurement for all combinations of trace pairs.

- Ensure to measure isolation between all directly adjacent traces as well as non-adjacent traces.

##### Compliance Check:

- Verify that the measured S21 values are ≤ -45 dB across the entire IF bandwidth.

##### Acceptance Criteria:

- The isolation between any two TR_IF_RX traces must be greater than 45 dB across the IF bandwidth in a 50-ohm system.

Table 45. TR_IF_RX Isolation Test Results

| Active Trace | Passive Trace | Frequency Range (MHz) | Measured Isolation (dB) | Pass/Fail |
| --- | --- | --- | --- | --- |
| J401 (TR5_ADC_IF) | J403 (TR4_ADC_IF) | 0.1–3000 |  |  |
| J401 (TR5_ADC_IF) | J405 (TR3_ADC_IF) | 0.1–3000 |  |  |
| J403 (TR4_ADC_IF) | J407 (TR2_ADC_IF) | 0.1–3000 |  |  |
| ... | ... | ... |  |  |

Notes:

- Ensure that all unused TR_IF_RX traces are terminated with 50-ohm loads to maintain test accuracy.

- Use high-quality test cables and calibration to minimize test setup errors.

- Record any anomalies, and ensure the isolation meets or exceeds the 45 dB requirement.

### Functional Test for AXI Chip2Chip Aurora Bus Verification

#### Objective:

- Verify that the AXI Chip2Chip Aurora interface provides three lanes (3 RX / 3 TX) that operate at 25 Gbps each and implement the Aurora 64B/66B protocol per Xilinx SP011 specification.

#### Test Setup:

##### Equipment and Tools:

- FPGA or hardware with firmware implementing the Aurora 64B/66B protocol.

- Host FPGA/processor console interface for status monitoring.

- High-speed signal generator and analyzer for testing lane bandwidth.

- Bit error rate tester (BERT) for data integrity testing.

- Oscilloscope with sufficient bandwidth for signal integrity measurements.

##### Connections:

- Ensure all RX and TX lanes are properly connected as per the pinout table provided.

- Connect the host processor to a console interface to monitor status and performance metrics.

- Terminate unused lanes appropriately for accurate measurements.

#### Test Steps:

##### Lane Initialization:

- Power up the device and establish communication between the host FPGA and the endpoint.

##### Verify lane initialization status using the console interface:

- Check for READY or LINK UP status for all RX and TX lanes.

- Record any errors or warnings during initialization.

##### Lane Bandwidth Verification:

- Configure the Aurora firmware to operate each lane at 25 Gbps.

- Use a high-speed signal generator to inject test data into each TX lane (e.g., J10-A15, J10-C14, J10-E13).

- Measure the received data on the corresponding RX lanes (e.g., J10-A9, J10-C8, J10-E7) using a signal analyzer.

- Verify that each lane achieves 25 Gbps bandwidth without errors.

##### Protocol Compliance:

- Inject protocol-compliant frames into the TX lanes (e.g., J10-C14, J10-E13, J10-G14) using a BERT or FPGA-based test generator.

- Monitor the RX lanes for proper frame decoding and error-free reception.

##### Verify the following via the console interface:

- Frame alignment.

- Error detection and reporting.

- Data integrity metrics such as bit error rate (BER ≤ 1e-12).

##### Signal Integrity Measurements:

- Use an oscilloscope to capture eye diagrams for each TX lane at 25 Gbps.

- Verify that the eye opening meets the required specifications for the Aurora protocol.

- Measure jitter and rise/fall times to ensure compliance with timing requirements.

##### Status Monitoring:

- Read status registers from the host processor console for each lane, including:

  - Link status (e.g., LINK UP or DOWN).

  - Error counters (e.g., frame loss, alignment errors).

  - Latency and throughput metrics.

- Verify all metrics meet the expected thresholds for error-free operation.

##### Acceptance Criteria:

- Each RX/TX lane must operate at 25 Gbps with a bit error rate (BER) ≤ 1e-12.

- The Aurora 64B/66B protocol must pass all compliance checks, including frame alignment and error reporting.

- Signal integrity metrics (e.g., eye diagram, jitter) must meet protocol specifications.

- All status indicators (e.g., LINK UP) must confirm proper initialization and operation of all lanes.

Table 46. AXI Chip2Chip Lane Test Results

| TX Lane | RX Lane | Bandwidth (Gbps) | BER | Signal Integrity (Pass/Fail) | Protocol Compliance (Pass/Fail) | Status |
| --- | --- | --- | --- | --- | --- | --- |
| J10-A15 | J10-A9 |  |  |  |  |  |
| J10-C14 | J10-C8 |  |  |  |  |  |
| J10-E13 | J10-E7 |  |  |  |  |  |
| ... | ... |  |  |  |  |  |

Notes:

- Ensure the firmware build on the FPGA/processor supports the Aurora 64B/66B protocol and includes proper error detection and reporting.

- Debug and document any observed anomalies or deviations during testing.

- Maintain logs of all status outputs from the host console for verification and traceability.

### General Test for Operational Initialization Requirements

#### Objective:

- Verify that the item transitions to the initialization state, performs all required initialization functions, and completes initialization within the specified duration after applying P28V input power.

#### Setup:

##### Equipment:

- Power supply capable of delivering P28V per 3.2.1.1.22 P28V POWER Characteristics.

- Digital oscilloscope or logic analyzer to monitor initialization timing and signal transitions.

- Test fixture to access relevant processor, digital electronics, and output signals.

- Communication interface (e.g., UART, JTAG, or Ethernet) to monitor processor status and boot activity.

- Multimeter to verify power voltage levels at the item input.

##### Connections:

- Connect the P28V power supply to the item's input power terminals.

###### Connect test equipment to monitor:

- Power-on-reset signal.

- Output signals.

- Processor initialization activity (e.g., via debug or status signals).

#### Test Steps:

##### Pre-Test Setup:

- Ensure the item is in its unpowered state.

- Configure the power supply to meet P28V input characteristics, including voltage, current, and ramp rate as specified in 3.2.1.1.22.

- Verify all test equipment is correctly connected and configured.

##### Power Application:

- Enable the power supply and apply P28V to the item.

- Record the time at which the P28V input power reaches its minimum specified value.

##### Monitor Initialization:

- Observe the power-on-reset signal to confirm it is asserted after full power is attained.

- Verify that the following initialization functions occur in sequence:

  - Digital electronics are initialized.

  - Processor(s) are reset.

  - Output signals transition to their initialized states.

##### Initialization Timing:

- Measure the time between the application of power (P28V reaching its minimum value) and the completion of initialization (processor fetching the first boot instruction).

- Verify that this duration is ≤ 5 seconds.

##### Processor Boot Verification:

- Confirm that the processor boots from NVM after reset.

- Monitor the processor's status (via debug or communication interface) to verify successful boot activity.

##### Post-Test Checks:

- Verify the initialized states of all output signals per the specifications.

- Confirm no errors or abnormal conditions were encountered during the test.

##### Acceptance Criteria:

- The item transitions to the initialization state upon application of P28V power.

- The item performs all specified initialization functions, including:

  - Digital electronics initialization.

  - Processor reset and boot from NVM.

  - Output signal initialization.

  - The initialization duration is ≤ 5 seconds from the time P28V reaches its minimum value.

  - The power-on-reset signal correctly initializes the logic after full power is attained.

Table 47. Test Data

| Test Step | Measurement/Observation | Specification | Result (Pass/Fail) |
| --- | --- | --- | --- |
| Power Application Time |  | P28V reaches minimum value |  |
| Initialization Duration |  | ≤ 5 seconds |  |
| Power-On-Reset Signal |  | Correctly asserted and deasserted |  |
| Processor Boot Status |  | Boots from NVM |  |
| Output Signal States |  | Initialized per specification |  |

### Local Power Rail Test Procedure

#### Objective

Verify that the specified power rail operates within its defined voltage, regulation, and ripple specifications during normal operation.

#### Setup

##### Prepare Equipment:

- Use a digital multimeter (DMM) to measure static voltage.

- Use an oscilloscope to measure ripple and assess signal integrity.

##### Configuration:

- Reference the top-level Power Integrity Test Procedure for the voltage rail net name, probe points, and test limits.

- Ensure a stable and controlled environment to minimize external noise or fluctuations.

#### Test Steps

##### Voltage Verification

- Connect the DMM probes to the specified net and GND as per the probe points provided in the top-level procedure.

- Measure the DC voltage of the rail under its operational conditions.

- Record the measured voltage in the results table.

- Verify that the measured voltage is within the specified range (e.g., ±100mV).

##### Ripple Voltage Verification

- Connect the oscilloscope probes to the specified net with AC coupling enabled.

- Configure the oscilloscope as follows:

  - Vertical Scale: 20mV/div (or as specified for the rail).

  - Horizontal Scale: 10µs/div.

- Measure the peak-to-peak ripple voltage of the rail.

- Record the measured ripple in the results table.

- Verify that the ripple voltage does not exceed the specified maximum value (e.g., ≤60mVpp).

##### Regulation Verification

- Measure the voltage stability over the observation period under operational conditions.

- Identify any transient deviations in voltage (e.g., during load changes or power state transitions).

- Record the observed regulation performance in the results table.

- Verify the regulation meets the specified range (e.g., ±100mV).

#### Results

Record the following parameters for each power rail in the top-level Power Integrity Results Table:

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Voltage (V) |  | Refer to Limits |  |
| Regulation (V) |  | Refer to Limits |  |
| Ripple (Vpp) |  | Refer to Limits |  |

Reference

For voltage rail net names, probe points, and specification limits, refer to the top-level Power Integrity Test Procedure.

# Appendix A

## Hardware Verification Test (HVT) for Master Array Processor (MAP) PWA

### Objective

The Hardware Verification Test (HVT) ensures that the Master Array Processor (MAP) PWA is properly constructed and operational after manufacturing. This procedure verifies basic functionality, connectivity, and hardware integrity to confirm the PWA is free of manufacturing defects. The HVT does not serve as a design verification but rather validates that the hardware is assembled correctly and capable of supporting further functional and system-level testing.

### Test Scope

The test validates the following aspects of the MAP PWA:

#### Power Integrity

This test validates the power integrity of the MAP PWA by ensuring the following:

##### Voltage Compliance

Verifies that all voltage rails meet their specified voltage and current requirements.

##### Stability and Regulation

Checks for voltage stability, proper regulation, and acceptable levels of ripple and noise.

The following voltage rail signals will be tested and verified during this procedure:

- MGTYAVCC

- MGTYAVTT

- MGTYVCCAUX

- P12V_PRS

- P12V_SRS0

- P12V_SRS1

- P12V_SRS2

- P1V1

- P1V2

- P1V8

- P2V5

- P3V3

- P6V

- PL_VCCINT

- PL_VPP

- PL_VREF

- PL_VTT

- PS_MGTRAVCC

- PS_MGTRAVTT

- PS_VCCINT

- PS_VPLL

- PS_VPP

- PS_VREF

Each voltage rail will be subjected to tests for compliance, regulation, and ripple noise as defined in the Power Integrity Test Procedures.

#### Connectivity

This section validates the connectivity of major subsets of signals—Power, Data, and Control—across the MAP PWA and its connected locations or categories. The objective is to ensure proper signal routing, continuity, and termination for reliable operation and integration.

The following Connectivity Groups will be tested:

##### PRS, SRS0-2

###### Power:

- P12V_PRS, P12V_SRS0-2: Verifies power distribution from the MAP PWA to PRS and SRS0-2 subsystems.

###### Data:

- Differential IF_ADC: Confirms connectivity for differential analog-to-digital converter inputs.

- Differential IF_DAC: Validates connectivity for differential digital-to-analog converter outputs.

- EXT_CLK_IN: Ensures proper clock signal routing for PRS and SRS0-2.

- Aurora Bus: Verifies high-speed serial communication over the Aurora bus to PRS and SRS0-2.

- C2C_AXI: Confirms proper chip-to-chip communication via the AXI bus to PRS and SRS0-2.

##### Power PWA

###### Power:

- P28V_POWER: Validates power delivery at 28V from the MAP PWA to the Power PWA.

- P48V_POWER: Ensures connectivity for the 48V power rail.

- P48V_ENABLE: Confirms proper routing and functionality of the enable signal for 48V power.

- FAN_ENABLE: Validates connectivity for fan control signals.

##### CAL, RXLO, and TXLO PWAs (Three Separate PWAs)

###### Power:

- P28V_POWER: Verifies proper 28V power delivery to CAL, RXLO, and TXLO PWAs.

- Control:

- Serial Communication: Ensures proper signal routing for serial communication between the MAP PWA and each of these PWAs.

###### Data:

- MBUS: Confirms connectivity for data transmission over the MBUS to CAL, RXLO, and TXLO PWAs.

##### TR0-63 (via Octo BP)

###### Power:

- P28V_POWER: Validates 28V power delivery to TR0-63 through the Octo BP.

- P48V_POWER: Ensures 48V power delivery is properly routed to TR0-63.

- Control:

- TR_EXT_SYNC: Verifies connectivity for external synchronization signals to TR0-63.

- CBUS/MBUS: Confirms proper routing of control signals over CBUS and MBUS to TR0-63.

###### Data:

- SE IF_ADC: Validates single-ended analog-to-digital converter input connectivity to TR0-63.

- SE IF_DAC: Confirms single-ended digital-to-analog converter output connectivity to TR0-63.

Each connectivity group will be tested using continuity checks, signal integrity validation, and functional tests to confirm proper routing and termination. This ensures that power, data, and control signals are delivered as expected to their respective destinations.

#### Basic Functional Checks

This section validates the basic functionality of the MAP PWA by ensuring that all critical components and subsystems operate as expected under nominal conditions. The following functions must be tested and confirmed:

##### Power Control

- Enable P48V Power: Confirms the MAP PWA can enable and distribute 48V power.

- Enable Fan: Verifies that the MAP PWA can control fan enable signals.

##### Processor Subsystem (PS)

- Program Processor via JTAG: Ensures the processor can be programmed through the JTAG interface.

- Boot Processor: Confirms successful processor initialization and operation.

- PS DDR4 Memory Test: Verifies data integrity for the PS DDR4 memory.

- PS NVM Memory Test: Confirms proper functionality and reliability of PS non-volatile memory.

##### Virtex Subsystem (VX)

- Program Virtex FPGA via JTAG: Validates that the Virtex FPGA can be successfully programmed using the JTAG interface.

- Boot FPGA: Ensures proper initialization and operation of the FPGA.

- PL_DDR4 Memory Test: Confirms data integrity for the programmable logic DDR4 memory.

- Virtex NVM Memory Test: Validates the functionality and reliability of Virtex non-volatile memory.

##### PRS and SRS0-2 Subsystems

- Program Processors via JTAG: Ensures processors on PRS and SRS0-2 subsystems can be programmed through the JTAG interface.

- Boot Processors: Confirms successful initialization and operation of the processors on PRS and SRS0-2.

##### Clock Initialization

- Initialize DAC/ADC Clocks: Verifies proper configuration and stability of the DAC and ADC clock signals.

##### PRS Subsystem Communication Interfaces

- Ethernet0 and Ethernet1: Confirms proper initialization and operation of two Ethernet interfaces.

- AXI Chip-to-Chip: Verifies communication through the AXI chip-to-chip interface.

- PRS_PS Ethernet: Ensures reliable Ethernet communication between PRS and PS subsystems.

- INS and TE_INS: Validates data exchange and communication through the INS and TE_INS interfaces.

- CBUS/MBUS: Confirms proper operation of CBUS and MBUS interfaces.

##### TR0-63 Array Initialization

- Initiate TR0-63 Array: Ensures the MAP PWA can initialize and control the TR0-63 array.

##### Command and Control Operations

- Write Commands to Power PWA, RX/TX/CAL PWAs, and TR0-63 via CBUS: Verifies the ability to send control commands to connected subsystems.

- Control Array via Ethernet0: Confirms that the MAP PWA can control the array through Ethernet0.

- Basic Commanded Built-In-Test (BIT): Validates the execution of a basic commanded BIT to assess system health and operation.

- Each of the listed functions will be verified through specific test steps designed to confirm initialization, execution, and basic operation. Any failures will be logged for further analysis to ensure the MAP PWA meets its functional requirements.

#### Communication Interfaces

This section validates the communication interfaces of the MAP PWA, ensuring robust and reliable data exchange between internal subsystems and external devices. Each interface is tested for functionality, signal integrity, and protocol compliance.

The following Communication Interfaces will be tested:

##### Ethernet BASE1000-TX, BASE100-TX, and BASE10-TX

- Ensures proper connectivity and communication across Ethernet interfaces operating in Gigabit, Fast Ethernet, and standard Ethernet modes.

- Verifies signal integrity, link initialization, and data transmission reliability.

##### Ethernet SGMII

- Tests serial gigabit media-independent interface (SGMII) for Ethernet communications.

- Confirms proper signal routing, stability, and communication performance.

##### Serial Communication to CAL PWA

- Verifies serial data exchange between the MAP PWA and CAL PWA.

- Confirms proper baud rate, data integrity, and termination.

##### Serial Communication to RXLO and TXLO PWAs

- Validates serial communication paths to both the RXLO and TXLO PWAs.

- Ensures data is transmitted and received correctly with minimal errors.

- USB for PS, PRS, and SRS0-2

- Confirms USB interface functionality for PS, PRS, and SRS0-2 subsystems.

- Validates USB signal integrity and correct data transfer under operational conditions.

##### PCIe (PRS_PS)

- Verifies PCIe communication between the PRS and PS subsystems.

- Assesses link initialization, data transfer reliability, and overall stability.

##### TE INS Interface

- Confirms connectivity and data exchange with the TE INS interface.

- Ensures proper initialization and continuous data flow.

##### TR0-63 EXT_Sync

- Validates the external synchronization signals (EXT_Sync) associated with the TR0-63 interface.

- Ensures synchronization integrity across all channels.

##### PRS AURORA and PRS AXI

- Verifies communication on high-speed serial interfaces using Aurora and AXI protocols.

- Confirms data integrity, link stability, and high-bandwidth data transfer capabilities.

Each communication interface will undergo testing for signal quality, protocol compliance, and data integrity to ensure robust communication across the MAP PWA.

### Test Setup

#### Equipment Required

- Power Supplies: Programmable power supplies capable of delivering and monitoring P28V and P48V power with configurable current limits and voltage ramps.

- Digital Multimeter (DMM): For precise voltage, current, and resistance measurements across power rails and signal lines.

- Oscilloscope: High-bandwidth oscilloscope with at least 4Ghz bandwidth for verifying signal integrity, ripple, and timing characteristics on critical signals, including clocks and communication lines.

- Logic Analyzer: For capturing and analyzing digital signals, ensuring proper operation of buses such as CBUS, MBUS, and other control and data paths.

- Signal Generator: Capable of generating waveforms in the S-band (2-4 GHz) and X-band (8-12GHz) frequency ranges.

  - Used to generate input stimuli, such as test clocks or specific signal patterns, for verifying the response of various subsystems.

- JTAG Debugger: Required for programming and debugging the PS and Virtex FPGA as well as PRS and SRS0-2 processors.

- Communication Interface Tools: Includes UART debuggers, Ethernet analyzers, and other protocol-specific tools for monitoring and testing communication interfaces like Ethernet (BASE1000-TX, BASE100-TX, BASE10-TX), INS, and TE_INS.

  - USB Ports: Serve as the primary debug interface for the processor systems (PS, PRS, SRS0-2), acting as console ports to interact with and monitor the software running on these systems.

- Programmable Electronic Load: To simulate load conditions for P48V and P28V power rails during stress and ripple testing.

- Breakout Boxes: Breakout boxes for each connector on MAP PWA.

- Software and Firmware Tools: Software tools for programming processors and FPGAs, executing Built-In-Tests (BITs), and collecting diagnostic data.

### First Article Test Procedure for MAP PWA

#### Visual Inspection

##### Action:

Perform the Visual Inspection procedure as detailed in "Safe-To-Turn-On Measurements", Section 3.6.1.1.

##### Verify:

- Orientation of active components, diodes, tantalum capacitors, and connectors.

- Solder joints for cracks, breaks, or poor adhesion to pins.

- Passive components for tombstone or billboard effects.

- Absence of FOD or debris that may damage circuits during power-up.

- Correct installation or absence of Do Not Populate (DNP) components per design documentation.

##### Reference:

Follow checklist items and comments section in Table 8 of the "Safe-To-Turn-On Measurements."

##### Result Data:

Record observations and comments in the Visual Inspection Log.

##### Pass Criteria:

No defects, anomalies, or missing components detected.

#### Impedance Checks on Input, Output Voltage Rail Nets and Resistors

##### Action:

Execute "Safe-To-Turn-On Measurements", Sections 3.6.1.2 to 3.6.1.5.

##### Scope:

- Power to Ground Isolation: Measure resistance between power and ground nets for each voltage rail.

- Power to Power Isolation: Verify resistance between distinct power rails.

- Ground to Ground Isolation / Continuity: Confirm resistance between ground nets.

##### Reference:

Use Tables 8, 9, 10 and 10a from the "Safe-To-Turn-On Measurements" for specific net pairs and resistance requirements.

##### Result Data:

- Record measured values in the respective isolation resistance tables.

- Identify any deviations from the required thresholds.

##### Pass Criteria:

All measured values meet or exceed the specified isolation, continuity, or resistance requirements.

#### Safe-to-Turn-On Validation

##### Action:

Follow the "Safe-To-Turn-On Measurements", Section 3.6.2.

##### Scope:

- Power-On Test Procedure (3.6.2.1):

- Ensure proper connections as shown in Figures 1–2.

- Configure the power supply for P28V operation with a ramp of 0.14V/ms and a current limit of 25A.

- Measure power-on current and voltage values across P28V, P12V, P1V8, P3V3, and other specified voltage nets.

##### Reference:

Use Tables 11, 12, and 13 in the "Safe-To-Turn-On Measurements" for net locations, expected values, and tolerances.

##### Result Data:

Record current and voltage results in the Power-On Safety Verification Table.

Note any anomalies, including overcurrent events or deviations in voltage regulation.

##### Pass Criteria:

Current and voltage values are within specified tolerances, with no anomalies observed.

#### Input Power Characteristics Validation

##### Action:

Perform "Input Power Characteristics", Section 3.6.3.

##### Scope:

Measure output voltage and ripple when the input voltage is set to 29V (Sections 3.6.3.2–3.6.3.3).

##### Reference:

Use Table 14 in the "Input Power Characteristics" for voltage and ripple specifications and net details.

##### Result Data:

Record measured voltage and ripple values in the Input Power Characteristics Table.

##### Pass Criteria:

Voltage and ripple values meet specified requirements across all monitored rails (e.g., P28V, P12V_PRS, P1V8).

#### Power Integrity Test Procedure for MAP PWA

##### Objective

Verify the power integrity of the MAP PWA by ensuring all voltage rails meet defined specifications for voltage compliance, ripple, and stability.

##### Procedure

###### Voltage Rails to Test

Execute the Local Power Rail Test Procedure for the voltage rails listed in Table 1. Reference the specified test points and limits for each rail.

###### Result Documentation

Record the measured values, specifications, and pass/fail status for each rail in the Power Integrity Results Table (Table 2).

Voltage Rail Details

Table 1. Voltage Rails, Test Points, and Limits

| Voltage Rail | Test Points (From/To) | Voltage (V) | Max Ripple (Vpp) |
| --- | --- | --- | --- |
| MGTYAVCC | L54-2 / U82-10 | 0.9 ± 0.036 | ≤ 12mV |
| MGTYAVTT | L81-2 / U82-10 | 1.2 ± 0.036 | ≤ 12mV |
| MGTYVCCAUX | L82-2 / U82-10 | 1.8 ± 0.054 | ≤ 18mV |
| P12V_PRS | D32-K / U82-10 | 12 ± 0.6 | ≤ 120mV |
| P12V_SRS0 | D20-K / U82-10 | 12 ± 0.6 | ≤ 120mV |
| P12V_SRS1 | D16-K / U82-10 | 12 ± 0.6 | ≤ 120mV |
| P12V_SRS2 | D29-K / U82-10 | 12 ± 0.6 | ≤ 120mV |
| P1V1 | U122-3 / U82-10 | 1.1 ± 0.033 | ≤ 11mV |
| P1V2 | U40-2 / U82-10 | 1.2 ± 0.036 | ≤ 12mV |
| P1V8 | L6-1 / U82-10 | 1.8 ± 0.054 | ≤ 18mV |
| P2V5 | U129-4 / U82-10 | 2.5 ± 0.075 | ≤ 25mV |
| P3V3 | U33-12 / U82-10 | 3.3 ± 0.1 | ≤ 33mV |
| P6V | U53-8 / U82-10 | 6 ± 0.18 | ≤ 60mV |
| PL_VCCINT | L77-2 / U82-10 | 0.85 ± 0.0255 | ≤ 8.5mV |
| PL_VPP | U89-3 / U82-10 | 2.5 ± 0.075 | ≤ 25mV |
| PL_VREF | U88-6 / U82-10 | 0.6 ± 0.0225 | ≤ 7.5mV |
| PL_VTT | U88-3 / U82-10 | 0.6 ± 0.0225 | ≤ 7.5mV |
| PS_MGTRAVCC | L11-2 / U82-10 | 0.85 ± 0.036 | ≤ 12mV |
| PS_MGTRAVTT | L6-2 / U82-10 | 1.8 ± 0.036 | ≤ 12mV |
| PS_VCCINT | L11-1 / U82-10 | 0.85 ± 0.0255 | ≤ 8.5mV |
| PS_VPLL | U31-4 / U82-10 | 1.2 ± 0.036 | ≤ 12mV |
| PS_VPP | U33-4 / U82-10 | 2.5 ± 0.075 | ≤ 25mV |
| PS_VREF | U40-6 / U82-10 | 0.6 ± 0.0225 | ≤ 7.5mV |
| PS_VTT | U40-3 / U82-10 | 0.6 ± 0.0225 | ≤ 7.5mV |

Power Integrity Results Table

Table 2. Test Results

| Voltage Rail | Measured Value (V) | Ripple (Vpp) | Pass/Fail |
| --- | --- | --- | --- |
| MGTYAVCC_PFLT | 0.8997 | .0027 | ( P / F ) |
| MGTYAVTT_ PFLT | 1.197 | .0025 | ( P / F ) |
| MGTYVCCAUX_ PFLT | 1.806 | .0027 | ( P / F ) |
| P12V_PRS | 11.99 | 0.0032 | ( P / F ) |
| P12V_SRS0 | 12.017 | 0.0053 | ( P / F ) |
| P12V_SRS1 | 12.013 | 0.0038 | ( P / F ) |
| P12V_SRS2 | 12.014 | 0.0038 | ( P / F ) |
| P1V1 | 1.09 | .00029 | ( P / F ) |
| P1V2 | 1.201 | .00028 | ( P / F ) |
| P1V8 | 1.799 | 0.00036 | ( P / F ) |
| P2V5 | 2.504 | .0013 | ( P / F ) |
| P3V3 | 3.31 | 0.0028 | ( P / F ) |
| P6V | 6.038 | .0021 | ( P / F ) |
| PL_VCCINT | 0.847 | .0011 | ( P / F ) |
| PL_VPP | 2.497 | .0027 | ( P / F ) |
| PL_VREF | 0.60 | .00089 | ( P / F ) |
| PL_VTT | 0.61 | .00087 | ( P / F ) |
| PS_MGTRAVCC | .847 | .0009 | ( P / F ) |
| PS_MGTRAVTT | 1.79 | .003 | ( P / F ) |
| PS_VCCINT | 0.847 | .007 | ( P / F ) |
| PS_VPLL | 1. 201 | .0009 | ( P / F ) |
| PS_VPP | 2.51 | .0007 | ( P / F ) |
| PS_VREF | 0.61 | .00083 | ( P / F ) |
| PS_VTT | 0.6 | .00082 | ( P / F ) |

##### Execution Instructions

- For each voltage rail listed in Table 1:

- Perform the Local Power Rail Test Procedure using the specified test points and limits.

- Measure and record the results for voltage and ripple in Table 2.

- Ensure all measured values meet the defined specifications in Table 1.

- Mark each voltage rail as Pass or Fail based on compliance with the limits.

#### PRS, SRS0-2 Connectivity Testing

##### Power Connectivity

###### Signals to Test:

| Net Name | Probe Point | Signal Type |
| --- | --- | --- |
| P12V_PRS | D32-K / U82-10 | P12V POWER |
| P12V_SRS0 | D20-K / U82-10 | P12V POWER |
| P12V_SRS1 | D16-K / U82-10 | P12V POWER |
| P12V_SRS2 | D29-K / U82-10 | P12V POWER |

###### Procedure:

Execute the Local Power Rail Test Procedure for these power signals to validate continuity, voltage compliance, and ripple performance.

###### Result Documentation:

Record results in Table 1 - Power Connectivity Results:

| Net Name | Measured Voltage (V) | Voltage Specification | Ripple (Vpp) | Ripple Specification | Pass/Fail | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| P12V_PRS | 12.02 | 12 ± 0.6 | 0.0035 | ≤120mV | Pass |  |
| P12V_SRS0 | 12.034 | 12 ± 0.6 | 0.0034 | ≤120mV | Pass |  |
| P12V_SRS1 | 12.02 | 12 ± 0.6 | 0.0035 | ≤120mV | Pass |  |
| P12V_SRS2 | 12.013 | 12 ± 0.6 | 0.0034 | ≤120mV | Pass |  |

##### RFSOM Installation and Power Up

###### Procedure

- Install PRS RFSOM PN: RTX16 on MAP PWA.

- Enable 28V Power.

- Record current displayed on 28V Power supply.

- Remove 28V Power.

- Repeat previous steps for SRS0, SRS1, and SRS2.

###### Result Documentation

Record results in Table 1 – PRS Installation and Power Up:

| Supply Name | RFSOM Name | Measured Current (A) | Voltage Specification | Pass/Fail | Comments |
| --- | --- | --- | --- | --- | --- |
| P28V | PRS | 1.52A | ≤ 5 A | Pass | SN00230A2 |
| P28V | SRS0 | 2.31A | ≤ 5 A | Pass | SN00218A2 |
| P28V | SRS1 | 3.10 | ≤ 5 A | Pass | SN00228A2 |
| P28V | SRS2 | 3.94 | ≤ 5 A | Pass | SN00214A2 (JTAG connector rotated) |

##### Data Connectivity

Signals to Test and Associated Procedures:

###### RFSOC JTAG

Procedure:

- Connect USB JTAG connector to MAP J29

- Use Vivado to Auto Connect to PRS RFSOC.

- Record PRS RFSOC deviceID.

- Repeat previous steps for SRS0 (J30), SRS1 (J31), and SRS2 (J32).

###### Result Documentation:

Record results in Table 2 - Data Connectivity Results:

| Signal Group | DeviceID | Pass/Fail | Comments |
| --- | --- | --- | --- |
| PRS | 147FE093 | Pass |  |
| SRS0 | 147FE093 | Pass |  |
| SRS1 | 147FE093 | Pass |  |
| SRS2 | 147FE093 | Pass |  |

#### Power PWA Connectivity Testing

##### Control Connectivity

Signals to Test and Associated Procedures:

###### P48V_ENABLE Signals

Net Names:

- P48V_ENABLE_N

- P48V_ENABLE_P

Signal Type: EIA-422

Procedure:

- Perform the P28V_DISCRETE Signal Test to validate the routing and enable functionality of the P48V power signals.

###### FAN_ENABLE Signals

Net Names:

- FAN_ENABLE_N

- FAN_ENABLE_P

Signal Type: EIA-422

Procedure:

- Perform the EIA-422 Signal Test to validate the routing and functionality of the fan control signals.

###### FAN_PS Signal

Net Name:

- FAN_PS

Signal Type: FAN_PS_SIG

Procedure:

- Perform the FAN_PS_SIG Signal Test to validate the power status feedback for the fan.

###### INS_RX and INS_TX Signals

Net Names:

- INS_RX_N, INS_RX_P

- INS_TX_N, INS_TX_P

Signal Type: EIA-422

Procedure:

- Perform the EIA-422 Signal Test to validate the routing and communication integrity for INS_RX and INS_TX signals.

###### Result Documentation

Record results in Table 2 - Power Connectivity Results:

| Signal Group | Signal Type | Net Name | Probe Point | Continuity Pass/Fail | Signal Integrity Pass/Fail | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| P48V_ENABLE | EIA-422 | P48V_ENABLE_N | J24-14 | Pass | Similar to: CAL_SPI_SCLK |  |
|  |  | P48V_ENABLE_P | J24-13 | Pass | Similar to: CAL_SPI_SCLK |  |
| FAN_ENABLE | EIA-422 | FAN_ENABLE_N | J24-34 | Pass | Similar to: CAL_SPI_SCLK |  |
|  |  | FAN_ENABLE_P | J24-18 | Pass | Similar to: CAL_SPI_SCLK |  |
| FAN_PS | FAN_PS_SIG | FAN_PS | J24-55 | Pass | Pass |  |
| INS_RX | EIA-422 | INS_RX_N | J24-66 | Pass | Similar to: CAL_SPI_SCLK |  |
|  |  | INS_RX_P | J24-65 | Pass | Similar to: CAL_SPI_SCLK |  |
| INS_TX | EIA-422 | INS_TX_N | J24-63 | Pass | Similar to: CAL_SPI_SCLK |  |
|  |  | INS_TX_P | J24-62 | Pass | Similar to: CAL_SPI_SCLK |  |

###### Execution Notes

- Ensure all test equipment is properly calibrated before testing.

- Perform each signal test using the referenced procedure and ensure the specified parameters are met.

- Record continuity and signal integrity results in the table above.

- Document any anomalies or deviations in the comments section.

- Escalate unresolved connectivity issues for further analysis.

#### CAL, RXLO, and TXLO PWAs Connectivity Testing

##### Power Connectivity

###### Signals to Test and Associated Procedures:

P28V_POWER

###### Procedure:

Perform the P28V POWER Signal Test to validate the 28V power delivery to CAL, RXLO, and TXLO PWAs.

##### Control Connectivity

Signals to Test and Associated Procedures:

###### CAL Signals

Net Names and Signal Types:

- CAL_DISC_N: EIA-422

- CAL_DISC_P: EIA-422

- CAL_SPI_CS_N_N: 1.8V LVCMOS

- CAL_SPI_CS_N_P: 1.8V LVCMOS

- CAL_SPI_SCLK_N: EIA-422

- CAL_SPI_SCLK_P: EIA-422

- CAL_SPI_SDO_N: EIA-422

- CAL_SPI_SDO_P: EIA-422

Procedure:

- Perform the EIA-422 Signal Test for EIA-422 signals and 1.8V LVCMOS Signal Test for LVCMOS signals.

###### RXLO Signals

Net Names and Signal Types:

- RXLO_DISC_N: EIA-422

- RXLO_DISC_P: EIA-422

- RXLO_SPI_CS_N_N: 1.8V LVCMOS

- RXLO_SPI_CS_N_P: 1.8V LVCMOS

- RXLO_SPI_SCLK_N: EIA-422

- RXLO_SPI_SCLK_P: EIA-422

- RXLO_SPI_SDO_N: EIA-422

- RXLO_SPI_SDO_P: EIA-422

Procedure:

- Perform the EIA-422 Signal Test for EIA-422 signals and 1.8V LVCMOS Signal Test for LVCMOS signals.

###### TXLO Signals

Net Names and Signal Types:

- TXLO_DISC_N: EIA-422

- TXLO_DISC_P: EIA-422

- TXLO_SPI_CS_N_N: 1.8V LVCMOS

- TXLO_SPI_CS_N_P: 1.8V LVCMOS

- TXLO_SPI_SCLK_N: EIA-422

- TXLO_SPI_SCLK_P: EIA-422

- TXLO_SPI_SDO_N: EIA-422

- TXLO_SPI_SDO_P: EIA-422

Procedure:

- Perform the EIA-422 Signal Test for EIA-422 signals and 1.8V LVCMOS Signal Test for LVCMOS signals.

##### Data Connectivity

Signals to Test and Associated Procedures:

###### CAL MBUS Signals

Net Names and Signal Types:

- CAL_MBUS_N: EIA-422

- CAL_MBUS_P: EIA-422

Procedure:

- Perform the EIA-422 Signal Test to validate MBUS data transmission.

###### RXLO MBUS Signals

Net Names and Signal Types:

- RXLO_MBUS_N: EIA-422

- RXLO_MBUS_P: EIA-422

Procedure:

- Perform the EIA-422 Signal Test to validate MBUS data transmission.

###### TXLO MBUS Signals

Net Names and Signal Types:

- TXLO_MBUS_N: EIA-422

- TXLO_MBUS_P: EIA-422

Procedure:

- Perform the EIA-422 Signal Test to validate MBUS data transmission.

###### Result Documentation

Record results in Table 3 - CAL, RXLO, and TXLO Connectivity Results:

| Signal Group | Signal Type | Net Name | Probe Point | Continuity Pass/Fail | Signal Integrity Pass/Fail | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| CAL_DISC | EIA-422 | CAL_DISC_N | J27-11 | Pass | Similar to: CAL_SPI_SCLK |  |
|  |  | CAL_DISC_P | J27-10 | Pass | Similar to: CAL_SPI_SCLK |  |
| RXLO_DISC | EIA-422 | RXLO_DISC_N | J25-12 | Pass | Similar to: CAL_SPI_SCLK |  |
|  |  | RXLO_DISC_P | J25-11 | Pass | Similar to: CAL_SPI_SCLK |  |
| TXLO_DISC | EIA-422 | TXLO_DISC_N | J26-13 | Pass | Similar to: CAL_SPI_SCLK |  |
|  |  | TXLO_DISC_P | J26-12 | Pass | Similar to: CAL_SPI_SCLK |  |
| CAL_SPI_CS_N | 1.8V LVCMOS | CAL_SPI_CS_N_N | J27-5 | Pass | Pass |  |
|  |  | CAL_SPI_CS_N_P | J27-4 | Pass | Pass |  |
| RXLO_SPI_CS_N | 1.8V LVCMOS | RXLO_SPI_CS_N_N | J25-6 | Pass | Pass |  |
|  |  | RXLO_SPI_CS_N_P | J25-5 | Pass | Pass |  |
| TXLO_SPI_CS_N | 1.8V LVCMOS | TXLO_SPI_CS_N_N | J26-5 | Pass | Pass |  |
|  |  | TXLO_SPI_CS_N_P | J26-4 | Pass | Pass |  |
| CAL_MBUS | EIA-422 | CAL_MBUS_N | J27-14 | Pass | Similar to: CAL_SPI_SCLK |  |
|  |  | CAL_MBUS_P | J27-13 | Pass | Similar to: CAL_SPI_SCLK |  |
| RXLO_MBUS | EIA-422 | RXLO_MBUS_N | J25-15 | Pass | Similar to: CAL_SPI_SCLK |  |
|  |  | RXLO_MBUS_P | J25-14 | Pass | Similar to: CAL_SPI_SCLK |  |
| TXLO_MBUS | EIA-422 | TXLO_MBUS_N | J26-16 | Pass | Similar to: CAL_SPI_SCLK |  |
|  |  | TXLO_MBUS_P | J26-15 | Pass | Similar to: CAL_SPI_SCLK |  |

###### Execution Notes

- Ensure all test equipment is properly calibrated prior to testing.

- Follow the specified procedure for each signal type and validate compliance with expected parameters.

- Record results in Table 3 and document any deviations or failures in the comments section.

- Escalate unresolved connectivity issues for further analysis.

| Pin | Name | Value |
| --- | --- | --- |
| J40-2 | TR5_DAC_IF | 165 mV |
| J40-4 | TR4_DAC_IF | 153 mV |
| J40-6 | TR3_DAC_IF | 160 mV |
| J40-8 | TR2_DAC_IF | 158 mV |
| J41-2 | TR1_DAC_IF | 151 mV |
| J41-4 | TR0_DAC_IF | 153 mV |
| J41-6 | TR7_DAC_IF | 161 mV |
| J41-8 | TR6_DAC_IF | 153 mV |
| J42-2 | TR13_DAC_IF | 152 mV |
| J42-4 | TR12_DAC_IF | 155 mV |
| J42-6 | TR11_DAC_IF | 149 mV |
| J42-8 | TR10_DAC_IF | 152 mV |
| J43-2 | TR9_DAC_IF | 147 mV |
| J43-4 | TR8_DAC_IF | 149 mV |
| J43-6 | TR15_DAC_IF | 146 mV |
| J43-8 | TR14_DAC_IF | 145 mV |
| J44-2 | TR21_DAC_IF | 166 mV |
| J44-4 | TR20_DAC_IF | 152 mV |
| J44-6 | TR19_DAC_IF | 162 mV |
| J44-8 | TR18_DAC_IF | 154 mV |
| J45-2 | TR17_DAC_IF | 153 mV |
| J45-4 | TR16_DAC_IF | 151 mV |
| J45-6 | TR23_DAC_IF | 162 mV |
| J45-8 | TR22_DAC_IF | 156 mV |
| J46-2 | TR29_DAC_IF | 150 mV |
| J46-4 | TR28_DAC_IF | 155 mV |
| J46-6 | TR27_DAC_IF | 50 mV |
| J46-8 | TR26_DAC_IF | 155 mV |
| J47-2 | TR25_DAC_IF | 143 mV |
| J47-4 | TR24_DAC_IF | 150 mV |
| J47-6 | TR31_DAC_IF | 153 mV |
| J47-8 | TR30_DAC_IF | 147 mV |
| J48-2 | TR37_DAC_IF | 164 mV |
| J48-4 | TR36_DAC_IF | 155 mV |
| J48-6 | TR35_DAC_IF | 158 mV |
| J48-8 | TR36_DAC_IF | 152 mV |
| J49-2 | TR33_DAC_IF | 149 mV |
| J49-4 | TR32_DAC_IF | 146 mV |
| J49-6 | TR39_DAC_IF | 156 mV |
| J49-8 | TR38_DAC_IF | 154 mV |
| J50-2 | TR45_DAC_IF | 149 mV |
| J50-4 | TR44_DAC_IF | 144 mV |
| J50-6 | TR43_DAC_IF | 153 mV |
| J50-8 | TR42_DAC_IF | 151 mV |
| J51-2 | TR41_DAC_IF | 147 mV |
| J51-4 | TR40_DAC_IF | 147 mV |
| J51-6 | TR47_DAC_IF | 149 mV |
| J51-8 | TR46_DAC_IF | 147 mV |
| J52-2 | TR53_DAC_IF | 169 mV |
| J52-4 | TR52_DAC_IF | 157 mV |
| J52-6 | TR51_DAC_IF | 164 mV |
| J52-8 | TR50_DAC_IF | 159 mV |
| J53-2 | TR49_DAC_IF | 152 mV |
| J53-4 | TR48_DAC_IF | 153 mV |
| J53-6 | TR55_DAC_IF | 164 mV |
| J53-8 | TR54_DAC_IF | 155 mV |
| J54-2 | TR61_DAC_IF | 155 mV |
| J54-4 | TR60_DAC_IF | 151 mV |
| J54-6 | TR59_DAC_IF | 151 mV |
| J54-8 | TR58_DAC_IF | 152 mV |
| J55-2 | TR57_DAC_IF | 153 mV |
| J55-4 | TR56_DAC_IF | 148 mV |
| J55-6 | TR63_DAC_IF | 150 mV |
| J55-8 | TR62_DAC_IF | 150 mV |

| Pin | Name | Value |
| --- | --- | --- |
| J40-1 | TR5_ADC_IF | FS ~ 5k |
| J44-1 | TR21_ADC_IF | FS ~ 5k |
| J48-1 | TR37_ADC_IF | FS ~ 2k |
| J52-1 | TR53_ADC_IF | FS ~ 5k |

Ethernet – MAP  MAC - c6:52:9a:1c:99:c0

# Appendix B – Results

## EIA-422 Signal Test

### CAL_SPI_SDO: U36-10 / U36-11 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 16.17 | 16.13 | ≤ 50ns | PASS |
| Fall Time (ns) | 15.96 | 16.18 | ≤ 50ns | PASS |
| Peak Voltage (V) | 2.34 | 2.33 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.90 | 0.70 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.40 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 4.59 |  | ≥ 2.0V | PASS |
| Noise Margin (V) | 2.08 |  | ≥ 0.2V | PASS |

### CAL_SPI_SCLK: U36-6 / U36-5 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 16.16 | 16.16 | ≤ 50ns | PASS |
| Fall Time (ns) | 16.00 | 16.00 | ≤ 50ns | PASS |
| Peak Voltage (V) | 2.32 | 2.31 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.02 | 0.03 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.10 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 4.63 |  | ≥ 2.0V | PASS |
| Noise Margin (V) | 2.10 |  | ≥ 0.2V | PASS |

### CAL_SPI_CS_N: U36-14 / U36-13 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 16.27 | 16.05 | ≤ 50ns | PASS |
| Fall Time (ns) | 16.18 | 16.00 | ≤ 50ns | PASS |
| Peak Voltage (V) | 2.31 | 2.34 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.00 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.70 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.50 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 4.62 |  | ≥ 2.0V | PASS |
| Noise Margin (V) | 2.10 |  | ≥ 0.2V | PASS |

### CAL_DISC: U36-2 / U36-3 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 15.87 | 15.91 | ≤ 50ns | PASS |
| Fall Time (ns) | 15.91 | 15.98 | ≤ 50ns | PASS |
| Peak Voltage (V) | 2.32 | 2.34 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.10 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.40 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 4.62 |  | ≥ 2.0V | PASS |
| Noise Margin (V) | 2.11 |  | ≥ 0.2V | PASS |

### TXLO_SPI_SDO: U837-10 / U837-11 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 16.03 | 15.82 | ≤ 50ns | PASS |
| Fall Time (ns) | 16.03 | 15.80 | ≤ 50ns | PASS |
| Peak Voltage (V) | 2.34 | 2.34 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.03 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.40 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.20 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 4.64 |  | ≥ 2.0V | PASS |
| Noise Margin (V) | 2.11 |  | ≥ 0.2V | PASS |

### TXLO_SPI_SCLK: U837-6 / U837-5 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 16.09 | 15.88 | ≤ 50ns | PASS |
| Fall Time (ns) | 15.90 | 15.87 | ≤ 50ns | PASS |
| Peak Voltage (V) | 2.31 | 2.34 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.00 | 0.03 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.30 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.40 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 4.61 |  | ≥ 2.0V | PASS |
| Noise Margin (V) | 2.10 |  | ≥ 0.2V | PASS |

### TXLO_SPI_CS_N: U837-14 / U837-13 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 16.06 | 15.93 | ≤ 50ns | PASS |
| Fall Time (ns) | 16.07 | 15.95 | ≤ 50ns | PASS |
| Peak Voltage (V) | 2.35 | 2.31 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.20 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.10 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 4.65 |  | ≥ 2.0V | PASS |
| Noise Margin (V) | 2.12 |  | ≥ 0.2V | PASS |

### TXLO_DISC: U837-2 / U837-3 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 16.15 | 16.19 | ≤ 50ns | PASS |
| Fall Time (ns) | 15.91 | 16.15 | ≤ 50ns | PASS |
| Peak Voltage (V) | 2.32 | 2.31 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | 0.00 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.20 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 4.66 |  | ≥ 2.0V | PASS |
| Noise Margin (V) | 2.12 |  | ≥ 0.2V | PASS |

### RXLO_SPI_SDO: U839-10 / U839-11 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 15.82 | 16.03 | ≤ 50ns | PASS |
| Fall Time (ns) | 15.87 | 15.85 | ≤ 50ns | PASS |
| Peak Voltage (V) | 2.31 | 2.31 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.03 | 0.03 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 4.61 |  | ≥ 2.0V | PASS |
| Noise Margin (V) | 2.10 |  | ≥ 0.2V | PASS |

### RXLO_SPI_SCLK: U839-6 / U839-5 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 15.93 | 15.96 | ≤ 50ns | PASS |
| Fall Time (ns) | 15.88 | 15.96 | ≤ 50ns | PASS |
| Peak Voltage (V) | 2.34 | 2.32 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.02 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.90 | 0.40 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 4.60 |  | ≥ 2.0V | PASS |
| Noise Margin (V) | 2.10 |  | ≥ 0.2V | PASS |

### RXLO_SPI_CS_N: U839-14 / U839-13 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 15.84 | 15.96 | ≤ 50ns | PASS |
| Fall Time (ns) | 16.04 | 16.13 | ≤ 50ns | PASS |
| Peak Voltage (V) | 2.30 | 2.30 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.00 | 0.03 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.10 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 4.64 |  | ≥ 2.0V | PASS |
| Noise Margin (V) | 2.11 |  | ≥ 0.2V | PASS |

### RXLO_DISC: U839-2 / U839-3 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 16.14 | 16.03 | ≤ 50ns | PASS |
| Fall Time (ns) | 16.10 | 16.02 | ≤ 50ns | PASS |
| Peak Voltage (V) | 2.30 | 2.30 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.00 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.70 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 4.61 |  | ≥ 2.0V | PASS |
| Noise Margin (V) | 2.10 |  | ≥ 0.2V | PASS |

### P48V_ENABLE: U136-2 / U136-3 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 16.30 | 16.18 | ≤ 50ns | PASS |
| Fall Time (ns) | 16.09 | 16.09 | ≤ 50ns | PASS |
| Peak Voltage (V) | 2.36 | 2.34 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.60 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 4.67 |  | ≥ 2.0V | PASS |
| Noise Margin (V) | 2.14 |  | ≥ 0.2V | PASS |

### FAN_ENABLE: U136-6 / U136-5 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 16.30 | 16.16 | ≤ 50ns | PASS |
| Fall Time (ns) | 16.39 | 16.29 | ≤ 50ns | PASS |
| Peak Voltage (V) | 2.35 | 2.35 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | -0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.40 | 0.80 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.60 | 1.20 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 4.63 |  | ≥ 2.0V | PASS |
| Noise Margin (V) | 2.11 |  | ≥ 0.2V | PASS |

### EXT_FAN_ENABLE: U136-14 / U136-13 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 16.17 | 16.13 | ≤ 50ns | PASS |
| Fall Time (ns) | 16.06 | 16.08 | ≤ 50ns | PASS |
| Peak Voltage (V) | 2.35 | 2.34 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.20 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 4.71 |  | ≥ 2.0V | PASS |
| Noise Margin (V) | 2.15 |  | ≥ 0.2V | PASS |

RS-422 specifications per TIA/EIA-422-B. Transition time limit = 50% of unit interval (100ns at 10 Mbps). Load: 3ft twisted pair cable (Z0=120Ω, vf=66%) into 124Ω differential termination. Driver model: AM26LV31ESDREP (Rout=25Ω, tr/tf=10ns, datasheet estimate).

## P3V3 POWER Signal Test

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Voltage (No Load, V) |  | 3.3V ± 100mV |  |
| Voltage (Full Load, V) |  | 3.3V ± 100mV |  |
| Current (mA) |  | ≤ 3000mA |  |
| Regulation (V) |  | ± 100mV |  |
| Ripple (Vpp) |  | ≤ 60mVpp |  |

## EIA-485 Signal Test

### TR0_EXT_SYNC: U128-6 / U128-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.82 | 14.86 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.98 | 15.83 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.60 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.20 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.24 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.41 |  | ≥ 0.2V | PASS |

### TR1_EXT_SYNC: U125-6 / U125-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.61 | 14.76 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.82 | 16.06 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.66 | 2.66 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.02 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.30 | 0.30 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.30 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.44 |  | ≥ 0.2V | PASS |

### TR2_EXT_SYNC: U119-6 / U119-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.66 | 14.75 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.09 | 15.84 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.64 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.00 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.40 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.29 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR3_EXT_SYNC: U113-6 / U113-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.68 | 14.69 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.88 | 15.98 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.66 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | 0.03 | ≥ -4V | PASS |
| Overshoot (%) | 0.50 | 1.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.80 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.24 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.41 |  | ≥ 0.2V | PASS |

### TR4_EXT_SYNC: U106-6 / U106-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.85 | 15.05 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.26 | 16.02 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.63 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.00 | -0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.70 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.29 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.44 |  | ≥ 0.2V | PASS |

### TR5_EXT_SYNC: U111-6 / U111-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.92 | 14.93 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.12 | 16.04 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.66 | 2.64 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | -0.01 | ≥ -4V | PASS |
| Overshoot (%) | 1.10 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.20 | 0.20 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.29 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.42 |  | ≥ 0.2V | PASS |

### TR6_EXT_SYNC: U118-6 / U118-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.76 | 14.83 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.98 | 15.91 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.00 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.60 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.60 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.25 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.40 |  | ≥ 0.2V | PASS |

### TR7_EXT_SYNC: U120-6 / U120-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.95 | 14.85 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.12 | 15.89 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.66 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.00 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.70 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.27 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.42 |  | ≥ 0.2V | PASS |

### TR8_EXT_SYNC: U55-6 / U55-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.74 | 14.77 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.10 | 15.83 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.66 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.02 | -0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.20 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.40 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.31 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.45 |  | ≥ 0.2V | PASS |

### TR9_EXT_SYNC: U58-6 / U58-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.69 | 14.55 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.80 | 16.00 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.63 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.02 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.27 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR10_EXT_SYNC: U64-6 / U64-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.72 | 14.60 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.97 | 15.89 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.00 | 0.00 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.80 | 0.20 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.26 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR11_EXT_SYNC: U66-6 / U66-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.76 | 14.88 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.10 | 16.00 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.66 | 2.63 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.02 | -0.00 | ≥ -4V | PASS |
| Overshoot (%) | 0.10 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.80 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.25 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.42 |  | ≥ 0.2V | PASS |

### TR12_EXT_SYNC: U62-6 / U62-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.86 | 15.02 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.11 | 16.03 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.02 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.30 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.27 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.42 |  | ≥ 0.2V | PASS |

### TR13_EXT_SYNC: U59-6 / U59-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.70 | 14.80 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.04 | 16.16 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.66 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | -0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.40 | 0.60 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.28 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.44 |  | ≥ 0.2V | PASS |

### TR14_EXT_SYNC: U52-6 / U52-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.62 | 14.65 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.93 | 15.96 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.65 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | -0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.90 | 0.40 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.80 | 0.40 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.24 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.42 |  | ≥ 0.2V | PASS |

### TR15_EXT_SYNC: U50-6 / U50-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.92 | 14.97 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.99 | 15.91 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | 0.00 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.30 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.80 | 0.10 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.26 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.42 |  | ≥ 0.2V | PASS |

### TR16_EXT_SYNC: U21-6 / U21-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.70 | 14.82 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.09 | 16.00 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.65 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.00 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.60 | 0.20 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.28 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.42 |  | ≥ 0.2V | PASS |

### TR17_EXT_SYNC: U23-6 / U23-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.55 | 14.71 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.93 | 15.94 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.62 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.30 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.30 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.44 |  | ≥ 0.2V | PASS |

### TR18_EXT_SYNC: U25-6 / U25-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.86 | 14.79 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.09 | 16.06 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.63 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | -0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.40 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.30 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR19_EXT_SYNC: U28-6 / U28-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.91 | 14.85 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.05 | 15.93 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.66 | 2.62 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.02 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.32 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.46 |  | ≥ 0.2V | PASS |

### TR20_EXT_SYNC: U27-6 / U27-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.88 | 14.90 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.25 | 16.05 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.50 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.20 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.29 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.44 |  | ≥ 0.2V | PASS |

### TR21_EXT_SYNC: U24-6 / U24-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.75 | 14.82 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.92 | 16.17 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.10 | 0.60 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.30 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.25 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.42 |  | ≥ 0.2V | PASS |

### TR22_EXT_SYNC: U22-6 / U22-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.69 | 14.90 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.05 | 16.06 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.63 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.00 | -0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.10 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.32 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.45 |  | ≥ 0.2V | PASS |

### TR23_EXT_SYNC: U19-6 / U19-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.67 | 14.72 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.09 | 15.99 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.66 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.30 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR24_EXT_SYNC: U1-6 / U1-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.61 | 14.66 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.09 | 16.06 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.65 | 2.63 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | 0.03 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.30 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.27 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR25_EXT_SYNC: U5-6 / U5-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.73 | 14.78 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.04 | 15.89 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.65 | 2.63 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.02 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.30 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.10 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.27 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.42 |  | ≥ 0.2V | PASS |

### TR26_EXT_SYNC: U140-6 / U140-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.76 | 14.66 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.97 | 15.95 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.66 | 2.64 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.00 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.40 | 0.10 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.20 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.29 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR27_EXT_SYNC: U18-6 / U18-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.76 | 14.70 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.88 | 15.94 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.65 | 2.63 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | 0.00 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.40 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.27 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR28_EXT_SYNC: U20-6 / U20-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.90 | 14.79 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.17 | 16.06 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.63 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.10 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.90 | 0.10 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.23 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.41 |  | ≥ 0.2V | PASS |

### TR29_EXT_SYNC: U14-6 / U14-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.72 | 14.82 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.01 | 16.07 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.64 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | -0.00 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.70 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.26 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR30_EXT_SYNC: U6-6 / U6-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.82 | 14.83 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.90 | 16.06 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.64 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.02 | -0.00 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.33 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.46 |  | ≥ 0.2V | PASS |

### TR31_EXT_SYNC: U2-6 / U2-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.75 | 14.92 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.07 | 16.14 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.40 | 0.30 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.40 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.27 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR32_EXT_SYNC: U3-6 / U3-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.83 | 14.80 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.97 | 16.06 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.62 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.30 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.30 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.44 |  | ≥ 0.2V | PASS |

### TR33_EXT_SYNC: U137-6 / U137-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.83 | 14.83 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.79 | 16.01 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.66 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | 0.00 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.10 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.80 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.30 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR34_EXT_SYNC: U9-6 / U9-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.73 | 14.59 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.93 | 16.10 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.66 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.00 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.10 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.31 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.45 |  | ≥ 0.2V | PASS |

### TR35_EXT_SYNC: U13-6 / U13-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.90 | 14.79 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.07 | 15.91 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.62 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.40 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.70 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.27 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.44 |  | ≥ 0.2V | PASS |

### TR36_EXT_SYNC: U16-6 / U16-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.77 | 14.78 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.16 | 16.17 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.63 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.03 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.40 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.30 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.22 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.40 |  | ≥ 0.2V | PASS |

### TR37_EXT_SYNC: U15-6 / U15-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.72 | 14.85 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.96 | 16.08 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.66 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.20 | 0.30 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.20 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.27 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.41 |  | ≥ 0.2V | PASS |

### TR38_EXT_SYNC: U139-6 / U139-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.79 | 14.75 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.98 | 16.07 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.00 | -0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.40 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.30 | 0.70 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.28 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR39_EXT_SYNC: U8-6 / U8-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.81 | 14.68 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.10 | 16.17 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.00 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.30 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.26 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.42 |  | ≥ 0.2V | PASS |

### TR40_EXT_SYNC: U61-6 / U61-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.74 | 14.85 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.11 | 15.96 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.63 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.00 | 0.03 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.70 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.27 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.42 |  | ≥ 0.2V | PASS |

### TR41_EXT_SYNC: U57-6 / U57-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.55 | 14.70 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.81 | 15.99 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.65 | 2.64 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.00 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.40 | 0.10 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.20 | 0.40 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.25 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.42 |  | ≥ 0.2V | PASS |

### TR42_EXT_SYNC: U51-6 / U51-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.65 | 14.87 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.00 | 15.95 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.64 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.02 | 0.00 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.50 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.28 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.44 |  | ≥ 0.2V | PASS |

### TR43_EXT_SYNC: U49-6 / U49-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.88 | 14.93 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.97 | 16.08 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | -0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.80 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.70 | 0.20 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.26 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.41 |  | ≥ 0.2V | PASS |

### TR44_EXT_SYNC: U54-6 / U54-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.87 | 14.78 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.21 | 16.22 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.65 | 2.66 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | -0.00 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.80 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.80 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.27 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR45_EXT_SYNC: U56-6 / U56-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.75 | 14.83 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.97 | 15.95 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.66 | 2.66 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.02 | -0.00 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.70 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.31 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.44 |  | ≥ 0.2V | PASS |

### TR46_EXT_SYNC: U63-6 / U63-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.64 | 14.83 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.11 | 15.92 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.64 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.03 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.20 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.29 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR47_EXT_SYNC: U65-6 / U65-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.79 | 14.86 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.94 | 15.90 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.66 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.02 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 1.10 | 0.60 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.60 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.23 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.40 |  | ≥ 0.2V | PASS |

### TR48_EXT_SYNC: U101-6 / U101-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.87 | 14.76 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.01 | 16.02 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.66 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.30 | 0.30 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.80 | 0.50 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.25 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.42 |  | ≥ 0.2V | PASS |

### TR49_EXT_SYNC: U99-6 / U99-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.81 | 14.81 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.85 | 15.93 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.66 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.00 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.90 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.28 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.44 |  | ≥ 0.2V | PASS |

### TR50_EXT_SYNC: U97-6 / U97-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.75 | 14.85 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.09 | 15.99 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.02 | -0.00 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.10 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.10 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.28 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.42 |  | ≥ 0.2V | PASS |

### TR51_EXT_SYNC: U95-6 / U95-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.81 | 14.72 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.16 | 16.03 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.62 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.02 | 0.03 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.20 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.28 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.44 |  | ≥ 0.2V | PASS |

### TR52_EXT_SYNC: U96-6 / U96-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.93 | 14.85 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.95 | 15.98 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.66 | 2.63 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | 0.00 | ≥ -4V | PASS |
| Overshoot (%) | 0.40 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.50 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.28 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR53_EXT_SYNC: U98-6 / U98-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.66 | 14.68 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.11 | 16.11 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.63 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.00 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.10 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.10 | 0.30 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.25 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.42 |  | ≥ 0.2V | PASS |

### TR54_EXT_SYNC: U100-6 / U100-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.80 | 14.65 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.12 | 15.94 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.30 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.20 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.28 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.44 |  | ≥ 0.2V | PASS |

### TR55_EXT_SYNC: U103-6 / U103-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.75 | 14.70 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.97 | 16.01 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.65 | 2.62 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.30 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.45 |  | ≥ 0.2V | PASS |

### TR56_EXT_SYNC: U131-6 / U131-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.60 | 14.68 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.06 | 16.01 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.66 | 2.66 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.50 | 1.10 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.27 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR57_EXT_SYNC: U127-6 / U127-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.78 | 14.65 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.96 | 16.00 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.66 | 2.64 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | -0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 1.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.28 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.44 |  | ≥ 0.2V | PASS |

### TR58_EXT_SYNC: U117-6 / U117-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.63 | 14.67 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.07 | 15.99 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.66 | 2.66 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.20 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.50 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.29 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR59_EXT_SYNC: U105-6 / U105-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.68 | 14.82 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.00 | 16.15 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.65 | 2.64 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.00 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.50 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.40 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.28 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR60_EXT_SYNC: U102-6 / U102-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.77 | 14.79 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.22 | 16.10 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.62 | 2.64 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.10 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.28 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR61_EXT_SYNC: U112-6 / U112-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.75 | 14.91 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.04 | 16.11 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.01 | -0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.70 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.80 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.27 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### TR62_EXT_SYNC: U123-6 / U123-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.77 | 14.70 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.99 | 15.97 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.65 | 2.63 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | 0.00 | ≥ -4V | PASS |
| Overshoot (%) | 0.50 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.40 | 0.60 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.27 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.41 |  | ≥ 0.2V | PASS |

### TR63_EXT_SYNC: U130-6 / U130-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.91 | 14.71 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.93 | 16.07 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.66 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.01 | -0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.50 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.60 | 0.70 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.26 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.43 |  | ≥ 0.2V | PASS |

### MBUS: U132-6 / U132-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 6.73 | 6.69 | ≤ 30ns | PASS |
| Fall Time (ns) | 8.31 | 8.22 | ≤ 30ns | PASS |
| Peak Voltage (V) | 1.99 | 2.00 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.15 | -0.16 | ≥ -4V | PASS |
| Overshoot (%) | 1.70 | 2.60 | < 10% of amplitude | PASS |
| Undershoot (%) | 1.40 | 1.30 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 4.08 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 1.82 |  | ≥ 0.2V | PASS |

### CBUS: U135-6 / U135-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 6.71 | 6.68 | ≤ 30ns | PASS |
| Fall Time (ns) | 8.25 | 8.33 | ≤ 30ns | PASS |
| Peak Voltage (V) | 1.97 | 1.98 | ≤ 8V | PASS |
| Trough Voltage (V) | -0.15 | -0.15 | ≥ -4V | PASS |
| Overshoot (%) | 1.40 | 0.80 | < 10% of amplitude | PASS |
| Undershoot (%) | 1.90 | 1.80 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 4.05 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 1.82 |  | ≥ 0.2V | PASS |

### CAL_MBUS: U38-6 / U38-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.62 | 14.43 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.68 | 15.75 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.59 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.04 | 0.04 | ≥ -4V | PASS |
| Overshoot (%) | 0.30 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.10 | 0.10 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.16 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.38 |  | ≥ 0.2V | PASS |

### TXLO_MBUS: U60-6 / U60-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.39 | 14.60 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.66 | 15.62 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.60 | 2.62 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.02 | 0.03 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.60 | < 10% of amplitude | PASS |
| Undershoot (%) | 1.00 | 0.40 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.12 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.34 |  | ≥ 0.2V | PASS |

### RXLO_MBUS: U838-6 / U838-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.40 | 14.51 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.76 | 15.76 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.60 | 2.62 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.03 | 0.04 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.80 | 0.10 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.15 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.37 |  | ≥ 0.2V | PASS |

### INS_TX: U875-6 / U875-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.66 | 14.88 | ≤ 30ns | PASS |
| Fall Time (ns) | 16.07 | 16.17 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.61 | 2.61 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.05 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.80 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.16 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.37 |  | ≥ 0.2V | PASS |

### INS_RX: U876-6 / U876-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.55 | 14.79 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.95 | 15.91 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.60 | 2.61 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.04 | 0.02 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.40 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.16 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.36 |  | ≥ 0.2V | PASS |

### TE_INS_TX: U802-6 / U802-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.67 | 14.61 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.67 | 15.73 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.63 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.02 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.30 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.45 |  | ≥ 0.2V | PASS |

### TE_INS_RX: U801-6 / U801-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.54 | 14.48 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.77 | 15.90 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.64 | 2.65 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.03 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.28 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.42 |  | ≥ 0.2V | PASS |

### TE_INS_SEL: U800-6 / U800-7 → Load R-1 / Load R-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 14.73 | 14.59 | ≤ 30ns | PASS |
| Fall Time (ns) | 15.83 | 15.83 | ≤ 30ns | PASS |
| Peak Voltage (V) | 2.63 | 2.63 | ≤ 8V | PASS |
| Trough Voltage (V) | 0.00 | 0.01 | ≥ -4V | PASS |
| Overshoot (%) | 0.00 | 0.00 | < 10% of amplitude | PASS |
| Undershoot (%) | 0.10 | 0.00 | < 10% of amplitude | PASS |
| VOD (VP – VN) (V) | 5.32 |  | ≥ 1.5V | PASS |
| Noise Margin (V) | 2.45 |  | ≥ 0.2V | PASS |

RS-485 specifications per TIA/EIA-485. Transition time limit ≤ 30ns (system data rate dependent). Load: 3ft twisted pair cable (Z0=120Ω, vf=66%) into 124Ω differential termination. Driver model: SN55HVD75 A/B (Rout=15Ω, tr/tf=6.3/7.8ns, IBIS).

## 1.8V LVCMOS Signal Test

### TR0_EXT_SYNC_P1V8

#### U10-H14 → U126-21

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.731 | ≥ 1.35V | PASS |
| VOL (V) | 0.069 | ≤ 0.45V | PASS |
| Rise Time (ns) | 5.14 | ≤ 10ns | PASS |
| Fall Time (ns) | 5.12 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.464 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.509 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.731V, Trough=0.069V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U126-21 → U10-H14

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.786 | ≥ 1.35V | PASS |
| VOL (V) | 0.014 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.57 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.96 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.616 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.616 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.950V, Trough=-0.205V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR10_EXT_SYNC_P1V8

#### U10-N14 → U71-19

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.713 | ≥ 1.35V | PASS |
| VOL (V) | 0.087 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.77 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.75 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.446 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.491 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.713V, Trough=0.087V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U71-19 → U10-N14

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.782 | ≥ 1.35V | PASS |
| VOL (V) | 0.018 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.37 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.83 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.612 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.612 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.019V, Trough=-0.312V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR11_EXT_SYNC_P1V8

#### U10-N13 → U71-18

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.713 | ≥ 1.35V | PASS |
| VOL (V) | 0.087 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.88 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.86 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.446 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.491 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.713V, Trough=0.087V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U71-18 → U10-N13

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.782 | ≥ 1.35V | PASS |
| VOL (V) | 0.018 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.37 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.83 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.612 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.612 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.019V, Trough=-0.312V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR12_EXT_SYNC_P1V8

#### U10-N16 → U71-17

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.714 | ≥ 1.35V | PASS |
| VOL (V) | 0.086 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.87 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.86 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.446 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.491 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.714V, Trough=0.086V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U71-17 → U10-N16

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.782 | ≥ 1.35V | PASS |
| VOL (V) | 0.018 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.37 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.83 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.612 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.612 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.019V, Trough=-0.313V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR13_EXT_SYNC_P1V8

#### U10-M16 → U71-16

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.714 | ≥ 1.35V | PASS |
| VOL (V) | 0.086 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.87 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.86 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.446 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.491 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.714V, Trough=0.086V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U71-16 → U10-M16

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.782 | ≥ 1.35V | PASS |
| VOL (V) | 0.018 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.37 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.83 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.612 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.612 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.019V, Trough=-0.313V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR14_EXT_SYNC_P1V8

#### U10-L15 → U71-15

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.714 | ≥ 1.35V | PASS |
| VOL (V) | 0.086 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.87 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.86 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.446 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.491 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.714V, Trough=0.086V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U71-15 → U10-L15

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.782 | ≥ 1.35V | PASS |
| VOL (V) | 0.018 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.37 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.83 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.612 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.612 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.019V, Trough=-0.313V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR15_EXT_SYNC_P1V8

#### U10-K15 → U71-14

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.714 | ≥ 1.35V | PASS |
| VOL (V) | 0.086 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.98 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.97 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.446 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.491 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.714V, Trough=0.086V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U71-14 → U10-K15

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.782 | ≥ 1.35V | PASS |
| VOL (V) | 0.018 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.37 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.83 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.612 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.612 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.019V, Trough=-0.313V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR16_EXT_SYNC_P1V8

#### U10-E36 → U41-21

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.674 | ≥ 1.35V | PASS |
| VOL (V) | 0.126 | ≤ 0.45V | PASS |
| Rise Time (ns) | 10.66 | ≤ 10ns | FAIL |
| Fall Time (ns) | 10.63 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.407 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.452 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.674V, Trough=0.126V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U41-21 → U10-E36

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.774 | ≥ 1.35V | PASS |
| VOL (V) | 0.026 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.25 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.85 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.604 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.604 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.231V, Trough=-0.526V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR17_EXT_SYNC_P1V8

#### U10-E37 → U41-20

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.674 | ≥ 1.35V | PASS |
| VOL (V) | 0.126 | ≤ 0.45V | PASS |
| Rise Time (ns) | 10.55 | ≤ 10ns | FAIL |
| Fall Time (ns) | 10.52 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.407 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.452 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.674V, Trough=0.126V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U41-20 → U10-E37

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.774 | ≥ 1.35V | PASS |
| VOL (V) | 0.026 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.25 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.85 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.604 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.604 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.231V, Trough=-0.526V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR18_EXT_SYNC_P1V8

#### U10-G38 → U41-19

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.674 | ≥ 1.35V | PASS |
| VOL (V) | 0.126 | ≤ 0.45V | PASS |
| Rise Time (ns) | 10.55 | ≤ 10ns | FAIL |
| Fall Time (ns) | 10.52 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.407 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.452 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.674V, Trough=0.126V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U41-19 → U10-G38

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.774 | ≥ 1.35V | PASS |
| VOL (V) | 0.026 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.25 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.85 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.604 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.604 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.231V, Trough=-0.526V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR19_EXT_SYNC_P1V8

#### U10-F38 → U41-18

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.679 | ≥ 1.35V | PASS |
| VOL (V) | 0.121 | ≤ 0.45V | PASS |
| Rise Time (ns) | 10.06 | ≤ 10ns | FAIL |
| Fall Time (ns) | 10.04 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.411 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.456 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.679V, Trough=0.121V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U41-18 → U10-F38

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.775 | ≥ 1.35V | PASS |
| VOL (V) | 0.025 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.24 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.84 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.605 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.605 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.212V, Trough=-0.516V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR1_EXT_SYNC_P1V8

#### U10-H13 → U126-20

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.736 | ≥ 1.35V | PASS |
| VOL (V) | 0.064 | ≤ 0.45V | PASS |
| Rise Time (ns) | 4.78 | ≤ 10ns | PASS |
| Fall Time (ns) | 4.77 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.469 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.514 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.736V, Trough=0.064V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U126-20 → U10-H13

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.787 | ≥ 1.35V | PASS |
| VOL (V) | 0.013 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.68 | ≤ 10ns | PASS |
| Fall Time (ns) | 2.06 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.617 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.617 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.919V, Trough=-0.149V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR20_EXT_SYNC_P1V8

#### U10-E39 → U41-17

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.679 | ≥ 1.35V | PASS |
| VOL (V) | 0.121 | ≤ 0.45V | PASS |
| Rise Time (ns) | 9.95 | ≤ 10ns | PASS |
| Fall Time (ns) | 9.93 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.411 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.456 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.679V, Trough=0.121V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U41-17 → U10-E39

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.775 | ≥ 1.35V | PASS |
| VOL (V) | 0.025 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.24 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.84 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.605 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.605 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.189V, Trough=-0.503V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR21_EXT_SYNC_P1V8

#### U10-E40 → U41-16

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.679 | ≥ 1.35V | PASS |
| VOL (V) | 0.121 | ≤ 0.45V | PASS |
| Rise Time (ns) | 10.06 | ≤ 10ns | FAIL |
| Fall Time (ns) | 10.04 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.411 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.456 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.679V, Trough=0.121V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U41-16 → U10-E40

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.775 | ≥ 1.35V | PASS |
| VOL (V) | 0.025 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.24 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.84 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.605 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.605 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.189V, Trough=-0.503V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR22_EXT_SYNC_P1V8

#### U10-F35 → U41-15

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.679 | ≥ 1.35V | PASS |
| VOL (V) | 0.121 | ≤ 0.45V | PASS |
| Rise Time (ns) | 10.28 | ≤ 10ns | FAIL |
| Fall Time (ns) | 10.26 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.411 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.456 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.679V, Trough=0.121V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U41-15 → U10-F35

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.775 | ≥ 1.35V | PASS |
| VOL (V) | 0.025 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.24 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.84 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.605 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.605 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.212V, Trough=-0.516V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR23_EXT_SYNC_P1V8

#### U10-E35 → U41-14

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.678 | ≥ 1.35V | PASS |
| VOL (V) | 0.122 | ≤ 0.45V | PASS |
| Rise Time (ns) | 10.34 | ≤ 10ns | FAIL |
| Fall Time (ns) | 10.32 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.410 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.455 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.678V, Trough=0.122V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U41-14 → U10-E35

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.774 | ≥ 1.35V | PASS |
| VOL (V) | 0.026 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.25 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.85 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.604 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.604 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.209V, Trough=-0.512V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR24_EXT_SYNC_P1V8

#### U10-L35 → U26-21

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.661 | ≥ 1.35V | PASS |
| VOL (V) | 0.139 | ≤ 0.45V | PASS |
| Rise Time (ns) | 14.63 | ≤ 10ns | FAIL |
| Fall Time (ns) | 14.63 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.394 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.439 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.661V, Trough=0.139V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U26-21 → U10-L35

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.771 | ≥ 1.35V | PASS |
| VOL (V) | 0.029 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.25 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.85 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.601 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.601 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.284V, Trough=-0.531V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR25_EXT_SYNC_P1V8

#### U10-L36 → U26-20

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.660 | ≥ 1.35V | PASS |
| VOL (V) | 0.140 | ≤ 0.45V | PASS |
| Rise Time (ns) | 14.78 | ≤ 10ns | FAIL |
| Fall Time (ns) | 14.77 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.393 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.438 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.660V, Trough=0.140V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U26-20 → U10-L36

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.770 | ≥ 1.35V | PASS |
| VOL (V) | 0.030 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.26 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.85 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.600 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.600 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.297V, Trough=-0.528V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR26_EXT_SYNC_P1V8

#### U10-L33 → U26-19

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.666 | ≥ 1.35V | PASS |
| VOL (V) | 0.134 | ≤ 0.45V | PASS |
| Rise Time (ns) | 13.95 | ≤ 10ns | FAIL |
| Fall Time (ns) | 13.98 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.398 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.443 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.666V, Trough=0.134V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U26-19 → U10-L33

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.772 | ≥ 1.35V | PASS |
| VOL (V) | 0.028 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.25 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.85 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.602 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.602 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.267V, Trough=-0.532V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR27_EXT_SYNC_P1V8

#### U10-K33 → U26-18

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.665 | ≥ 1.35V | PASS |
| VOL (V) | 0.135 | ≤ 0.45V | PASS |
| Rise Time (ns) | 13.91 | ≤ 10ns | FAIL |
| Fall Time (ns) | 13.92 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.397 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.442 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.665V, Trough=0.135V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U26-18 → U10-K33

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.771 | ≥ 1.35V | PASS |
| VOL (V) | 0.029 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.26 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.85 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.601 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.601 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.264V, Trough=-0.528V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR28_EXT_SYNC_P1V8

#### U10-K35 → U26-17

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.669 | ≥ 1.35V | PASS |
| VOL (V) | 0.131 | ≤ 0.45V | PASS |
| Rise Time (ns) | 11.60 | ≤ 10ns | FAIL |
| Fall Time (ns) | 11.59 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.401 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.446 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.669V, Trough=0.131V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U26-17 → U10-K35

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.772 | ≥ 1.35V | PASS |
| VOL (V) | 0.028 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.25 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.85 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.602 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.602 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.247V, Trough=-0.527V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR29_EXT_SYNC_P1V8

#### U10-J35 → U26-16

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.670 | ≥ 1.35V | PASS |
| VOL (V) | 0.130 | ≤ 0.45V | PASS |
| Rise Time (ns) | 11.28 | ≤ 10ns | FAIL |
| Fall Time (ns) | 11.26 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.402 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.447 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.670V, Trough=0.130V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U26-16 → U10-J35

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.773 | ≥ 1.35V | PASS |
| VOL (V) | 0.027 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.25 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.85 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.603 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.603 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.250V, Trough=-0.531V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR2_EXT_SYNC_P1V8

#### U10-F13 → U126-19

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.741 | ≥ 1.35V | PASS |
| VOL (V) | 0.059 | ≤ 0.45V | PASS |
| Rise Time (ns) | 4.65 | ≤ 10ns | PASS |
| Fall Time (ns) | 4.64 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.473 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.518 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.741V, Trough=0.059V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U126-19 → U10-F13

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.788 | ≥ 1.35V | PASS |
| VOL (V) | 0.012 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.67 | ≤ 10ns | PASS |
| Fall Time (ns) | 2.06 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.618 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.618 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.928V, Trough=-0.151V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR30_EXT_SYNC_P1V8

#### U10-H33 → U26-15

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.669 | ≥ 1.35V | PASS |
| VOL (V) | 0.131 | ≤ 0.45V | PASS |
| Rise Time (ns) | 11.71 | ≤ 10ns | FAIL |
| Fall Time (ns) | 11.70 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.401 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.446 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.669V, Trough=0.131V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U26-15 → U10-H33

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.772 | ≥ 1.35V | PASS |
| VOL (V) | 0.028 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.25 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.85 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.602 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.602 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.247V, Trough=-0.527V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR31_EXT_SYNC_P1V8

#### U10-H34 → U26-14

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.670 | ≥ 1.35V | PASS |
| VOL (V) | 0.130 | ≤ 0.45V | PASS |
| Rise Time (ns) | 11.28 | ≤ 10ns | FAIL |
| Fall Time (ns) | 11.26 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.402 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.447 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.670V, Trough=0.130V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U26-14 → U10-H34

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.773 | ≥ 1.35V | PASS |
| VOL (V) | 0.027 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.25 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.85 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.603 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.603 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.250V, Trough=-0.531V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR32_EXT_SYNC_P1V8

#### U10-AW35 → U138-21

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.614 | ≥ 1.35V | PASS |
| VOL (V) | 0.186 | ≤ 0.45V | PASS |
| Rise Time (ns) | 0.28 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.24 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.346 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.391 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.614V, Trough=0.186V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U138-21 → U10-AW35

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.759 | ≥ 1.35V | PASS |
| VOL (V) | 0.041 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.28 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.88 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.589 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.589 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.302V, Trough=-0.502V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR33_EXT_SYNC_P1V8

#### U10-AW36 → U138-20

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.614 | ≥ 1.35V | PASS |
| VOL (V) | 0.186 | ≤ 0.45V | PASS |
| Rise Time (ns) | 0.28 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.24 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.346 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.391 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.614V, Trough=0.186V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U138-20 → U10-AW36

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.759 | ≥ 1.35V | PASS |
| VOL (V) | 0.041 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.28 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.88 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.589 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.589 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.302V, Trough=-0.502V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR34_EXT_SYNC_P1V8

#### U10-AV36 → U138-19

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.609 | ≥ 1.35V | PASS |
| VOL (V) | 0.191 | ≤ 0.45V | PASS |
| Rise Time (ns) | 0.28 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.24 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.341 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.386 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.609V, Trough=0.191V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U138-19 → U10-AV36

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.758 | ≥ 1.35V | PASS |
| VOL (V) | 0.042 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.29 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.88 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.588 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.588 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.297V, Trough=-0.497V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR35_EXT_SYNC_P1V8

#### U10-AV37 → U138-18

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.609 | ≥ 1.35V | PASS |
| VOL (V) | 0.191 | ≤ 0.45V | PASS |
| Rise Time (ns) | 0.28 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.24 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.341 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.386 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.609V, Trough=0.191V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U138-18 → U10-AV37

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.758 | ≥ 1.35V | PASS |
| VOL (V) | 0.042 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.29 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.88 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.588 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.588 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.297V, Trough=-0.497V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR36_EXT_SYNC_P1V8

#### U10-AU34 → U138-17

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.605 | ≥ 1.35V | PASS |
| VOL (V) | 0.195 | ≤ 0.45V | PASS |
| Rise Time (ns) | 0.28 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.24 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.337 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.382 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.605V, Trough=0.195V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U138-17 → U10-AU34

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.757 | ≥ 1.35V | PASS |
| VOL (V) | 0.043 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.29 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.88 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.587 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.587 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.295V, Trough=-0.495V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR37_EXT_SYNC_P1V8

#### U10-AU35 → U138-16

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.609 | ≥ 1.35V | PASS |
| VOL (V) | 0.191 | ≤ 0.45V | PASS |
| Rise Time (ns) | 0.28 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.24 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.341 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.386 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.609V, Trough=0.191V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U138-16 → U10-AU35

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.758 | ≥ 1.35V | PASS |
| VOL (V) | 0.042 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.29 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.88 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.588 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.588 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.297V, Trough=-0.497V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR38_EXT_SYNC_P1V8

#### U10-AV38 → U138-15

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.609 | ≥ 1.35V | PASS |
| VOL (V) | 0.191 | ≤ 0.45V | PASS |
| Rise Time (ns) | 0.28 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.24 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.341 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.386 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.609V, Trough=0.191V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U138-15 → U10-AV38

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.758 | ≥ 1.35V | PASS |
| VOL (V) | 0.042 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.29 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.88 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.588 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.588 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.297V, Trough=-0.497V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR39_EXT_SYNC_P1V8

#### U10-AW38 → U138-14

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.609 | ≥ 1.35V | PASS |
| VOL (V) | 0.191 | ≤ 0.45V | PASS |
| Rise Time (ns) | 0.28 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.24 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.341 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.386 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.609V, Trough=0.191V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U138-14 → U10-AW38

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.758 | ≥ 1.35V | PASS |
| VOL (V) | 0.042 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.29 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.88 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.588 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.588 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.297V, Trough=-0.497V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR3_EXT_SYNC_P1V8

#### U10-E13 → U126-18

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.741 | ≥ 1.35V | PASS |
| VOL (V) | 0.059 | ≤ 0.45V | PASS |
| Rise Time (ns) | 4.66 | ≤ 10ns | PASS |
| Fall Time (ns) | 4.65 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.473 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.518 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.741V, Trough=0.059V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U126-18 → U10-E13

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.788 | ≥ 1.35V | PASS |
| VOL (V) | 0.012 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.67 | ≤ 10ns | PASS |
| Fall Time (ns) | 2.06 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.618 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.618 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.928V, Trough=-0.150V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR40_EXT_SYNC_P1V8

#### U10-BE38 → U70-21

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.723 | ≥ 1.35V | PASS |
| VOL (V) | 0.077 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.04 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.02 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.456 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.501 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.723V, Trough=0.077V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U70-21 → U10-BE38

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.784 | ≥ 1.35V | PASS |
| VOL (V) | 0.016 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.47 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.87 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.614 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.614 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.976V, Trough=-0.261V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR41_EXT_SYNC_P1V8

#### U10-BF38 → U70-20

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.723 | ≥ 1.35V | PASS |
| VOL (V) | 0.077 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.04 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.02 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.456 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.501 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.723V, Trough=0.077V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U70-20 → U10-BF38

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.784 | ≥ 1.35V | PASS |
| VOL (V) | 0.016 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.47 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.87 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.614 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.614 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.976V, Trough=-0.261V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR42_EXT_SYNC_P1V8

#### U10-BF42 → U70-19

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.728 | ≥ 1.35V | PASS |
| VOL (V) | 0.072 | ≤ 0.45V | PASS |
| Rise Time (ns) | 5.80 | ≤ 10ns | PASS |
| Fall Time (ns) | 5.78 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.460 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.505 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.728V, Trough=0.072V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U70-19 → U10-BF42

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.785 | ≥ 1.35V | PASS |
| VOL (V) | 0.015 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.52 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.91 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.615 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.615 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.954V, Trough=-0.234V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR43_EXT_SYNC_P1V8

#### U10-BF43 → U70-18

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.728 | ≥ 1.35V | PASS |
| VOL (V) | 0.072 | ≤ 0.45V | PASS |
| Rise Time (ns) | 5.69 | ≤ 10ns | PASS |
| Fall Time (ns) | 5.67 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.460 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.505 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.728V, Trough=0.072V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U70-18 → U10-BF43

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.785 | ≥ 1.35V | PASS |
| VOL (V) | 0.015 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.52 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.91 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.615 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.615 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.954V, Trough=-0.234V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR44_EXT_SYNC_P1V8

#### U10-BD39 → U70-17

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.728 | ≥ 1.35V | PASS |
| VOL (V) | 0.072 | ≤ 0.45V | PASS |
| Rise Time (ns) | 5.79 | ≤ 10ns | PASS |
| Fall Time (ns) | 5.77 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.461 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.506 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.728V, Trough=0.072V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U70-17 → U10-BD39

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.785 | ≥ 1.35V | PASS |
| VOL (V) | 0.015 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.46 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.86 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.615 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.615 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.978V, Trough=-0.263V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR45_EXT_SYNC_P1V8

#### U10-BD40 → U70-16

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.728 | ≥ 1.35V | PASS |
| VOL (V) | 0.072 | ≤ 0.45V | PASS |
| Rise Time (ns) | 5.80 | ≤ 10ns | PASS |
| Fall Time (ns) | 5.78 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.460 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.505 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.728V, Trough=0.072V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U70-16 → U10-BD40

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.785 | ≥ 1.35V | PASS |
| VOL (V) | 0.015 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.52 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.91 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.615 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.615 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.954V, Trough=-0.234V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR46_EXT_SYNC_P1V8

#### U10-BC37 → U70-15

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.728 | ≥ 1.35V | PASS |
| VOL (V) | 0.072 | ≤ 0.45V | PASS |
| Rise Time (ns) | 5.91 | ≤ 10ns | PASS |
| Fall Time (ns) | 5.89 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.460 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.505 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.728V, Trough=0.072V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U70-15 → U10-BC37

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.785 | ≥ 1.35V | PASS |
| VOL (V) | 0.015 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.46 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.87 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.615 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.615 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.977V, Trough=-0.262V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR47_EXT_SYNC_P1V8

#### U10-BC38 → U70-14

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.728 | ≥ 1.35V | PASS |
| VOL (V) | 0.072 | ≤ 0.45V | PASS |
| Rise Time (ns) | 5.79 | ≤ 10ns | PASS |
| Fall Time (ns) | 5.77 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.461 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.506 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.728V, Trough=0.072V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U70-14 → U10-BC38

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.785 | ≥ 1.35V | PASS |
| VOL (V) | 0.015 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.52 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.91 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.615 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.615 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.954V, Trough=-0.234V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR48_EXT_SYNC_P1V8

#### U10-AY11 → U134-21

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.718 | ≥ 1.35V | PASS |
| VOL (V) | 0.082 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.53 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.51 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.450 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.495 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.718V, Trough=0.082V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U134-21 → U10-AY11

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.783 | ≥ 1.35V | PASS |
| VOL (V) | 0.017 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.42 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.84 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.613 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.613 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.996V, Trough=-0.285V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR49_EXT_SYNC_P1V8

#### U10-BA11 → U134-20

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.718 | ≥ 1.35V | PASS |
| VOL (V) | 0.082 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.53 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.51 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.450 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.495 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.718V, Trough=0.082V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U134-20 → U10-BA11

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.783 | ≥ 1.35V | PASS |
| VOL (V) | 0.017 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.42 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.84 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.613 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.613 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.996V, Trough=-0.285V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR4_EXT_SYNC_P1V8

#### U10-G14 → U126-17

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.736 | ≥ 1.35V | PASS |
| VOL (V) | 0.064 | ≤ 0.45V | PASS |
| Rise Time (ns) | 4.79 | ≤ 10ns | PASS |
| Fall Time (ns) | 4.77 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.469 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.514 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.736V, Trough=0.064V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U126-17 → U10-G14

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.787 | ≥ 1.35V | PASS |
| VOL (V) | 0.013 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.68 | ≤ 10ns | PASS |
| Fall Time (ns) | 2.06 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.617 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.617 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.927V, Trough=-0.149V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR50_EXT_SYNC_P1V8

#### U10-AU14 → U134-19

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.722 | ≥ 1.35V | PASS |
| VOL (V) | 0.078 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.06 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.04 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.455 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.500 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.722V, Trough=0.078V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U134-19 → U10-AU14

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.784 | ≥ 1.35V | PASS |
| VOL (V) | 0.016 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.47 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.87 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.614 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.614 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.975V, Trough=-0.259V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR51_EXT_SYNC_P1V8

#### U10-AV14 → U134-18

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.718 | ≥ 1.35V | PASS |
| VOL (V) | 0.082 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.31 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.29 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.450 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.495 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.718V, Trough=0.082V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U134-18 → U10-AV14

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.783 | ≥ 1.35V | PASS |
| VOL (V) | 0.017 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.42 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.84 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.613 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.613 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.996V, Trough=-0.285V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR52_EXT_SYNC_P1V8

#### U10-AW15 → U134-17

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.722 | ≥ 1.35V | PASS |
| VOL (V) | 0.078 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.06 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.04 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.455 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.500 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.722V, Trough=0.078V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U134-17 → U10-AW15

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.784 | ≥ 1.35V | PASS |
| VOL (V) | 0.016 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.47 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.87 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.614 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.614 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.975V, Trough=-0.259V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR53_EXT_SYNC_P1V8

#### U10-AY15 → U134-16

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.722 | ≥ 1.35V | PASS |
| VOL (V) | 0.078 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.06 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.04 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.455 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.500 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.722V, Trough=0.078V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U134-16 → U10-AY15

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.784 | ≥ 1.35V | PASS |
| VOL (V) | 0.016 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.47 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.87 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.614 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.614 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.975V, Trough=-0.259V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR54_EXT_SYNC_P1V8

#### U10-AU13 → U134-15

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.723 | ≥ 1.35V | PASS |
| VOL (V) | 0.077 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.17 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.14 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.455 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.500 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.723V, Trough=0.077V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U134-15 → U10-AU13

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.784 | ≥ 1.35V | PASS |
| VOL (V) | 0.016 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.47 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.87 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.614 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.614 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.975V, Trough=-0.259V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR55_EXT_SYNC_P1V8

#### U10-AV13 → U134-14

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.723 | ≥ 1.35V | PASS |
| VOL (V) | 0.077 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.17 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.14 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.455 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.500 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.723V, Trough=0.077V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U134-14 → U10-AV13

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.784 | ≥ 1.35V | PASS |
| VOL (V) | 0.016 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.47 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.87 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.614 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.614 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.975V, Trough=-0.259V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR56_EXT_SYNC_P1V8

#### U10-BF10 → U109-21

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.782 | ≥ 1.35V | PASS |
| VOL (V) | 0.018 | ≤ 0.45V | PASS |
| Rise Time (ns) | 1.59 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.57 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.514 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.559 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.782V, Trough=0.018V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U109-21 → U10-BF10

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.796 | ≥ 1.35V | PASS |
| VOL (V) | 0.004 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.92 | ≤ 10ns | PASS |
| Fall Time (ns) | 2.38 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.626 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.626 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.807V, Trough=-0.009V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR57_EXT_SYNC_P1V8

#### U10-BF9 → U109-20

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.782 | ≥ 1.35V | PASS |
| VOL (V) | 0.018 | ≤ 0.45V | PASS |
| Rise Time (ns) | 1.56 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.53 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.515 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.560 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.782V, Trough=0.018V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U109-20 → U10-BF9

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.797 | ≥ 1.35V | PASS |
| VOL (V) | 0.003 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.90 | ≤ 10ns | PASS |
| Fall Time (ns) | 2.36 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.627 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.627 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.814V, Trough=-0.018V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR58_EXT_SYNC_P1V8

#### U10-BC7 → U109-19

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.783 | ≥ 1.35V | PASS |
| VOL (V) | 0.017 | ≤ 0.45V | PASS |
| Rise Time (ns) | 1.51 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.49 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.515 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.560 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.783V, Trough=0.017V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U109-19 → U10-BC7

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.797 | ≥ 1.35V | PASS |
| VOL (V) | 0.003 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.90 | ≤ 10ns | PASS |
| Fall Time (ns) | 2.36 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.627 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.627 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.812V, Trough=-0.016V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR59_EXT_SYNC_P1V8

#### U10-BD7 → U109-18

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.783 | ≥ 1.35V | PASS |
| VOL (V) | 0.017 | ≤ 0.45V | PASS |
| Rise Time (ns) | 1.50 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.48 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.516 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.561 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.783V, Trough=0.017V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U109-18 → U10-BD7

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.797 | ≥ 1.35V | PASS |
| VOL (V) | 0.003 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.90 | ≤ 10ns | PASS |
| Fall Time (ns) | 2.36 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.627 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.627 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.813V, Trough=-0.016V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR5_EXT_SYNC_P1V8

#### U10-G13 → U126-16

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.741 | ≥ 1.35V | PASS |
| VOL (V) | 0.059 | ≤ 0.45V | PASS |
| Rise Time (ns) | 4.65 | ≤ 10ns | PASS |
| Fall Time (ns) | 4.64 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.473 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.518 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.741V, Trough=0.059V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U126-16 → U10-G13

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.788 | ≥ 1.35V | PASS |
| VOL (V) | 0.012 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.67 | ≤ 10ns | PASS |
| Fall Time (ns) | 2.06 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.618 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.618 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.928V, Trough=-0.151V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR60_EXT_SYNC_P1V8

#### U10-BD9 → U109-17

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.782 | ≥ 1.35V | PASS |
| VOL (V) | 0.018 | ≤ 0.45V | PASS |
| Rise Time (ns) | 1.50 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.48 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.515 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.560 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.782V, Trough=0.018V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U109-17 → U10-BD9

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.796 | ≥ 1.35V | PASS |
| VOL (V) | 0.004 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.91 | ≤ 10ns | PASS |
| Fall Time (ns) | 2.37 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.626 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.626 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.812V, Trough=-0.015V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR61_EXT_SYNC_P1V8

#### U10-BD8 → U109-16

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.783 | ≥ 1.35V | PASS |
| VOL (V) | 0.017 | ≤ 0.45V | PASS |
| Rise Time (ns) | 1.50 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.48 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.515 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.560 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.783V, Trough=0.017V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U109-16 → U10-BD8

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.797 | ≥ 1.35V | PASS |
| VOL (V) | 0.003 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.90 | ≤ 10ns | PASS |
| Fall Time (ns) | 2.36 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.627 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.627 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.812V, Trough=-0.016V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR62_EXT_SYNC_P1V8

#### U10-BA10 → U109-15

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.778 | ≥ 1.35V | PASS |
| VOL (V) | 0.022 | ≤ 0.45V | PASS |
| Rise Time (ns) | 1.76 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.73 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.511 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.556 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.778V, Trough=0.022V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U109-15 → U10-BA10

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.796 | ≥ 1.35V | PASS |
| VOL (V) | 0.004 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.88 | ≤ 10ns | PASS |
| Fall Time (ns) | 2.34 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.626 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.626 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.825V, Trough=-0.032V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR63_EXT_SYNC_P1V8

#### U10-BA9 → U109-14

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.776 | ≥ 1.35V | PASS |
| VOL (V) | 0.024 | ≤ 0.45V | PASS |
| Rise Time (ns) | 1.96 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.94 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.508 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.553 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.776V, Trough=0.024V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U109-14 → U10-BA9

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.795 | ≥ 1.35V | PASS |
| VOL (V) | 0.005 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.86 | ≤ 10ns | PASS |
| Fall Time (ns) | 2.32 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.625 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.625 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.836V, Trough=-0.045V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR6_EXT_SYNC_P1V8

#### U10-E16 → U126-15

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.731 | ≥ 1.35V | PASS |
| VOL (V) | 0.069 | ≤ 0.45V | PASS |
| Rise Time (ns) | 5.25 | ≤ 10ns | PASS |
| Fall Time (ns) | 5.23 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.464 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.509 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.731V, Trough=0.069V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U126-15 → U10-E16

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.786 | ≥ 1.35V | PASS |
| VOL (V) | 0.014 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.57 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.96 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.616 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.616 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.950V, Trough=-0.205V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR7_EXT_SYNC_P1V8

#### U10-E15 → U126-14

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.736 | ≥ 1.35V | PASS |
| VOL (V) | 0.064 | ≤ 0.45V | PASS |
| Rise Time (ns) | 5.01 | ≤ 10ns | PASS |
| Fall Time (ns) | 4.99 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.469 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.514 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.736V, Trough=0.064V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U126-14 → U10-E15

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.787 | ≥ 1.35V | PASS |
| VOL (V) | 0.013 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.62 | ≤ 10ns | PASS |
| Fall Time (ns) | 2.01 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.617 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.617 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.935V, Trough=-0.178V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### TR8_EXT_SYNC_P1V8

#### U10-R16 → U71-21

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.713 | ≥ 1.35V | PASS |
| VOL (V) | 0.087 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.77 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.75 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.446 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.491 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.713V, Trough=0.087V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U71-21 → U10-R16

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.782 | ≥ 1.35V | PASS |
| VOL (V) | 0.018 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.37 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.83 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.612 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.612 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.019V, Trough=-0.312V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TR9_EXT_SYNC_P1V8

#### U10-P15 → U71-20

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.713 | ≥ 1.35V | PASS |
| VOL (V) | 0.087 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.88 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.86 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.446 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.491 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.713V, Trough=0.087V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U71-20 → U10-P15

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.782 | ≥ 1.35V | PASS |
| VOL (V) | 0.018 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.37 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.83 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.612 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.612 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.019V, Trough=-0.312V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

### TE_EXT_SYNC_P1V8

#### U72-19 → U10-B29

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.785 | ≥ 1.35V | PASS |
| VOL (V) | 0.015 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.57 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.96 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.615 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.615 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.936V, Trough=-0.205V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U10-B29 → U72-19

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.728 | ≥ 1.35V | PASS |
| VOL (V) | 0.072 | ≤ 0.45V | PASS |
| Rise Time (ns) | 5.47 | ≤ 10ns | PASS |
| Fall Time (ns) | 5.45 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.460 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.505 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.728V, Trough=0.072V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### SYNC_EN_P1V8: U10-F15 → 8×U83 pins (14-21)

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.739 | ≥ 1.35V | PASS |
| VOL (V) | 0.061 | ≤ 0.45V | PASS |
| Rise Time (ns) | 8.43 | ≤ 10ns | PASS |
| Fall Time (ns) | 8.43 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.471 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.516 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.739V, Trough=0.061V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### DIR_AB_P1V8: U10-M14 → 8×U91 pins (14-21)

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.743 | ≥ 1.35V | PASS |
| VOL (V) | 0.057 | ≤ 0.45V | PASS |
| Rise Time (ns) | 8.00 | ≤ 10ns | PASS |
| Fall Time (ns) | 8.00 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.475 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.520 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.743V, Trough=0.057V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### SRS0_UCD_ALERT_P1V8: U10-AV33 → U17-3

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.638 | ≥ 1.35V | PASS |
| VOL (V) | 0.162 | ≤ 0.45V | PASS |
| Rise Time (ns) | 18.38 | ≤ 10ns | FAIL |
| Fall Time (ns) | 18.35 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.468 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.468 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.638V, Trough=0.162V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### SRS0_UCD_CLK_P1V8: U10-AV31 → U17-5

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.638 | ≥ 1.35V | PASS |
| VOL (V) | 0.162 | ≤ 0.45V | PASS |
| Rise Time (ns) | 18.49 | ≤ 10ns | FAIL |
| Fall Time (ns) | 18.46 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.468 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.468 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.638V, Trough=0.162V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### SRS0_UCD_DATA_P1V8: U10-AW33 → U17-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.638 | ≥ 1.35V | PASS |
| VOL (V) | 0.162 | ≤ 0.45V | PASS |
| Rise Time (ns) | 18.49 | ≤ 10ns | FAIL |
| Fall Time (ns) | 18.46 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.468 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.468 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.638V, Trough=0.162V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### SRS1_UCD_ALERT_P1V8: U10-D34 → U43-3

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.655 | ≥ 1.35V | PASS |
| VOL (V) | 0.145 | ≤ 0.45V | PASS |
| Rise Time (ns) | 15.09 | ≤ 10ns | FAIL |
| Fall Time (ns) | 15.06 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.485 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.485 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.655V, Trough=0.145V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### SRS1_UCD_CLK_P1V8: U10-J29 → U43-5

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.664 | ≥ 1.35V | PASS |
| VOL (V) | 0.136 | ≤ 0.45V | PASS |
| Rise Time (ns) | 13.94 | ≤ 10ns | FAIL |
| Fall Time (ns) | 13.94 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.494 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.494 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.664V, Trough=0.136V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### SRS1_UCD_DATA_P1V8: U10-D35 → U43-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.659 | ≥ 1.35V | PASS |
| VOL (V) | 0.141 | ≤ 0.45V | PASS |
| Rise Time (ns) | 14.81 | ≤ 10ns | FAIL |
| Fall Time (ns) | 14.79 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.489 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.489 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.659V, Trough=0.141V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### SRS2_UCD_ALERT_P1V8: U10-AU29 → U110-3

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.751 | ≥ 1.35V | PASS |
| VOL (V) | 0.049 | ≤ 0.45V | PASS |
| Rise Time (ns) | 3.83 | ≤ 10ns | PASS |
| Fall Time (ns) | 3.82 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.581 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.581 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.751V, Trough=0.049V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### SRS2_UCD_CLK_P1V8: U10-AU27 → U110-5

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.753 | ≥ 1.35V | PASS |
| VOL (V) | 0.047 | ≤ 0.45V | PASS |
| Rise Time (ns) | 3.49 | ≤ 10ns | PASS |
| Fall Time (ns) | 3.49 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.583 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.583 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.753V, Trough=0.047V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### SRS2_UCD_DATA_P1V8: U10-AU30 → U110-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.750 | ≥ 1.35V | PASS |
| VOL (V) | 0.050 | ≤ 0.45V | PASS |
| Rise Time (ns) | 3.94 | ≤ 10ns | PASS |
| Fall Time (ns) | 3.94 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.580 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.580 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.750V, Trough=0.050V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### SRS0_UCD_CNTRL_P1V8: U10-AY31 → U17-2

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.639 | ≥ 1.35V | PASS |
| VOL (V) | 0.161 | ≤ 0.45V | PASS |
| Rise Time (ns) | 17.97 | ≤ 10ns | FAIL |
| Fall Time (ns) | 17.93 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.469 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.469 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.639V, Trough=0.161V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### SRS1_UCD_CNTRL_P1V8: U10-K31 → U43-2

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.668 | ≥ 1.35V | PASS |
| VOL (V) | 0.132 | ≤ 0.45V | PASS |
| Rise Time (ns) | 13.41 | ≤ 10ns | FAIL |
| Fall Time (ns) | 13.44 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.498 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.498 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.668V, Trough=0.132V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### SRS2_UCD_CNTRL_P1V8: U10-AV29 → U110-2

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.753 | ≥ 1.35V | PASS |
| VOL (V) | 0.047 | ≤ 0.45V | PASS |
| Rise Time (ns) | 3.66 | ≤ 10ns | PASS |
| Fall Time (ns) | 3.67 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.583 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.583 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.753V, Trough=0.047V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### SRS0_PLL_SYNC_P1V8: U10-P28 → U67-3

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.691 | ≥ 1.35V | PASS |
| VOL (V) | 0.109 | ≤ 0.45V | PASS |
| Rise Time (ns) | 8.66 | ≤ 10ns | PASS |
| Fall Time (ns) | 8.63 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.521 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.521 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.691V, Trough=0.109V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### SRS1_PLL_SYNC_P1V8: U10-A29 → U67-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.714 | ≥ 1.35V | PASS |
| VOL (V) | 0.086 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.74 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.70 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.544 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.544 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.714V, Trough=0.086V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### SRS2_PLL_SYNC_P1V8: U10-A30 → U67-5

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.714 | ≥ 1.35V | PASS |
| VOL (V) | 0.086 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.74 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.70 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.544 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.544 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.714V, Trough=0.086V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### PRS_UCD_ALERT_P1V8: U10-D19 → U86-3

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.713 | ≥ 1.35V | PASS |
| VOL (V) | 0.087 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.63 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.60 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.543 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.543 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.713V, Trough=0.087V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### PRS_UCD_CLK_P1V8: U10-F17 → U86-5

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.713 | ≥ 1.35V | PASS |
| VOL (V) | 0.087 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.74 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.71 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.543 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.543 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.713V, Trough=0.087V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### PRS_UCD_DATA_P1V8: U10-F18 → U86-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.714 | ≥ 1.35V | PASS |
| VOL (V) | 0.086 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.74 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.70 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.544 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.544 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.714V, Trough=0.086V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### PRS_UCD_CNTRL_P1V8: U10-D20 → U86-2

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.719 | ≥ 1.35V | PASS |
| VOL (V) | 0.081 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.49 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.46 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.549 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.549 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.719V, Trough=0.081V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### PRS_SPI0_VX_CS_N_P1V8: U10-C18 → U90-5

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.773 | ≥ 1.35V | PASS |
| VOL (V) | 0.027 | ≤ 0.45V | PASS |
| Rise Time (ns) | 1.94 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.90 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.603 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.603 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.773V, Trough=0.027V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### PRS_SPI0_PS_CS_N_P1V8: U4-AC21 → U90-6

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.617 | ≥ 1.35V | PASS |
| VOL (V) | 0.183 | ≤ 0.45V | PASS |
| Rise Time (ns) | 0.28 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.24 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.447 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.447 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.617V, Trough=0.183V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### PRS_SPI0_SCLK_P1V8_VX: U10-C21 → U90-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.776 | ≥ 1.35V | PASS |
| VOL (V) | 0.024 | ≤ 0.45V | PASS |
| Rise Time (ns) | 1.86 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.84 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.606 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.606 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.776V, Trough=0.024V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### PRS_SPI0_SCLK_P1V8_PS: U4-AB20 → U90-10

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.647 | ≥ 1.35V | PASS |
| VOL (V) | 0.153 | ≤ 0.45V | PASS |
| Rise Time (ns) | 16.28 | ≤ 10ns | FAIL |
| Fall Time (ns) | 16.26 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.477 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.477 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.647V, Trough=0.153V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### PRS_SPI0_MOSI_P1V8

#### U10-C19 → U4-AB18

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.604 | ≥ 1.35V | PASS |
| VOL (V) | 0.196 | ≤ 0.45V | PASS |
| Rise Time (ns) | 0.28 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.24 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.434 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.434 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.604V, Trough=0.196V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### U4-AB18 → U10-C19

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.604 | ≥ 1.35V | PASS |
| VOL (V) | 0.196 | ≤ 0.45V | PASS |
| Rise Time (ns) | 0.28 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.24 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.434 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.434 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.604V, Trough=0.196V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### PRS_SPI0_MISO_P1V8

#### U81-16 → U10-B21

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.750 | ≥ 1.35V | PASS |
| VOL (V) | 0.050 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.31 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.90 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.580 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.580 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=2.277V, Trough=-0.477V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL |

#### U10-B21 → U81-16

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.577 | ≥ 1.35V | PASS |
| VOL (V) | 0.223 | ≤ 0.45V | PASS |
| Rise Time (ns) | 0.28 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.24 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.309 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.354 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.577V, Trough=0.223V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### PRS_PS_PCIE_RESET_N_P1V8: U4-G16 → U80-8

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.682 | ≥ 1.35V | PASS |
| VOL (V) | 0.118 | ≤ 0.45V | PASS |
| Rise Time (ns) | 9.45 | ≤ 10ns | PASS |
| Fall Time (ns) | 9.43 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.512 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.512 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.682V, Trough=0.118V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### PRS_PLL_SYNC_P1V8: U10-R28 → U67-2

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.691 | ≥ 1.35V | PASS |
| VOL (V) | 0.109 | ≤ 0.45V | PASS |
| Rise Time (ns) | 8.55 | ≤ 10ns | PASS |
| Fall Time (ns) | 8.52 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.521 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.521 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.691V, Trough=0.109V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### LMK_STATUS_LD1_P1V8: U10-D28 → U90-8

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.775 | ≥ 1.35V | PASS |
| VOL (V) | 0.025 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.03 | ≤ 10ns | PASS |
| Fall Time (ns) | 2.01 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.507 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.552 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.775V, Trough=0.025V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### LMK_STATUS_LD2_P1V8: U10-E28 → U90-7

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.775 | ≥ 1.35V | PASS |
| VOL (V) | 0.025 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.03 | ≤ 10ns | PASS |
| Fall Time (ns) | 2.01 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.508 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.553 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.775V, Trough=0.025V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### LMK_RST_P1V8: U81-21 → U10-G22

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.787 | ≥ 1.35V | PASS |
| VOL (V) | 0.013 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.62 | ≤ 10ns | PASS |
| Fall Time (ns) | 2.01 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.617 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.617 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.943V, Trough=-0.178V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### LMK_PLL_SYNC_P1V8: U72-14 → U10-C27

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.785 | ≥ 1.35V | PASS |
| VOL (V) | 0.015 | ≤ 0.45V | PASS |
| Rise Time (ns) | 2.52 | ≤ 10ns | PASS |
| Fall Time (ns) | 1.91 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.615 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.615 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.953V, Trough=-0.232V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### GE1_MDIO_P1V8

#### U874-41 → U78-5

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.755 | ≥ 1.35V | PASS |
| VOL (V) | 0.045 | ≤ 0.45V | PASS |
| Rise Time (ns) | 0.84 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.84 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.585 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.585 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.755V, Trough=0.045V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

#### Y2-4, Y1-4, U874-30, U874-18, U138-24, U138-23, U134-24, U134-23, U126-24, U126-23, U110-8, U110-1, U109-24, U109-23, U93-8, U93-1, U91-24, U91-23, U90-1, U87-24, U87-23, U86-8, U86-1, U83-24, U83-23, U82-3, U82-2, U81-24, U81-23, U80-1, U78-8, U78-1, U72-24, U72-23, U71-24, U71-23, U70-24, U70-23, U68-24, U68-23, U67-8, U67-1, U43-8, U43-1, U42-24, U42-23, U41-24, U41-23, U32-3, U32-2, U29-3, U29-2, U26-24, U26-23, U17-8, U17-1, U10-Y31, U10-Y29, U10-U13, U10-R12, U10-M28, U10-M23, U10-M18, U10-L31, U10-K34, U10-K14, U10-J37, U10-J27, U10-H30, U10-H20, U10-G23, U10-F26, U10-F16, U10-E19, U10-D37, U10-D32, U10-D22, U10-C15, U10-BD37, U10-BB33, U10-BB13, U10-BB8, U10-BA36, U10-AY29, U10-AW32, U10-AV35, U10-AU28, U10-AR14, U10-AP27, U10-AN30, U10-AH13, U10-AF31, U10-AF29, U10-AE32, U10-AE30, U10-AD31, U10-AD29, U10-AC32, U10-AC30, U10-AB31, U10-AB29, U10-AB23, U10-AA32, U10-AA30, U4-Y20, U4-W19, U4-V19, U4-U20, U4-U19, U4-P18, U4-P12, U4-N16, U4-M17, U4-M16, U4-M15, U4-M14, U4-M13, U4-L19, U4-H20, U4-G17, U4-F14, U4-E11, U4-D18, U4-C15, U4-B12, U4-AG17, U4-AG12, U4-AE16, U4-AD13, U4-AC10, U4-AB17, U4-AA14, L47-2, L6-1, J39-3, J34-2, J33-2, C1717-2, C1716-2, C1701-2, C1700-2, C1699-2, C1698-2, C1697-2, C1696-2, C1695-2, C1694-2, C1537-2, C1534-2, C1507-2, C1496-2, C1473-2, C1472-2, C1440-2, C1439-2, C1438-2, C1437-2, C1401-2, C1380-2, C1379-2, C1378-2, C1369-2, C1368-2, C1367-2, C1366-2, C1365-2, C1364-2, C1363-2, C1324-2, C1323-2, C1322-2, C1304-2, C1303-2, C1302-2, C1253-1, C1252-2, C978-2, C971-2, C963-2, C947-2, C805-1, C804-2, C713-2, C712-2, C691-2, C674-2, C673-2, C654-2, C647-2, C646-2, C458-2, C442-2, C441-2, C417-2, C379-2, C378-2, C373-2, C363-2, C358-2, C357-2, C350-2, C336-2, C323-1, C316-1, C294-2, C293-2, C292-2, C279-2, C275-2, C272-2, C270-2, C265-2, C258-2, C257-2, C255-2, C201-2, C198-2, C195-2, C194-2, C193-2, C147-2, C146-2, C100-2, C99-2, C95-2, C89-2, C83-2, C59-2, C53-2, C51-2, C30-2, C22-2, C21-2 → U874-41

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.755 | ≥ 1.35V | PASS |
| VOL (V) | 0.045 | ≤ 0.45V | PASS |
| Rise Time (ns) | 0.46 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.46 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.551 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.299 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.755V, Trough=0.045V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### GE1_MDC_P1V8: U874-42 → U80-9

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.758 | ≥ 1.35V | PASS |
| VOL (V) | 0.042 | ≤ 0.45V | PASS |
| Rise Time (ns) | 0.63 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.63 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.588 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.588 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.758V, Trough=0.042V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### FAN_EN_P1V8: U10-C28 → U72-20

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.723 | ≥ 1.35V | PASS |
| VOL (V) | 0.077 | ≤ 0.45V | PASS |
| Rise Time (ns) | 6.17 | ≤ 10ns | PASS |
| Fall Time (ns) | 6.14 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.456 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.501 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.723V, Trough=0.077V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### RUN_P1V8: U73-1 → U75-2

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.799 | ≥ 1.35V | PASS |
| VOL (V) | 0.001 | ≤ 0.45V | PASS |
| Rise Time (ns) | 0.52 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.49 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.629 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.629 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.799V, Trough=0.001V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### P48V_EN_P1V8: U10-C29 → U72-21

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.727 | ≥ 1.35V | PASS |
| VOL (V) | 0.073 | ≤ 0.45V | PASS |
| Rise Time (ns) | 5.48 | ≤ 10ns | PASS |
| Fall Time (ns) | 5.46 | ≤ 10ns | PASS |
| Noise Margin High (V) | 0.460 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.505 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.727V, Trough=0.073V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

### P3V3_P1V8_MGTV_RUN: TPS3808/ext → 4×LTC7151+TPS3808

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.668 | ≥ 1.35V | PASS |
| VOL (V) | 0.132 | ≤ 0.45V | PASS |
| Rise Time (ns) | 13.63 | ≤ 10ns | FAIL |
| Fall Time (ns) | 13.68 | ≤ 10ns | FAIL |
| Noise Margin High (V) | 0.498 | ≥ 0.18V | PASS |
| Noise Margin Low (V) | 0.498 | ≥ 0.18V | PASS |
| Overshoot/Undershoot (V) | Peak=1.668V, Trough=0.132V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS |

Specifications per JESD8-7A (1.8V LVCMOS).

## P1V8 POWER Signal Test

### CAL_SPI_CS_N

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 1.82 V | ≥ 1.17V (65% of VDD) | PASS |
| VOL (V) | -140.1 uV | ≤ 0.63V (35% of VDD) | PASS |
| Rise Time (ns) | 4.83 nS | ≤ 4ns | PASS |
| Fall Time (ns) | 4.27 nS | ≤ 4ns | PASS |
| Noise Margin High (V) | 650 mV | ≥ 0.27V | PASS |
| Noise Margin Low (V) | 629.9 mV | ≥ 0.27V | PASS |
| Overshoot/Undershoot (V) | 5.5%/3.8% | ≤ ±10% of 1.8V amplitude | PASS |

## LVDS Signal Test

### GTY_REFCLK_122: U77-60 / U77-61

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 0.24 | 0.24 | ≤ 2.08ns | PASS |
| Fall Time (ns) | 0.24 | 0.26 | ≤ 2.08ns | PASS |
| Peak Voltage (V) | 1.47 | 1.47 | ≤ 2.4V | PASS |
| Trough Voltage (V) | 1.02 | 1.02 | ≥ 0.0V | PASS |
| Overshoot (%) | 7.68 | 6.61 | < 10% | PASS |
| Undershoot (%) | 7.18 | 7.20 | < 10% | PASS |
| VOD (VP – VN) (mV) | 778 |  | 250–450mV | FAIL |
| VOS (V) | 1.244 |  | 1.125–1.375V | PASS |
| Noise Margin (V) | 0.29 |  | ≥ 0.1V | PASS |

### GTY_REFCLK_123: U77-56 / U77-57

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 0.24 | 0.25 | ≤ 2.08ns | PASS |
| Fall Time (ns) | 0.26 | 0.24 | ≤ 2.08ns | PASS |
| Peak Voltage (V) | 1.47 | 1.46 | ≤ 2.4V | PASS |
| Trough Voltage (V) | 1.03 | 1.03 | ≥ 0.0V | PASS |
| Overshoot (%) | 7.99 | 4.56 | < 10% | PASS |
| Undershoot (%) | 7.58 | 5.58 | < 10% | PASS |
| VOD (VP – VN) (mV) | 774 |  | 250–450mV | FAIL |
| VOS (V) | 1.247 |  | 1.125–1.375V | PASS |
| Noise Margin (V) | 0.29 |  | ≥ 0.1V | PASS |

### GTY_REFCLK_130: U77-51 / U77-52

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 0.24 | 0.24 | ≤ 2.08ns | PASS |
| Fall Time (ns) | 0.24 | 0.25 | ≤ 2.08ns | PASS |
| Peak Voltage (V) | 1.47 | 1.46 | ≤ 2.4V | PASS |
| Trough Voltage (V) | 1.02 | 1.03 | ≥ 0.0V | PASS |
| Overshoot (%) | 6.85 | 6.35 | < 10% | PASS |
| Undershoot (%) | 7.36 | 6.23 | < 10% | PASS |
| VOD (VP – VN) (mV) | 769 |  | 250–450mV | FAIL |
| VOS (V) | 1.244 |  | 1.125–1.375V | PASS |
| Noise Margin (V) | 0.28 |  | ≥ 0.1V | PASS |

### GTY_REFCLK_131: U77-49 / U77-50

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 0.25 | 0.24 | ≤ 2.08ns | PASS |
| Fall Time (ns) | 0.24 | 0.25 | ≤ 2.08ns | PASS |
| Peak Voltage (V) | 1.46 | 1.47 | ≤ 2.4V | PASS |
| Trough Voltage (V) | 1.03 | 1.03 | ≥ 0.0V | PASS |
| Overshoot (%) | 7.09 | 7.72 | < 10% | PASS |
| Undershoot (%) | 5.96 | 5.56 | < 10% | PASS |
| VOD (VP – VN) (mV) | 771 |  | 250–450mV | FAIL |
| VOS (V) | 1.248 |  | 1.125–1.375V | PASS |
| Noise Margin (V) | 0.29 |  | ≥ 0.1V | PASS |

### GTY_REFCLK_226: U77-3 / U77-4

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 0.28 | 0.27 | ≤ 2.08ns | PASS |
| Fall Time (ns) | 0.28 | 0.29 | ≤ 2.08ns | PASS |
| Peak Voltage (V) | 1.50 | 1.49 | ≤ 2.4V | PASS |
| Trough Voltage (V) | 0.99 | 1.00 | ≥ 0.0V | PASS |
| Overshoot (%) | 16.02 | 13.64 | < 10% | FAIL |
| Undershoot (%) | 13.93 | 11.95 | < 10% | FAIL |
| VOD (VP – VN) (mV) | 781 |  | 250–450mV | FAIL |
| VOS (V) | 1.245 |  | 1.125–1.375V | PASS |
| Noise Margin (V) | 0.29 |  | ≥ 0.1V | PASS |

### GTY_REFCLK_227: U77-13 / U77-14

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 0.29 | 0.29 | ≤ 2.08ns | PASS |
| Fall Time (ns) | 0.28 | 0.28 | ≤ 2.08ns | PASS |
| Peak Voltage (V) | 1.49 | 1.50 | ≤ 2.4V | PASS |
| Trough Voltage (V) | 0.99 | 0.99 | ≥ 0.0V | PASS |
| Overshoot (%) | 14.58 | 13.74 | < 10% | FAIL |
| Undershoot (%) | 13.60 | 15.73 | < 10% | FAIL |
| VOD (VP – VN) (mV) | 778 |  | 250–450mV | FAIL |
| VOS (V) | 1.246 |  | 1.125–1.375V | PASS |
| Noise Margin (V) | 0.28 |  | ≥ 0.1V | PASS |

### GTY_REFCLK_230: U77-1 / U77-2

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 0.24 | 0.24 | ≤ 2.08ns | PASS |
| Fall Time (ns) | 0.23 | 0.23 | ≤ 2.08ns | PASS |
| Peak Voltage (V) | 1.45 | 1.45 | ≤ 2.4V | PASS |
| Trough Voltage (V) | 1.04 | 1.05 | ≥ 0.0V | PASS |
| Overshoot (%) | 1.50 | 1.27 | < 10% | PASS |
| Undershoot (%) | 1.95 | 0.49 | < 10% | PASS |
| VOD (VP – VN) (mV) | 789 |  | 250–450mV | FAIL |
| VOS (V) | 1.245 |  | 1.125–1.375V | PASS |
| Noise Margin (V) | 0.29 |  | ≥ 0.1V | PASS |

### GTY_REFCLK_231: U77-15 / U77-16

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 0.24 | 0.23 | ≤ 2.08ns | PASS |
| Fall Time (ns) | 0.23 | 0.23 | ≤ 2.08ns | PASS |
| Peak Voltage (V) | 1.45 | 1.44 | ≤ 2.4V | PASS |
| Trough Voltage (V) | 1.04 | 1.05 | ≥ 0.0V | PASS |
| Overshoot (%) | 1.76 | 0.00 | < 10% | PASS |
| Undershoot (%) | 1.20 | 0.00 | < 10% | PASS |
| VOD (VP – VN) (mV) | 791 |  | 250–450mV | FAIL |
| VOS (V) | 1.241 |  | 1.125–1.375V | PASS |
| Noise Margin (V) | 0.29 |  | ≥ 0.1V | PASS |

### SRS2_EXT_CLK_IN: U77-24 / U77-25

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 0.28 | 0.27 | ≤ 2.08ns | PASS |
| Fall Time (ns) | 0.28 | 0.28 | ≤ 2.08ns | PASS |
| Peak Voltage (V) | 1.49 | 1.49 | ≤ 2.4V | PASS |
| Trough Voltage (V) | 1.00 | 1.00 | ≥ 0.0V | PASS |
| Overshoot (%) | 14.07 | 13.89 | < 10% | FAIL |
| Undershoot (%) | 11.91 | 12.54 | < 10% | FAIL |
| VOD (VP – VN) (mV) | 782 |  | 250–450mV | FAIL |
| VOS (V) | 1.245 |  | 1.125–1.375V | PASS |
| Noise Margin (V) | 0.29 |  | ≥ 0.1V | PASS |

### PRS_EXT_CLK_IN: U77-22 / U77-23

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 0.26 | 0.25 | ≤ 2.08ns | PASS |
| Fall Time (ns) | 0.24 | 0.25 | ≤ 2.08ns | PASS |
| Peak Voltage (V) | 1.46 | 1.46 | ≤ 2.4V | PASS |
| Trough Voltage (V) | 1.03 | 1.03 | ≥ 0.0V | PASS |
| Overshoot (%) | 6.56 | 5.88 | < 10% | PASS |
| Undershoot (%) | 4.74 | 5.64 | < 10% | PASS |
| VOD (VP – VN) (mV) | 779 |  | 250–450mV | FAIL |
| VOS (V) | 1.243 |  | 1.125–1.375V | PASS |
| Noise Margin (V) | 0.29 |  | ≥ 0.1V | PASS |

### SRS1_EXT_CLK_IN: U77-27 / U77-28

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 0.29 | 0.29 | ≤ 2.08ns | PASS |
| Fall Time (ns) | 0.29 | 0.28 | ≤ 2.08ns | PASS |
| Peak Voltage (V) | 1.49 | 1.49 | ≤ 2.4V | PASS |
| Trough Voltage (V) | 1.00 | 1.00 | ≥ 0.0V | PASS |
| Overshoot (%) | 13.90 | 13.63 | < 10% | FAIL |
| Undershoot (%) | 14.13 | 14.38 | < 10% | FAIL |
| VOD (VP – VN) (mV) | 778 |  | 250–450mV | FAIL |
| VOS (V) | 1.244 |  | 1.125–1.375V | PASS |
| Noise Margin (V) | 0.29 |  | ≥ 0.1V | PASS |

### SRS0_EXT_CLK_IN: U77-29 / U77-30

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 0.29 | 0.29 | ≤ 2.08ns | PASS |
| Fall Time (ns) | 0.29 | 0.29 | ≤ 2.08ns | PASS |
| Peak Voltage (V) | 1.50 | 1.50 | ≤ 2.4V | PASS |
| Trough Voltage (V) | 0.99 | 0.98 | ≥ 0.0V | PASS |
| Overshoot (%) | 16.10 | 15.92 | < 10% | FAIL |
| Undershoot (%) | 17.82 | 17.24 | < 10% | FAIL |
| VOD (VP – VN) (mV) | 778 |  | 250–450mV | FAIL |
| VOS (V) | 1.246 |  | 1.125–1.375V | PASS |
| Noise Margin (V) | 0.29 |  | ≥ 0.1V | PASS |

### PL_DDR4_REFCLK: U77-54 / U77-55

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 0.23 | 0.24 | ≤ 2.08ns | PASS |
| Fall Time (ns) | 0.24 | 0.24 | ≤ 2.08ns | PASS |
| Peak Voltage (V) | 1.45 | 1.45 | ≤ 2.4V | PASS |
| Trough Voltage (V) | 1.04 | 1.04 | ≥ 0.0V | PASS |
| Overshoot (%) | 1.15 | 0.55 | < 10% | PASS |
| Undershoot (%) | 1.37 | 2.95 | < 10% | PASS |
| VOD (VP – VN) (mV) | 788 |  | 250–450mV | FAIL |
| VOS (V) | 1.249 |  | 1.125–1.375V | PASS |
| Noise Margin (V) | 0.29 |  | ≥ 0.1V | PASS |

### AXI_REFCLK_233: U77-62 / U77-63

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 0.24 | 0.23 | ≤ 2.08ns | PASS |
| Fall Time (ns) | 0.23 | 0.24 | ≤ 2.08ns | PASS |
| Peak Voltage (V) | 1.45 | 1.45 | ≤ 2.4V | PASS |
| Trough Voltage (V) | 1.04 | 1.04 | ≥ 0.0V | PASS |
| Overshoot (%) | 2.61 | 2.82 | < 10% | PASS |
| Undershoot (%) | 1.81 | 1.81 | < 10% | PASS |
| VOD (VP – VN) (mV) | 778 |  | 250–450mV | FAIL |
| VOS (V) | 1.244 |  | 1.125–1.375V | PASS |
| Noise Margin (V) | 0.28 |  | ≥ 0.1V | PASS |

### PRS_PS_PCIE_REFCLK: U79-7 / U79-8

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 0.35 | 0.36 | ≤ 2.08ns | PASS |
| Fall Time (ns) | 0.35 | 0.35 | ≤ 2.08ns | PASS |
| Peak Voltage (V) | 1.47 | 1.48 | ≤ 2.4V | PASS |
| Trough Voltage (V) | 0.96 | 0.97 | ≥ 0.0V | PASS |
| Overshoot (%) | 10.91 | 13.87 | < 10% | FAIL |
| Undershoot (%) | 14.25 | 12.64 | < 10% | FAIL |
| VOD (VP – VN) (mV) | 809 |  | 250–450mV | FAIL |
| VOS (V) | 1.218 |  | 1.125–1.375V | PASS |
| Noise Margin (V) | 0.30 |  | ≥ 0.1V | PASS |

### PS_PCIE_REFCLK: U79-16 / U79-17

| Parameter | P Leg | N Leg | Specification | Pass/Fail |
| --- | --- | --- | --- | --- |
| Rise Time (ns) | 0.33 | 0.34 | ≤ 2.08ns | PASS |
| Fall Time (ns) | 0.33 | 0.33 | ≤ 2.08ns | PASS |
| Peak Voltage (V) | 1.43 | 1.43 | ≤ 2.4V | PASS |
| Trough Voltage (V) | 1.01 | 1.01 | ≥ 0.0V | PASS |
| Overshoot (%) | 1.42 | 3.36 | < 10% | PASS |
| Undershoot (%) | 2.66 | 2.55 | < 10% | PASS |
| VOD (VP – VN) (mV) | 803 |  | 250–450mV | FAIL |
| VOS (V) | 1.225 |  | 1.125–1.375V | PASS |
| Noise Margin (V) | 0.30 |  | ≥ 0.1V | PASS |

LVDS specifications per EIA-644 at 125MHz (T_bit=8ns). Transition time limit ≤ 0.26×T_bit = 2.08ns. Load: PCB differential pair (Z0=53–90Ω) into 100Ω differential termination at FPGA MGTREFCLK. LMK04828 driver: IBIS LVDS model (I_tail=3.9mA, tr/tf=202ps). AD9508 driver: IBIS LVDS model (I_tail=4.08mA, tr/tf=310/302ps).

## 3.3V LVCMOS Signal Test

### TR0_EXT_SYNC_P3V3

#### U128-1 → U128-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.295 | ≥ 2.4V | PASS |
| VOL (V) | 0.005 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.08 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.12 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.295 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.795 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.342V, Trough=-0.047V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U126-3 → U128-4, U128-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.13 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.80 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.694V, Trough=-0.584V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR10_EXT_SYNC_P3V3

#### U64-1 → U64-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.08 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.329V, Trough=-0.033V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U71-5 → U64-4, U64-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.27 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.98 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.690V, Trough=-0.385V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR11_EXT_SYNC_P3V3

#### U66-1 → U66-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.301V, Trough=-0.001V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U71-6 → U66-4, U66-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.42 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.15 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.336V, Trough=-0.047V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR12_EXT_SYNC_P3V3

#### U62-1 → U62-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.318V, Trough=-0.020V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U71-7 → U62-4, U62-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.30 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.03 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.528V, Trough=-0.269V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR13_EXT_SYNC_P3V3

#### U59-1 → U59-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.315V, Trough=-0.017V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U71-8 → U59-4, U59-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.31 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.07 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.479V, Trough=-0.259V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR14_EXT_SYNC_P3V3

#### U52-1 → U52-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.295 | ≥ 2.4V | PASS |
| VOL (V) | 0.005 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.07 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.11 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.295 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.795 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.332V, Trough=-0.036V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U71-9 → U52-4, U52-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.24 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.88 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.611V, Trough=-0.389V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR15_EXT_SYNC_P3V3

#### U50-1 → U50-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.294 | ≥ 2.4V | PASS |
| VOL (V) | 0.006 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.10 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.14 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.294 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.794 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.335V, Trough=-0.039V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U71-10 → U50-4, U50-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.13 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.80 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.614V, Trough=-0.580V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR16_EXT_SYNC_P3V3

#### U21-1 → U21-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.293 | ≥ 2.4V | PASS |
| VOL (V) | 0.007 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.09 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.09 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.293 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.793 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.351V, Trough=-0.057V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U41-3 → U21-4, U21-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 0.96 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.78 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.931V, Trough=-0.970V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR17_EXT_SYNC_P3V3

#### U23-1 → U23-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.294 | ≥ 2.4V | PASS |
| VOL (V) | 0.006 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.11 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.12 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.294 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.794 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.344V, Trough=-0.050V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U41-4 → U23-4, U23-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.03 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.78 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.771V, Trough=-0.776V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR18_EXT_SYNC_P3V3

#### U25-1 → U25-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.295 | ≥ 2.4V | PASS |
| VOL (V) | 0.005 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.10 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.13 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.295 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.795 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.338V, Trough=-0.043V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U41-5 → U25-4, U25-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.13 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.80 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.625V, Trough=-0.583V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR19_EXT_SYNC_P3V3

#### U28-1 → U28-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.05 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.09 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.326V, Trough=-0.029V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U41-6 → U28-4, U28-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.30 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.98 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.609V, Trough=-0.293V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR1_EXT_SYNC_P3V3

#### U125-1 → U125-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.06 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.10 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.325V, Trough=-0.028V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U126-4 → U125-4, U125-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.33 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.98 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.532V, Trough=-0.207V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR20_EXT_SYNC_P3V3

#### U27-1 → U27-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.06 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.10 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.324V, Trough=-0.027V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U41-7 → U27-4, U27-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.33 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.98 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.532V, Trough=-0.210V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR21_EXT_SYNC_P3V3

#### U24-1 → U24-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.295 | ≥ 2.4V | PASS |
| VOL (V) | 0.005 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.10 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.13 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.295 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.795 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.339V, Trough=-0.044V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U41-8 → U24-4, U24-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.13 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.80 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.624V, Trough=-0.586V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR22_EXT_SYNC_P3V3

#### U22-1 → U22-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.294 | ≥ 2.4V | PASS |
| VOL (V) | 0.006 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.08 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.08 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.294 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.794 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.352V, Trough=-0.059V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U41-9 → U22-4, U22-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 0.96 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.78 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.929V, Trough=-0.966V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR23_EXT_SYNC_P3V3

#### U19-1 → U19-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.293 | ≥ 2.4V | PASS |
| VOL (V) | 0.007 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.06 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.06 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.293 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.793 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.358V, Trough=-0.065V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U41-10 → U19-4, U19-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 0.95 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.78 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=4.084V, Trough=-1.145V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR24_EXT_SYNC_P3V3

#### U1-1 → U1-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.294 | ≥ 2.4V | PASS |
| VOL (V) | 0.006 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.11 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.12 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.294 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.794 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.343V, Trough=-0.048V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U26-3 → U1-4, U1-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.03 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.78 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.770V, Trough=-0.769V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR25_EXT_SYNC_P3V3

#### U5-1 → U5-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.295 | ≥ 2.4V | PASS |
| VOL (V) | 0.005 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.08 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.12 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.295 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.795 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.340V, Trough=-0.045V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U26-4 → U5-4, U5-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.13 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.80 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.692V, Trough=-0.579V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR26_EXT_SYNC_P3V3

#### U140-1 → U140-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.05 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.09 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.326V, Trough=-0.029V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U26-5 → U140-4, U140-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.30 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.98 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.610V, Trough=-0.293V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR27_EXT_SYNC_P3V3

#### U18-1 → U18-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.317V, Trough=-0.019V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U26-6 → U18-4, U18-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.33 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.04 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.488V, Trough=-0.220V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR28_EXT_SYNC_P3V3

#### U20-1 → U20-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.315V, Trough=-0.017V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U26-7 → U20-4, U20-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.33 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.07 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.469V, Trough=-0.217V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR29_EXT_SYNC_P3V3

#### U14-1 → U14-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.03 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.321V, Trough=-0.023V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U26-8 → U14-4, U14-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.31 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.06 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.587V, Trough=-0.288V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR2_EXT_SYNC_P3V3

#### U119-1 → U119-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.05 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.10 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.326V, Trough=-0.030V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U126-5 → U119-4, U119-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.30 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.98 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.613V, Trough=-0.293V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR30_EXT_SYNC_P3V3

#### U6-1 → U6-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.06 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.10 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.323V, Trough=-0.026V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U26-9 → U6-4, U6-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.33 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.98 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.532V, Trough=-0.211V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR31_EXT_SYNC_P3V3

#### U2-1 → U2-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.295 | ≥ 2.4V | PASS |
| VOL (V) | 0.005 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.10 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.13 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.295 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.795 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.337V, Trough=-0.042V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U26-10 → U2-4, U2-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.13 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.80 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.625V, Trough=-0.580V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR32_EXT_SYNC_P3V3

#### U3-1 → U3-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.295 | ≥ 2.4V | PASS |
| VOL (V) | 0.005 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.07 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.11 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.295 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.795 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.332V, Trough=-0.036V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U138-3 → U3-4, U3-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.24 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.88 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.611V, Trough=-0.389V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR33_EXT_SYNC_P3V3

#### U137-1 → U137-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.05 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.10 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.326V, Trough=-0.029V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U138-4 → U137-4, U137-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.30 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.98 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.611V, Trough=-0.294V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR34_EXT_SYNC_P3V3

#### U9-1 → U9-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.323V, Trough=-0.026V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U138-5 → U9-4, U9-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.27 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.03 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.595V, Trough=-0.385V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR35_EXT_SYNC_P3V3

#### U13-1 → U13-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.300V, Trough=-0.001V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U138-6 → U13-4, U13-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.44 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.17 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.310V, Trough=-0.013V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR36_EXT_SYNC_P3V3

#### U16-1 → U16-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.310V, Trough=-0.011V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U138-7 → U16-4, U16-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.36 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.08 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.424V, Trough=-0.141V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR37_EXT_SYNC_P3V3

#### U15-1 → U15-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.323V, Trough=-0.026V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U138-8 → U15-4, U15-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.27 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.03 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.596V, Trough=-0.386V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR38_EXT_SYNC_P3V3

#### U139-1 → U139-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.05 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.09 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.326V, Trough=-0.029V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U138-9 → U139-4, U139-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.30 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.98 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.611V, Trough=-0.293V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR39_EXT_SYNC_P3V3

#### U8-1 → U8-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.295 | ≥ 2.4V | PASS |
| VOL (V) | 0.005 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.09 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.13 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.295 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.795 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.338V, Trough=-0.042V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U138-10 → U8-4, U8-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.13 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.80 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.625V, Trough=-0.580V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR3_EXT_SYNC_P3V3

#### U113-1 → U113-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.317V, Trough=-0.020V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U126-6 → U113-4, U113-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.33 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.04 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.489V, Trough=-0.221V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR40_EXT_SYNC_P3V3

#### U61-1 → U61-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.309V, Trough=-0.010V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U70-3 → U61-4, U61-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.38 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.10 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.395V, Trough=-0.113V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR41_EXT_SYNC_P3V3

#### U57-1 → U57-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.309V, Trough=-0.010V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U70-4 → U57-4, U57-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.39 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.12 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.365V, Trough=-0.087V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR42_EXT_SYNC_P3V3

#### U51-1 → U51-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.309V, Trough=-0.010V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U70-5 → U51-4, U51-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.38 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.10 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.395V, Trough=-0.113V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR43_EXT_SYNC_P3V3

#### U49-1 → U49-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.317V, Trough=-0.019V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U70-6 → U49-4, U49-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.30 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.03 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.509V, Trough=-0.268V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR44_EXT_SYNC_P3V3

#### U54-1 → U54-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.309V, Trough=-0.010V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U70-7 → U54-4, U54-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.38 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.10 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.395V, Trough=-0.113V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR45_EXT_SYNC_P3V3

#### U56-1 → U56-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.310V, Trough=-0.011V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U70-8 → U56-4, U56-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.37 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.10 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.406V, Trough=-0.137V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR46_EXT_SYNC_P3V3

#### U63-1 → U63-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.309V, Trough=-0.010V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U70-9 → U63-4, U63-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.38 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.10 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.395V, Trough=-0.113V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR47_EXT_SYNC_P3V3

#### U65-1 → U65-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.318V, Trough=-0.020V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U70-10 → U65-4, U65-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.30 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.03 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.528V, Trough=-0.269V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR48_EXT_SYNC_P3V3

#### U101-1 → U101-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.290 | ≥ 2.4V | PASS |
| VOL (V) | 0.010 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.07 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.11 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.290 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.790 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.321V, Trough=-0.024V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U134-3 → U101-4, U101-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.294 | ≥ 2.4V | PASS |
| VOL (V) | 0.006 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.30 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.98 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.294 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.794 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.609V, Trough=-0.289V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR49_EXT_SYNC_P3V3

#### U99-1 → U99-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.289 | ≥ 2.4V | PASS |
| VOL (V) | 0.011 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.08 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.12 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.289 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.789 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.329V, Trough=-0.033V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U134-4 → U99-4, U99-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.293 | ≥ 2.4V | PASS |
| VOL (V) | 0.007 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.25 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.88 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.293 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.793 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.686V, Trough=-0.398V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR4_EXT_SYNC_P3V3

#### U106-1 → U106-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.07 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.315V, Trough=-0.017V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U126-7 → U106-4, U106-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.33 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.07 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.452V, Trough=-0.219V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR50_EXT_SYNC_P3V3

#### U97-1 → U97-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.288 | ≥ 2.4V | PASS |
| VOL (V) | 0.012 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.11 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.14 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.288 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.788 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.334V, Trough=-0.039V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U134-5 → U97-4, U97-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.293 | ≥ 2.4V | PASS |
| VOL (V) | 0.007 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.13 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.80 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.293 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.793 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.688V, Trough=-0.577V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR51_EXT_SYNC_P3V3

#### U95-1 → U95-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.287 | ≥ 2.4V | PASS |
| VOL (V) | 0.013 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.12 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.15 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.287 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.787 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.330V, Trough=-0.035V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U134-6 → U95-4, U95-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.292 | ≥ 2.4V | PASS |
| VOL (V) | 0.008 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.13 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.80 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.292 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.792 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.619V, Trough=-0.576V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR52_EXT_SYNC_P3V3

#### U96-1 → U96-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.287 | ≥ 2.4V | PASS |
| VOL (V) | 0.013 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.12 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.15 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.287 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.787 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.331V, Trough=-0.036V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U134-7 → U96-4, U96-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.292 | ≥ 2.4V | PASS |
| VOL (V) | 0.008 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.13 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.80 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.292 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.792 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.619V, Trough=-0.577V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR53_EXT_SYNC_P3V3

#### U98-1 → U98-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.289 | ≥ 2.4V | PASS |
| VOL (V) | 0.011 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.09 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.13 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.289 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.789 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.327V, Trough=-0.031V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U134-8 → U98-4, U98-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.293 | ≥ 2.4V | PASS |
| VOL (V) | 0.007 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.25 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.88 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.293 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.793 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.607V, Trough=-0.384V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR54_EXT_SYNC_P3V3

#### U100-1 → U100-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.291 | ≥ 2.4V | PASS |
| VOL (V) | 0.009 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.07 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.11 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.291 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.791 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.322V, Trough=-0.025V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U134-9 → U100-4, U100-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.294 | ≥ 2.4V | PASS |
| VOL (V) | 0.006 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.30 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.98 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.294 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.794 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.610V, Trough=-0.289V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR55_EXT_SYNC_P3V3

#### U103-1 → U103-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.292 | ≥ 2.4V | PASS |
| VOL (V) | 0.008 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.05 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.08 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.292 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.792 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.317V, Trough=-0.019V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U134-10 → U103-4, U103-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.295 | ≥ 2.4V | PASS |
| VOL (V) | 0.005 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.31 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.06 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.295 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.795 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.553V, Trough=-0.287V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR56_EXT_SYNC_P3V3

#### U131-1 → U131-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.290 | ≥ 2.4V | PASS |
| VOL (V) | 0.010 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.08 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.12 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.290 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.790 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.318V, Trough=-0.021V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U109-3 → U131-4, U131-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.294 | ≥ 2.4V | PASS |
| VOL (V) | 0.006 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.34 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.98 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.294 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.794 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.528V, Trough=-0.203V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR57_EXT_SYNC_P3V3

#### U127-1 → U127-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.291 | ≥ 2.4V | PASS |
| VOL (V) | 0.009 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.07 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.11 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.291 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.791 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.321V, Trough=-0.024V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U109-4 → U127-4, U127-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.295 | ≥ 2.4V | PASS |
| VOL (V) | 0.005 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.30 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.98 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.295 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.795 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.609V, Trough=-0.289V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR58_EXT_SYNC_P3V3

#### U117-1 → U117-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.292 | ≥ 2.4V | PASS |
| VOL (V) | 0.008 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.05 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.08 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.292 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.792 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.316V, Trough=-0.019V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U109-5 → U117-4, U117-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.295 | ≥ 2.4V | PASS |
| VOL (V) | 0.005 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.31 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.06 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.295 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.795 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.552V, Trough=-0.286V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR59_EXT_SYNC_P3V3

#### U105-1 → U105-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.293 | ≥ 2.4V | PASS |
| VOL (V) | 0.007 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.05 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.08 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.293 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.793 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.311V, Trough=-0.013V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U109-6 → U105-4, U105-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.31 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.07 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.477V, Trough=-0.259V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR5_EXT_SYNC_P3V3

#### U111-1 → U111-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.04 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.09 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.329V, Trough=-0.033V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U126-8 → U111-4, U111-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.27 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.98 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.669V, Trough=-0.388V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR60_EXT_SYNC_P3V3

#### U102-1 → U102-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.292 | ≥ 2.4V | PASS |
| VOL (V) | 0.008 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.05 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.08 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.292 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.792 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.317V, Trough=-0.019V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U109-7 → U102-4, U102-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.295 | ≥ 2.4V | PASS |
| VOL (V) | 0.005 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.31 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.06 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.295 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.795 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.587V, Trough=-0.286V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR61_EXT_SYNC_P3V3

#### U112-1 → U112-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.289 | ≥ 2.4V | PASS |
| VOL (V) | 0.011 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.08 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.12 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.289 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.789 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.317V, Trough=-0.020V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U109-8 → U112-4, U112-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.294 | ≥ 2.4V | PASS |
| VOL (V) | 0.006 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.34 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.98 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.294 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.794 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.527V, Trough=-0.203V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR62_EXT_SYNC_P3V3

#### U123-1 → U123-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.287 | ≥ 2.4V | PASS |
| VOL (V) | 0.013 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.12 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.15 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.287 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.787 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.330V, Trough=-0.035V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U109-9 → U123-4, U123-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.292 | ≥ 2.4V | PASS |
| VOL (V) | 0.008 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.13 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.80 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.292 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.792 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.619V, Trough=-0.576V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR63_EXT_SYNC_P3V3

#### U130-1 → U130-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.286 | ≥ 2.4V | PASS |
| VOL (V) | 0.014 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.14 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.14 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.286 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.786 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.336V, Trough=-0.041V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U109-10 → U130-4, U130-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.291 | ≥ 2.4V | PASS |
| VOL (V) | 0.009 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.03 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.78 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.291 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.791 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.765V, Trough=-0.769V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR6_EXT_SYNC_P3V3

#### U118-1 → U118-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.06 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.10 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.325V, Trough=-0.028V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U126-9 → U118-4, U118-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.33 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.98 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.532V, Trough=-0.208V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TR7_EXT_SYNC_P3V3

#### U120-1 → U120-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.295 | ≥ 2.4V | PASS |
| VOL (V) | 0.005 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.10 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.13 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.295 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.795 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.339V, Trough=-0.044V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U126-10 → U120-4, U120-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.13 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.80 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.624V, Trough=-0.585V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR8_EXT_SYNC_P3V3

#### U55-1 → U55-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.295 | ≥ 2.4V | PASS |
| VOL (V) | 0.005 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.07 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.11 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.295 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.795 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.332V, Trough=-0.036V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U71-3 → U55-4, U55-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.297 | ≥ 2.4V | PASS |
| VOL (V) | 0.003 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.24 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.88 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.297 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.797 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.611V, Trough=-0.389V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### TR9_EXT_SYNC_P3V3

#### U58-1 → U58-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.05 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.09 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.326V, Trough=-0.029V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

#### U71-4 → U58-4, U58-1

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.30 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.98 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.611V, Trough=-0.293V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### SYNC_EN0_P3V3: U83-3 → 8×HVD75 pins 2, 3 (16 loads)

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.250 | ≥ 2.4V | PASS |
| VOL (V) | 0.050 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.23 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.06 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.250 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.750 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=4.798V, Trough=-1.499V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### SYNC_EN1_P3V3: U83-4 → 8×HVD75 pins 2, 3 (16 loads)

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.267 | ≥ 2.4V | PASS |
| VOL (V) | 0.033 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.22 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.04 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.267 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.767 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=4.844V, Trough=-1.553V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### SYNC_EN2_P3V3: U83-5 → 8×HVD75 pins 2, 3 (16 loads)

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.249 | ≥ 2.4V | PASS |
| VOL (V) | 0.051 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.23 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.06 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.249 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.749 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=4.791V, Trough=-1.491V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### SYNC_EN3_P3V3: U83-6 → 8×HVD75 pins 2, 3 (16 loads)

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.267 | ≥ 2.4V | PASS |
| VOL (V) | 0.033 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.22 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.04 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.267 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.767 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=4.839V, Trough=-1.550V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### SYNC_EN4_P3V3: U83-7 → 8×HVD75 pins 2, 3 (16 loads)

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.254 | ≥ 2.4V | PASS |
| VOL (V) | 0.046 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.23 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.05 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.254 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.754 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=4.807V, Trough=-1.508V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### SYNC_EN5_P3V3: U83-8 → 8×HVD75 pins 2, 3 (16 loads)

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.293 | ≥ 2.4V | PASS |
| VOL (V) | 0.007 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.20 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.03 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.293 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.793 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=4.415V, Trough=-1.241V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### SYNC_EN6_P3V3: U83-9 → 8×HVD75 pins 2, 3 (16 loads)

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.278 | ≥ 2.4V | PASS |
| VOL (V) | 0.022 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.21 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.04 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.278 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.778 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=4.680V, Trough=-1.441V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### SYNC_EN7_P3V3: U83-10 → 8×HVD75 pins 2, 3 (16 loads)

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.282 | ≥ 2.4V | PASS |
| VOL (V) | 0.018 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.21 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.03 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.282 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.782 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=4.564V, Trough=-1.357V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### CAL_MBUS_TX_P3V3: U42-8 → U38-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.40 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.13 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.373V, Trough=-0.090V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### CBUS_TX_P3V3: U81-6 → U135-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.271 | ≥ 2.4V | PASS |
| VOL (V) | 0.029 | ≤ 0.4V | PASS |
| Rise Time (ns) | 0.96 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.78 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.271 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.771 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=4.884V, Trough=-1.584V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### MBUS_TX_P3V3: U81-4 → U132-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.271 | ≥ 2.4V | PASS |
| VOL (V) | 0.029 | ≤ 0.4V | PASS |
| Rise Time (ns) | 0.96 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.78 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.271 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.771 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=4.881V, Trough=-1.581V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### RXLO_MBUS_TX_P3V3: U87-8 → U838-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.40 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.11 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.395V, Trough=-0.098V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TXLO_MBUS_TX_P3V3: U68-8 → U60-4

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.31 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.06 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.496V, Trough=-0.292V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### CAL_MBUS_RX_P3V3: U38-1 → U80-19

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.271 | ≥ 2.4V | PASS |
| VOL (V) | 0.029 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.00 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.00 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.271 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.771 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.342V, Trough=-0.049V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### CBUS_RX_P3V3: U135-1 → U80-17

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.252 | ≥ 2.4V | PASS |
| VOL (V) | 0.048 | ≤ 0.4V | PASS |
| Rise Time (ns) | 9.77 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 8.76 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.252 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.752 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.395V, Trough=-0.111V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### MBUS_RX_P3V3: U132-1 → U80-18

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.252 | ≥ 2.4V | PASS |
| VOL (V) | 0.048 | ≤ 0.4V | PASS |
| Rise Time (ns) | 9.78 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 8.77 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.252 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.752 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.393V, Trough=-0.109V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### RXLO_MBUS_RX_P3V3: U838-1 → U80-21

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.290 | ≥ 2.4V | PASS |
| VOL (V) | 0.010 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.06 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.10 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.290 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.790 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.293V, Trough=0.007V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TXLO_MBUS_RX_P3V3: U60-1 → U80-20

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.289 | ≥ 2.4V | PASS |
| VOL (V) | 0.011 | ≤ 0.4V | PASS |
| Rise Time (ns) | 10.06 | ≤ 33ns †† | PASS |
| Fall Time (ns) | 9.10 | ≤ 33ns †† | PASS |
| Noise Margin High (V) | 1.289 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.789 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.292V, Trough=0.007V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### CAL_MBUS_EN_P3V3: U42-7 → U38-2, U38-3

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.298 | ≥ 2.4V | PASS |
| VOL (V) | 0.002 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.41 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.14 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.298 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.798 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.313V, Trough=-0.017V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### CBUS_EN_P3V3: U81-7 → U135-2, U135-3

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.271 | ≥ 2.4V | PASS |
| VOL (V) | 0.029 | ≤ 0.4V | PASS |
| Rise Time (ns) | 0.97 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.80 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.271 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.771 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=4.881V, Trough=-1.581V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### MBUS_EN_P3V3: U81-5 → U132-2, U132-3

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.270 | ≥ 2.4V | PASS |
| VOL (V) | 0.030 | ≤ 0.4V | PASS |
| Rise Time (ns) | 0.97 | ≤ 50ns † | PASS |
| Fall Time (ns) | 0.80 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.270 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.770 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=4.878V, Trough=-1.578V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### RXLO_MBUS_EN_P3V3: U87-7 → U838-2, U838-3

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.299 | ≥ 2.4V | PASS |
| VOL (V) | 0.001 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.33 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.07 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.299 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.799 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.452V, Trough=-0.219V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### TXLO_MBUS_EN_P3V3: U68-7 → U60-2, U60-3

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.296 | ≥ 2.4V | PASS |
| VOL (V) | 0.004 | ≤ 0.4V | PASS |
| Rise Time (ns) | 1.31 | ≤ 50ns † | PASS |
| Fall Time (ns) | 1.06 | ≤ 50ns † | PASS |
| Noise Margin High (V) | 1.296 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.796 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.514V, Trough=-0.288V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### LMK_STATUS_LD1_P3V3: U77-31 → U90-17

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.274 | ≥ 2.4V | PASS |
| VOL (V) | 0.026 | ≤ 0.4V | PASS |
| Rise Time (ns) | 0.77 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.77 | ≤ 10ns | PASS |
| Noise Margin High (V) | 1.274 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.774 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.355V, Trough=-0.055V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### LMK_STATUS_LD2_P3V3: U77-48 → U90-16

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.273 | ≥ 2.4V | PASS |
| VOL (V) | 0.027 | ≤ 0.4V | PASS |
| Rise Time (ns) | 0.77 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.77 | ≤ 10ns | PASS |
| Noise Margin High (V) | 1.273 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.773 | ≥ 0.4V | PASS |
| Overshoot/Undershoot (V) | Peak=3.340V, Trough=-0.040V | Peak ≤ 3.6V, Trough ≥ −0.3V | PASS |

### LMK_RST_P3V3: U81-3 → U77-5

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.289 | ≥ 2.4V | PASS |
| VOL (V) | 0.011 | ≤ 0.4V | PASS |
| Rise Time (ns) | 0.95 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.77 | ≤ 10ns | PASS |
| Noise Margin High (V) | 1.689 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.389 | ≥ 0.4V | FAIL |
| Overshoot/Undershoot (V) | Peak=4.936V, Trough=-1.636V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

### LMK_PLL_SYNC_P3V3: U72-10 → U77-6

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| VOH (V) | 3.288 | ≥ 2.4V | PASS |
| VOL (V) | 0.012 | ≤ 0.4V | PASS |
| Rise Time (ns) | 0.94 | ≤ 10ns | PASS |
| Fall Time (ns) | 0.77 | ≤ 10ns | PASS |
| Noise Margin High (V) | 2.088 | ≥ 0.4V | PASS |
| Noise Margin Low (V) | 0.388 | ≥ 0.4V | FAIL |
| Overshoot/Undershoot (V) | Peak=4.934V, Trough=-1.634V | Peak ≤ 3.6V, Trough ≥ −0.3V | FAIL |

Specifications per JESD8C.01 (3.3V LVCMOS).

† Rise/fall time (LVC8T245 → SN55HVD75 direction): JESD8C.01 specifies ≤ 10ns for general-purpose 3.3V LVCMOS. These signals interface with SN55HVD75 RS-422 transceivers operating at ≤ 10 Mbps. Per TIA/EIA-422-B, the transition time shall not exceed 50% of the unit interval (100ns at 10 Mbps), yielding a relaxed limit of ≤ 50ns.

†† Rise/fall time (SN55HVD75 → LVC8T245 direction): The SN74LVC8T245 datasheet specifies a maximum input transition rate of 10ns/V. At VCC = 3.3V, the maximum allowable input rise/fall time is 10 × 3.3 = 33ns.

## P12V POWER Signal Test

### PRS_P12V22

| Parameter | Measured Value | Specification | Pass/Fail |
| --- | --- | --- | --- |
| Voltage (No Load, V) | 1 | 12V ± 600mV |  |
| Voltage (Full Load, V) |  | 12V ± 600mV |  |
| Current (mA) |  | ≤ 10,000mA |  |
| Regulation (V) |  | ≤ 600mV |  |
| Ripple (Vpp) |  | ≤ 50mVpp |  |

## P28V POWER Signal Test

## P48V POWER Signal Test

## IF_ADC Signal Test

## IF_DAC Signal Test

## AXI CHIP2CHIP Signal Test

## PHY_ETH Signal Test

## OC_LOW Signal Test

## SGMII Signal Test

## FAN_PS_SIG Signal Test

## IF_ADC_SE Signal Test

## IF_DAC_SE Signal Test

## USB1_0 Signal Test

## USB_1VBUS Signal Test

## 2.5V LVCMOS Signal Test

## P28V_DISCRETE Signal Test

## PL_MGT Signal Test

## PS_GTR Signal Test