# 3.8    1.8V LVCMOS Signal Test (JESD8-7A)

Per JESD8-7A: VIH ≥ 0.65×VDD = 1.17V, VIL ≤ 0.35×VDD = 0.63V, VOH ≥ VDD−0.45V = 1.35V, VOL ≤ 0.45V, NM ≥ 0.18V, tr/tf ≤ 10ns @ 50pF, Absolute max: VDD+0.3V = 2.1V (overshoot), GND−0.3V = −0.3V (undershoot).

## 3.8.1    TR0_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR0_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.731 | ≥ 1.35V (VDD−0.45V) | PASS | U10-H14 | LVC8T245 via R428 |
| TR0_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.069 | ≤ 0.45V | PASS | U10-H14 | LVC8T245 via R428 |
| TR0_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 5.14 | ≤ 10ns (JESD8-7A) | PASS | U10-H14 | LVC8T245 via R428 |
| TR0_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 5.12 | ≤ 10ns (JESD8-7A) | PASS | U10-H14 | LVC8T245 via R428 |
| TR0_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.464 | ≥ 0.18V | PASS | U10-H14 | LVC8T245 via R428 |
| TR0_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.509 | ≥ 0.18V | PASS | U10-H14 | LVC8T245 via R428 |
| TR0_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.731V, Trough=0.069V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-H14 | LVC8T245 via R428 |
| TR0_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.786 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R428 | U10-H14 |
| TR0_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.014 | ≤ 0.45V | PASS | LVC8T245 via R428 | U10-H14 |
| TR0_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.57 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R428 | U10-H14 |
| TR0_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.96 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R428 | U10-H14 |
| TR0_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.616 | ≥ 0.18V | PASS | LVC8T245 via R428 | U10-H14 |
| TR0_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.616 | ≥ 0.18V | PASS | LVC8T245 via R428 | U10-H14 |
| TR0_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.950V, Trough=-0.205V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R428 | U10-H14 |

## 3.8.2    TR10_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR10_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.713 | ≥ 1.35V (VDD−0.45V) | PASS | U10-N14 | LVC8T245 via R267 |
| TR10_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.087 | ≤ 0.45V | PASS | U10-N14 | LVC8T245 via R267 |
| TR10_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.77 | ≤ 10ns (JESD8-7A) | PASS | U10-N14 | LVC8T245 via R267 |
| TR10_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.75 | ≤ 10ns (JESD8-7A) | PASS | U10-N14 | LVC8T245 via R267 |
| TR10_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.446 | ≥ 0.18V | PASS | U10-N14 | LVC8T245 via R267 |
| TR10_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.491 | ≥ 0.18V | PASS | U10-N14 | LVC8T245 via R267 |
| TR10_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.713V, Trough=0.087V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-N14 | LVC8T245 via R267 |
| TR10_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.782 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R267 | U10-N14 |
| TR10_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.018 | ≤ 0.45V | PASS | LVC8T245 via R267 | U10-N14 |
| TR10_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.37 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R267 | U10-N14 |
| TR10_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.83 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R267 | U10-N14 |
| TR10_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.612 | ≥ 0.18V | PASS | LVC8T245 via R267 | U10-N14 |
| TR10_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.612 | ≥ 0.18V | PASS | LVC8T245 via R267 | U10-N14 |
| TR10_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.019V, Trough=-0.312V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R267 | U10-N14 |

## 3.8.3    TR11_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR11_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.713 | ≥ 1.35V (VDD−0.45V) | PASS | U10-N13 | LVC8T245 via R266 |
| TR11_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.087 | ≤ 0.45V | PASS | U10-N13 | LVC8T245 via R266 |
| TR11_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.88 | ≤ 10ns (JESD8-7A) | PASS | U10-N13 | LVC8T245 via R266 |
| TR11_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.86 | ≤ 10ns (JESD8-7A) | PASS | U10-N13 | LVC8T245 via R266 |
| TR11_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.446 | ≥ 0.18V | PASS | U10-N13 | LVC8T245 via R266 |
| TR11_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.491 | ≥ 0.18V | PASS | U10-N13 | LVC8T245 via R266 |
| TR11_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.713V, Trough=0.087V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-N13 | LVC8T245 via R266 |
| TR11_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.782 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R266 | U10-N13 |
| TR11_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.018 | ≤ 0.45V | PASS | LVC8T245 via R266 | U10-N13 |
| TR11_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.37 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R266 | U10-N13 |
| TR11_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.83 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R266 | U10-N13 |
| TR11_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.612 | ≥ 0.18V | PASS | LVC8T245 via R266 | U10-N13 |
| TR11_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.612 | ≥ 0.18V | PASS | LVC8T245 via R266 | U10-N13 |
| TR11_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.019V, Trough=-0.312V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R266 | U10-N13 |

## 3.8.4    TR12_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR12_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.714 | ≥ 1.35V (VDD−0.45V) | PASS | U10-N16 | LVC8T245 via R265 |
| TR12_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.086 | ≤ 0.45V | PASS | U10-N16 | LVC8T245 via R265 |
| TR12_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.87 | ≤ 10ns (JESD8-7A) | PASS | U10-N16 | LVC8T245 via R265 |
| TR12_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.86 | ≤ 10ns (JESD8-7A) | PASS | U10-N16 | LVC8T245 via R265 |
| TR12_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.446 | ≥ 0.18V | PASS | U10-N16 | LVC8T245 via R265 |
| TR12_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.491 | ≥ 0.18V | PASS | U10-N16 | LVC8T245 via R265 |
| TR12_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.714V, Trough=0.086V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-N16 | LVC8T245 via R265 |
| TR12_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.782 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R265 | U10-N16 |
| TR12_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.018 | ≤ 0.45V | PASS | LVC8T245 via R265 | U10-N16 |
| TR12_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.37 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R265 | U10-N16 |
| TR12_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.83 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R265 | U10-N16 |
| TR12_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.612 | ≥ 0.18V | PASS | LVC8T245 via R265 | U10-N16 |
| TR12_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.612 | ≥ 0.18V | PASS | LVC8T245 via R265 | U10-N16 |
| TR12_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.019V, Trough=-0.313V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R265 | U10-N16 |

## 3.8.5    TR13_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR13_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.714 | ≥ 1.35V (VDD−0.45V) | PASS | U10-M16 | LVC8T245 via R264 |
| TR13_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.086 | ≤ 0.45V | PASS | U10-M16 | LVC8T245 via R264 |
| TR13_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.87 | ≤ 10ns (JESD8-7A) | PASS | U10-M16 | LVC8T245 via R264 |
| TR13_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.86 | ≤ 10ns (JESD8-7A) | PASS | U10-M16 | LVC8T245 via R264 |
| TR13_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.446 | ≥ 0.18V | PASS | U10-M16 | LVC8T245 via R264 |
| TR13_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.491 | ≥ 0.18V | PASS | U10-M16 | LVC8T245 via R264 |
| TR13_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.714V, Trough=0.086V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-M16 | LVC8T245 via R264 |
| TR13_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.782 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R264 | U10-M16 |
| TR13_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.018 | ≤ 0.45V | PASS | LVC8T245 via R264 | U10-M16 |
| TR13_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.37 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R264 | U10-M16 |
| TR13_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.83 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R264 | U10-M16 |
| TR13_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.612 | ≥ 0.18V | PASS | LVC8T245 via R264 | U10-M16 |
| TR13_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.612 | ≥ 0.18V | PASS | LVC8T245 via R264 | U10-M16 |
| TR13_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.019V, Trough=-0.313V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R264 | U10-M16 |

## 3.8.6    TR14_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR14_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.714 | ≥ 1.35V (VDD−0.45V) | PASS | U10-L15 | LVC8T245 via R263 |
| TR14_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.086 | ≤ 0.45V | PASS | U10-L15 | LVC8T245 via R263 |
| TR14_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.87 | ≤ 10ns (JESD8-7A) | PASS | U10-L15 | LVC8T245 via R263 |
| TR14_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.86 | ≤ 10ns (JESD8-7A) | PASS | U10-L15 | LVC8T245 via R263 |
| TR14_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.446 | ≥ 0.18V | PASS | U10-L15 | LVC8T245 via R263 |
| TR14_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.491 | ≥ 0.18V | PASS | U10-L15 | LVC8T245 via R263 |
| TR14_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.714V, Trough=0.086V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-L15 | LVC8T245 via R263 |
| TR14_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.782 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R263 | U10-L15 |
| TR14_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.018 | ≤ 0.45V | PASS | LVC8T245 via R263 | U10-L15 |
| TR14_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.37 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R263 | U10-L15 |
| TR14_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.83 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R263 | U10-L15 |
| TR14_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.612 | ≥ 0.18V | PASS | LVC8T245 via R263 | U10-L15 |
| TR14_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.612 | ≥ 0.18V | PASS | LVC8T245 via R263 | U10-L15 |
| TR14_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.019V, Trough=-0.313V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R263 | U10-L15 |

## 3.8.7    TR15_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR15_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.714 | ≥ 1.35V (VDD−0.45V) | PASS | U10-K15 | LVC8T245 via R262 |
| TR15_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.086 | ≤ 0.45V | PASS | U10-K15 | LVC8T245 via R262 |
| TR15_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.98 | ≤ 10ns (JESD8-7A) | PASS | U10-K15 | LVC8T245 via R262 |
| TR15_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.97 | ≤ 10ns (JESD8-7A) | PASS | U10-K15 | LVC8T245 via R262 |
| TR15_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.446 | ≥ 0.18V | PASS | U10-K15 | LVC8T245 via R262 |
| TR15_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.491 | ≥ 0.18V | PASS | U10-K15 | LVC8T245 via R262 |
| TR15_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.714V, Trough=0.086V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-K15 | LVC8T245 via R262 |
| TR15_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.782 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R262 | U10-K15 |
| TR15_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.018 | ≤ 0.45V | PASS | LVC8T245 via R262 | U10-K15 |
| TR15_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.37 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R262 | U10-K15 |
| TR15_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.83 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R262 | U10-K15 |
| TR15_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.612 | ≥ 0.18V | PASS | LVC8T245 via R262 | U10-K15 |
| TR15_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.612 | ≥ 0.18V | PASS | LVC8T245 via R262 | U10-K15 |
| TR15_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.019V, Trough=-0.313V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R262 | U10-K15 |

## 3.8.8    TR16_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR16_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.674 | ≥ 1.35V (VDD−0.45V) | PASS | U10-E36 | LVC8T245 via R141 |
| TR16_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.126 | ≤ 0.45V | PASS | U10-E36 | LVC8T245 via R141 |
| TR16_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 10.66 | ≤ 10ns (JESD8-7A) | FAIL | U10-E36 | LVC8T245 via R141 |
| TR16_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 10.63 | ≤ 10ns (JESD8-7A) | FAIL | U10-E36 | LVC8T245 via R141 |
| TR16_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.407 | ≥ 0.18V | PASS | U10-E36 | LVC8T245 via R141 |
| TR16_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.452 | ≥ 0.18V | PASS | U10-E36 | LVC8T245 via R141 |
| TR16_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.674V, Trough=0.126V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-E36 | LVC8T245 via R141 |
| TR16_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.774 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R141 | U10-E36 |
| TR16_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.026 | ≤ 0.45V | PASS | LVC8T245 via R141 | U10-E36 |
| TR16_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.25 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R141 | U10-E36 |
| TR16_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.85 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R141 | U10-E36 |
| TR16_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.604 | ≥ 0.18V | PASS | LVC8T245 via R141 | U10-E36 |
| TR16_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.604 | ≥ 0.18V | PASS | LVC8T245 via R141 | U10-E36 |
| TR16_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.231V, Trough=-0.526V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R141 | U10-E36 |

## 3.8.9    TR17_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR17_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.674 | ≥ 1.35V (VDD−0.45V) | PASS | U10-E37 | LVC8T245 via R140 |
| TR17_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.126 | ≤ 0.45V | PASS | U10-E37 | LVC8T245 via R140 |
| TR17_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 10.55 | ≤ 10ns (JESD8-7A) | FAIL | U10-E37 | LVC8T245 via R140 |
| TR17_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 10.52 | ≤ 10ns (JESD8-7A) | FAIL | U10-E37 | LVC8T245 via R140 |
| TR17_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.407 | ≥ 0.18V | PASS | U10-E37 | LVC8T245 via R140 |
| TR17_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.452 | ≥ 0.18V | PASS | U10-E37 | LVC8T245 via R140 |
| TR17_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.674V, Trough=0.126V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-E37 | LVC8T245 via R140 |
| TR17_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.774 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R140 | U10-E37 |
| TR17_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.026 | ≤ 0.45V | PASS | LVC8T245 via R140 | U10-E37 |
| TR17_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.25 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R140 | U10-E37 |
| TR17_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.85 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R140 | U10-E37 |
| TR17_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.604 | ≥ 0.18V | PASS | LVC8T245 via R140 | U10-E37 |
| TR17_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.604 | ≥ 0.18V | PASS | LVC8T245 via R140 | U10-E37 |
| TR17_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.231V, Trough=-0.526V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R140 | U10-E37 |

## 3.8.10    TR18_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR18_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.674 | ≥ 1.35V (VDD−0.45V) | PASS | U10-G38 | LVC8T245 via R139 |
| TR18_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.126 | ≤ 0.45V | PASS | U10-G38 | LVC8T245 via R139 |
| TR18_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 10.55 | ≤ 10ns (JESD8-7A) | FAIL | U10-G38 | LVC8T245 via R139 |
| TR18_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 10.52 | ≤ 10ns (JESD8-7A) | FAIL | U10-G38 | LVC8T245 via R139 |
| TR18_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.407 | ≥ 0.18V | PASS | U10-G38 | LVC8T245 via R139 |
| TR18_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.452 | ≥ 0.18V | PASS | U10-G38 | LVC8T245 via R139 |
| TR18_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.674V, Trough=0.126V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-G38 | LVC8T245 via R139 |
| TR18_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.774 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R139 | U10-G38 |
| TR18_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.026 | ≤ 0.45V | PASS | LVC8T245 via R139 | U10-G38 |
| TR18_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.25 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R139 | U10-G38 |
| TR18_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.85 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R139 | U10-G38 |
| TR18_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.604 | ≥ 0.18V | PASS | LVC8T245 via R139 | U10-G38 |
| TR18_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.604 | ≥ 0.18V | PASS | LVC8T245 via R139 | U10-G38 |
| TR18_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.231V, Trough=-0.526V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R139 | U10-G38 |

## 3.8.11    TR19_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR19_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.679 | ≥ 1.35V (VDD−0.45V) | PASS | U10-F38 | LVC8T245 via R138 |
| TR19_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.121 | ≤ 0.45V | PASS | U10-F38 | LVC8T245 via R138 |
| TR19_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 10.06 | ≤ 10ns (JESD8-7A) | FAIL | U10-F38 | LVC8T245 via R138 |
| TR19_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 10.04 | ≤ 10ns (JESD8-7A) | FAIL | U10-F38 | LVC8T245 via R138 |
| TR19_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.411 | ≥ 0.18V | PASS | U10-F38 | LVC8T245 via R138 |
| TR19_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.456 | ≥ 0.18V | PASS | U10-F38 | LVC8T245 via R138 |
| TR19_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.679V, Trough=0.121V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-F38 | LVC8T245 via R138 |
| TR19_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.775 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R138 | U10-F38 |
| TR19_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.025 | ≤ 0.45V | PASS | LVC8T245 via R138 | U10-F38 |
| TR19_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.24 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R138 | U10-F38 |
| TR19_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.84 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R138 | U10-F38 |
| TR19_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.605 | ≥ 0.18V | PASS | LVC8T245 via R138 | U10-F38 |
| TR19_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.605 | ≥ 0.18V | PASS | LVC8T245 via R138 | U10-F38 |
| TR19_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.212V, Trough=-0.516V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R138 | U10-F38 |

## 3.8.12    TR1_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR1_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.736 | ≥ 1.35V (VDD−0.45V) | PASS | U10-H13 | LVC8T245 via R429 |
| TR1_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.064 | ≤ 0.45V | PASS | U10-H13 | LVC8T245 via R429 |
| TR1_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 4.78 | ≤ 10ns (JESD8-7A) | PASS | U10-H13 | LVC8T245 via R429 |
| TR1_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 4.77 | ≤ 10ns (JESD8-7A) | PASS | U10-H13 | LVC8T245 via R429 |
| TR1_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.469 | ≥ 0.18V | PASS | U10-H13 | LVC8T245 via R429 |
| TR1_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.514 | ≥ 0.18V | PASS | U10-H13 | LVC8T245 via R429 |
| TR1_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.736V, Trough=0.064V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-H13 | LVC8T245 via R429 |
| TR1_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.787 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R429 | U10-H13 |
| TR1_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.013 | ≤ 0.45V | PASS | LVC8T245 via R429 | U10-H13 |
| TR1_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.68 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R429 | U10-H13 |
| TR1_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 2.06 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R429 | U10-H13 |
| TR1_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.617 | ≥ 0.18V | PASS | LVC8T245 via R429 | U10-H13 |
| TR1_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.617 | ≥ 0.18V | PASS | LVC8T245 via R429 | U10-H13 |
| TR1_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.919V, Trough=-0.149V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R429 | U10-H13 |

## 3.8.13    TR20_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR20_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.679 | ≥ 1.35V (VDD−0.45V) | PASS | U10-E39 | LVC8T245 via R137 |
| TR20_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.121 | ≤ 0.45V | PASS | U10-E39 | LVC8T245 via R137 |
| TR20_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 9.95 | ≤ 10ns (JESD8-7A) | PASS | U10-E39 | LVC8T245 via R137 |
| TR20_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 9.93 | ≤ 10ns (JESD8-7A) | PASS | U10-E39 | LVC8T245 via R137 |
| TR20_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.411 | ≥ 0.18V | PASS | U10-E39 | LVC8T245 via R137 |
| TR20_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.456 | ≥ 0.18V | PASS | U10-E39 | LVC8T245 via R137 |
| TR20_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.679V, Trough=0.121V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-E39 | LVC8T245 via R137 |
| TR20_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.775 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R137 | U10-E39 |
| TR20_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.025 | ≤ 0.45V | PASS | LVC8T245 via R137 | U10-E39 |
| TR20_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.24 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R137 | U10-E39 |
| TR20_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.84 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R137 | U10-E39 |
| TR20_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.605 | ≥ 0.18V | PASS | LVC8T245 via R137 | U10-E39 |
| TR20_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.605 | ≥ 0.18V | PASS | LVC8T245 via R137 | U10-E39 |
| TR20_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.189V, Trough=-0.503V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R137 | U10-E39 |

## 3.8.14    TR21_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR21_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.679 | ≥ 1.35V (VDD−0.45V) | PASS | U10-E40 | LVC8T245 via R136 |
| TR21_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.121 | ≤ 0.45V | PASS | U10-E40 | LVC8T245 via R136 |
| TR21_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 10.06 | ≤ 10ns (JESD8-7A) | FAIL | U10-E40 | LVC8T245 via R136 |
| TR21_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 10.04 | ≤ 10ns (JESD8-7A) | FAIL | U10-E40 | LVC8T245 via R136 |
| TR21_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.411 | ≥ 0.18V | PASS | U10-E40 | LVC8T245 via R136 |
| TR21_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.456 | ≥ 0.18V | PASS | U10-E40 | LVC8T245 via R136 |
| TR21_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.679V, Trough=0.121V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-E40 | LVC8T245 via R136 |
| TR21_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.775 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R136 | U10-E40 |
| TR21_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.025 | ≤ 0.45V | PASS | LVC8T245 via R136 | U10-E40 |
| TR21_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.24 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R136 | U10-E40 |
| TR21_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.84 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R136 | U10-E40 |
| TR21_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.605 | ≥ 0.18V | PASS | LVC8T245 via R136 | U10-E40 |
| TR21_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.605 | ≥ 0.18V | PASS | LVC8T245 via R136 | U10-E40 |
| TR21_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.189V, Trough=-0.503V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R136 | U10-E40 |

## 3.8.15    TR22_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR22_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.679 | ≥ 1.35V (VDD−0.45V) | PASS | U10-F35 | LVC8T245 via R135 |
| TR22_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.121 | ≤ 0.45V | PASS | U10-F35 | LVC8T245 via R135 |
| TR22_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 10.28 | ≤ 10ns (JESD8-7A) | FAIL | U10-F35 | LVC8T245 via R135 |
| TR22_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 10.26 | ≤ 10ns (JESD8-7A) | FAIL | U10-F35 | LVC8T245 via R135 |
| TR22_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.411 | ≥ 0.18V | PASS | U10-F35 | LVC8T245 via R135 |
| TR22_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.456 | ≥ 0.18V | PASS | U10-F35 | LVC8T245 via R135 |
| TR22_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.679V, Trough=0.121V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-F35 | LVC8T245 via R135 |
| TR22_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.775 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R135 | U10-F35 |
| TR22_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.025 | ≤ 0.45V | PASS | LVC8T245 via R135 | U10-F35 |
| TR22_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.24 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R135 | U10-F35 |
| TR22_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.84 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R135 | U10-F35 |
| TR22_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.605 | ≥ 0.18V | PASS | LVC8T245 via R135 | U10-F35 |
| TR22_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.605 | ≥ 0.18V | PASS | LVC8T245 via R135 | U10-F35 |
| TR22_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.212V, Trough=-0.516V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R135 | U10-F35 |

## 3.8.16    TR23_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR23_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.678 | ≥ 1.35V (VDD−0.45V) | PASS | U10-E35 | LVC8T245 via R134 |
| TR23_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.122 | ≤ 0.45V | PASS | U10-E35 | LVC8T245 via R134 |
| TR23_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 10.34 | ≤ 10ns (JESD8-7A) | FAIL | U10-E35 | LVC8T245 via R134 |
| TR23_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 10.32 | ≤ 10ns (JESD8-7A) | FAIL | U10-E35 | LVC8T245 via R134 |
| TR23_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.410 | ≥ 0.18V | PASS | U10-E35 | LVC8T245 via R134 |
| TR23_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.455 | ≥ 0.18V | PASS | U10-E35 | LVC8T245 via R134 |
| TR23_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.678V, Trough=0.122V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-E35 | LVC8T245 via R134 |
| TR23_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.774 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R134 | U10-E35 |
| TR23_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.026 | ≤ 0.45V | PASS | LVC8T245 via R134 | U10-E35 |
| TR23_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.25 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R134 | U10-E35 |
| TR23_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.85 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R134 | U10-E35 |
| TR23_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.604 | ≥ 0.18V | PASS | LVC8T245 via R134 | U10-E35 |
| TR23_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.604 | ≥ 0.18V | PASS | LVC8T245 via R134 | U10-E35 |
| TR23_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.209V, Trough=-0.512V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R134 | U10-E35 |

## 3.8.17    TR24_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR24_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.661 | ≥ 1.35V (VDD−0.45V) | PASS | U10-L35 | LVC8T245 via R84 |
| TR24_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.139 | ≤ 0.45V | PASS | U10-L35 | LVC8T245 via R84 |
| TR24_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 14.63 | ≤ 10ns (JESD8-7A) | FAIL | U10-L35 | LVC8T245 via R84 |
| TR24_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 14.63 | ≤ 10ns (JESD8-7A) | FAIL | U10-L35 | LVC8T245 via R84 |
| TR24_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.394 | ≥ 0.18V | PASS | U10-L35 | LVC8T245 via R84 |
| TR24_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.439 | ≥ 0.18V | PASS | U10-L35 | LVC8T245 via R84 |
| TR24_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.661V, Trough=0.139V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-L35 | LVC8T245 via R84 |
| TR24_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.771 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R84 | U10-L35 |
| TR24_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.029 | ≤ 0.45V | PASS | LVC8T245 via R84 | U10-L35 |
| TR24_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.25 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R84 | U10-L35 |
| TR24_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.85 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R84 | U10-L35 |
| TR24_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.601 | ≥ 0.18V | PASS | LVC8T245 via R84 | U10-L35 |
| TR24_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.601 | ≥ 0.18V | PASS | LVC8T245 via R84 | U10-L35 |
| TR24_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.284V, Trough=-0.531V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R84 | U10-L35 |

## 3.8.18    TR25_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR25_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.660 | ≥ 1.35V (VDD−0.45V) | PASS | U10-L36 | LVC8T245 via R83 |
| TR25_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.140 | ≤ 0.45V | PASS | U10-L36 | LVC8T245 via R83 |
| TR25_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 14.78 | ≤ 10ns (JESD8-7A) | FAIL | U10-L36 | LVC8T245 via R83 |
| TR25_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 14.77 | ≤ 10ns (JESD8-7A) | FAIL | U10-L36 | LVC8T245 via R83 |
| TR25_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.393 | ≥ 0.18V | PASS | U10-L36 | LVC8T245 via R83 |
| TR25_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.438 | ≥ 0.18V | PASS | U10-L36 | LVC8T245 via R83 |
| TR25_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.660V, Trough=0.140V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-L36 | LVC8T245 via R83 |
| TR25_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.770 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R83 | U10-L36 |
| TR25_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.030 | ≤ 0.45V | PASS | LVC8T245 via R83 | U10-L36 |
| TR25_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.26 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R83 | U10-L36 |
| TR25_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.85 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R83 | U10-L36 |
| TR25_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.600 | ≥ 0.18V | PASS | LVC8T245 via R83 | U10-L36 |
| TR25_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.600 | ≥ 0.18V | PASS | LVC8T245 via R83 | U10-L36 |
| TR25_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.297V, Trough=-0.528V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R83 | U10-L36 |

## 3.8.19    TR26_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR26_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.666 | ≥ 1.35V (VDD−0.45V) | PASS | U10-L33 | LVC8T245 via R82 |
| TR26_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.134 | ≤ 0.45V | PASS | U10-L33 | LVC8T245 via R82 |
| TR26_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 13.95 | ≤ 10ns (JESD8-7A) | FAIL | U10-L33 | LVC8T245 via R82 |
| TR26_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 13.98 | ≤ 10ns (JESD8-7A) | FAIL | U10-L33 | LVC8T245 via R82 |
| TR26_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.398 | ≥ 0.18V | PASS | U10-L33 | LVC8T245 via R82 |
| TR26_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.443 | ≥ 0.18V | PASS | U10-L33 | LVC8T245 via R82 |
| TR26_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.666V, Trough=0.134V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-L33 | LVC8T245 via R82 |
| TR26_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.772 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R82 | U10-L33 |
| TR26_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.028 | ≤ 0.45V | PASS | LVC8T245 via R82 | U10-L33 |
| TR26_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.25 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R82 | U10-L33 |
| TR26_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.85 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R82 | U10-L33 |
| TR26_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.602 | ≥ 0.18V | PASS | LVC8T245 via R82 | U10-L33 |
| TR26_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.602 | ≥ 0.18V | PASS | LVC8T245 via R82 | U10-L33 |
| TR26_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.267V, Trough=-0.532V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R82 | U10-L33 |

## 3.8.20    TR27_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR27_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.665 | ≥ 1.35V (VDD−0.45V) | PASS | U10-K33 | LVC8T245 via R81 |
| TR27_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.135 | ≤ 0.45V | PASS | U10-K33 | LVC8T245 via R81 |
| TR27_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 13.91 | ≤ 10ns (JESD8-7A) | FAIL | U10-K33 | LVC8T245 via R81 |
| TR27_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 13.92 | ≤ 10ns (JESD8-7A) | FAIL | U10-K33 | LVC8T245 via R81 |
| TR27_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.397 | ≥ 0.18V | PASS | U10-K33 | LVC8T245 via R81 |
| TR27_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.442 | ≥ 0.18V | PASS | U10-K33 | LVC8T245 via R81 |
| TR27_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.665V, Trough=0.135V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-K33 | LVC8T245 via R81 |
| TR27_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.771 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R81 | U10-K33 |
| TR27_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.029 | ≤ 0.45V | PASS | LVC8T245 via R81 | U10-K33 |
| TR27_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.26 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R81 | U10-K33 |
| TR27_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.85 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R81 | U10-K33 |
| TR27_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.601 | ≥ 0.18V | PASS | LVC8T245 via R81 | U10-K33 |
| TR27_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.601 | ≥ 0.18V | PASS | LVC8T245 via R81 | U10-K33 |
| TR27_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.264V, Trough=-0.528V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R81 | U10-K33 |

## 3.8.21    TR28_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR28_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.669 | ≥ 1.35V (VDD−0.45V) | PASS | U10-K35 | LVC8T245 via R80 |
| TR28_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.131 | ≤ 0.45V | PASS | U10-K35 | LVC8T245 via R80 |
| TR28_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 11.60 | ≤ 10ns (JESD8-7A) | FAIL | U10-K35 | LVC8T245 via R80 |
| TR28_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 11.59 | ≤ 10ns (JESD8-7A) | FAIL | U10-K35 | LVC8T245 via R80 |
| TR28_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.401 | ≥ 0.18V | PASS | U10-K35 | LVC8T245 via R80 |
| TR28_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.446 | ≥ 0.18V | PASS | U10-K35 | LVC8T245 via R80 |
| TR28_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.669V, Trough=0.131V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-K35 | LVC8T245 via R80 |
| TR28_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.772 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R80 | U10-K35 |
| TR28_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.028 | ≤ 0.45V | PASS | LVC8T245 via R80 | U10-K35 |
| TR28_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.25 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R80 | U10-K35 |
| TR28_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.85 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R80 | U10-K35 |
| TR28_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.602 | ≥ 0.18V | PASS | LVC8T245 via R80 | U10-K35 |
| TR28_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.602 | ≥ 0.18V | PASS | LVC8T245 via R80 | U10-K35 |
| TR28_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.247V, Trough=-0.527V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R80 | U10-K35 |

## 3.8.22    TR29_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR29_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.670 | ≥ 1.35V (VDD−0.45V) | PASS | U10-J35 | LVC8T245 via R79 |
| TR29_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.130 | ≤ 0.45V | PASS | U10-J35 | LVC8T245 via R79 |
| TR29_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 11.28 | ≤ 10ns (JESD8-7A) | FAIL | U10-J35 | LVC8T245 via R79 |
| TR29_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 11.26 | ≤ 10ns (JESD8-7A) | FAIL | U10-J35 | LVC8T245 via R79 |
| TR29_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.402 | ≥ 0.18V | PASS | U10-J35 | LVC8T245 via R79 |
| TR29_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.447 | ≥ 0.18V | PASS | U10-J35 | LVC8T245 via R79 |
| TR29_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.670V, Trough=0.130V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-J35 | LVC8T245 via R79 |
| TR29_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.773 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R79 | U10-J35 |
| TR29_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.027 | ≤ 0.45V | PASS | LVC8T245 via R79 | U10-J35 |
| TR29_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.25 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R79 | U10-J35 |
| TR29_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.85 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R79 | U10-J35 |
| TR29_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.603 | ≥ 0.18V | PASS | LVC8T245 via R79 | U10-J35 |
| TR29_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.603 | ≥ 0.18V | PASS | LVC8T245 via R79 | U10-J35 |
| TR29_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.250V, Trough=-0.531V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R79 | U10-J35 |

## 3.8.23    TR2_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR2_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.741 | ≥ 1.35V (VDD−0.45V) | PASS | U10-F13 | LVC8T245 via R440 |
| TR2_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.059 | ≤ 0.45V | PASS | U10-F13 | LVC8T245 via R440 |
| TR2_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 4.65 | ≤ 10ns (JESD8-7A) | PASS | U10-F13 | LVC8T245 via R440 |
| TR2_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 4.64 | ≤ 10ns (JESD8-7A) | PASS | U10-F13 | LVC8T245 via R440 |
| TR2_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.473 | ≥ 0.18V | PASS | U10-F13 | LVC8T245 via R440 |
| TR2_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.518 | ≥ 0.18V | PASS | U10-F13 | LVC8T245 via R440 |
| TR2_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.741V, Trough=0.059V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-F13 | LVC8T245 via R440 |
| TR2_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.788 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R440 | U10-F13 |
| TR2_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.012 | ≤ 0.45V | PASS | LVC8T245 via R440 | U10-F13 |
| TR2_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.67 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R440 | U10-F13 |
| TR2_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 2.06 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R440 | U10-F13 |
| TR2_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.618 | ≥ 0.18V | PASS | LVC8T245 via R440 | U10-F13 |
| TR2_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.618 | ≥ 0.18V | PASS | LVC8T245 via R440 | U10-F13 |
| TR2_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.928V, Trough=-0.151V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R440 | U10-F13 |

## 3.8.24    TR30_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR30_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.669 | ≥ 1.35V (VDD−0.45V) | PASS | U10-H33 | LVC8T245 via R78 |
| TR30_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.131 | ≤ 0.45V | PASS | U10-H33 | LVC8T245 via R78 |
| TR30_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 11.71 | ≤ 10ns (JESD8-7A) | FAIL | U10-H33 | LVC8T245 via R78 |
| TR30_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 11.70 | ≤ 10ns (JESD8-7A) | FAIL | U10-H33 | LVC8T245 via R78 |
| TR30_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.401 | ≥ 0.18V | PASS | U10-H33 | LVC8T245 via R78 |
| TR30_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.446 | ≥ 0.18V | PASS | U10-H33 | LVC8T245 via R78 |
| TR30_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.669V, Trough=0.131V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-H33 | LVC8T245 via R78 |
| TR30_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.772 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R78 | U10-H33 |
| TR30_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.028 | ≤ 0.45V | PASS | LVC8T245 via R78 | U10-H33 |
| TR30_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.25 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R78 | U10-H33 |
| TR30_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.85 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R78 | U10-H33 |
| TR30_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.602 | ≥ 0.18V | PASS | LVC8T245 via R78 | U10-H33 |
| TR30_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.602 | ≥ 0.18V | PASS | LVC8T245 via R78 | U10-H33 |
| TR30_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.247V, Trough=-0.527V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R78 | U10-H33 |

## 3.8.25    TR31_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR31_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.670 | ≥ 1.35V (VDD−0.45V) | PASS | U10-H34 | LVC8T245 via R77 |
| TR31_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.130 | ≤ 0.45V | PASS | U10-H34 | LVC8T245 via R77 |
| TR31_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 11.28 | ≤ 10ns (JESD8-7A) | FAIL | U10-H34 | LVC8T245 via R77 |
| TR31_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 11.26 | ≤ 10ns (JESD8-7A) | FAIL | U10-H34 | LVC8T245 via R77 |
| TR31_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.402 | ≥ 0.18V | PASS | U10-H34 | LVC8T245 via R77 |
| TR31_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.447 | ≥ 0.18V | PASS | U10-H34 | LVC8T245 via R77 |
| TR31_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.670V, Trough=0.130V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-H34 | LVC8T245 via R77 |
| TR31_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.773 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R77 | U10-H34 |
| TR31_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.027 | ≤ 0.45V | PASS | LVC8T245 via R77 | U10-H34 |
| TR31_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.25 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R77 | U10-H34 |
| TR31_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.85 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R77 | U10-H34 |
| TR31_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.603 | ≥ 0.18V | PASS | LVC8T245 via R77 | U10-H34 |
| TR31_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.603 | ≥ 0.18V | PASS | LVC8T245 via R77 | U10-H34 |
| TR31_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.250V, Trough=-0.531V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R77 | U10-H34 |

## 3.8.26    TR32_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR32_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.614 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AW35 | LVC8T245 via R42 |
| TR32_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.186 | ≤ 0.45V | PASS | U10-AW35 | LVC8T245 via R42 |
| TR32_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 0.28 | ≤ 10ns (JESD8-7A) | PASS | U10-AW35 | LVC8T245 via R42 |
| TR32_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 0.24 | ≤ 10ns (JESD8-7A) | PASS | U10-AW35 | LVC8T245 via R42 |
| TR32_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.346 | ≥ 0.18V | PASS | U10-AW35 | LVC8T245 via R42 |
| TR32_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.391 | ≥ 0.18V | PASS | U10-AW35 | LVC8T245 via R42 |
| TR32_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.614V, Trough=0.186V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AW35 | LVC8T245 via R42 |
| TR32_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.759 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R42 | U10-AW35 |
| TR32_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.041 | ≤ 0.45V | PASS | LVC8T245 via R42 | U10-AW35 |
| TR32_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.28 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R42 | U10-AW35 |
| TR32_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.88 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R42 | U10-AW35 |
| TR32_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.589 | ≥ 0.18V | PASS | LVC8T245 via R42 | U10-AW35 |
| TR32_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.589 | ≥ 0.18V | PASS | LVC8T245 via R42 | U10-AW35 |
| TR32_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.302V, Trough=-0.502V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R42 | U10-AW35 |

## 3.8.27    TR33_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR33_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.614 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AW36 | LVC8T245 via R36 |
| TR33_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.186 | ≤ 0.45V | PASS | U10-AW36 | LVC8T245 via R36 |
| TR33_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 0.28 | ≤ 10ns (JESD8-7A) | PASS | U10-AW36 | LVC8T245 via R36 |
| TR33_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 0.24 | ≤ 10ns (JESD8-7A) | PASS | U10-AW36 | LVC8T245 via R36 |
| TR33_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.346 | ≥ 0.18V | PASS | U10-AW36 | LVC8T245 via R36 |
| TR33_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.391 | ≥ 0.18V | PASS | U10-AW36 | LVC8T245 via R36 |
| TR33_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.614V, Trough=0.186V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AW36 | LVC8T245 via R36 |
| TR33_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.759 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R36 | U10-AW36 |
| TR33_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.041 | ≤ 0.45V | PASS | LVC8T245 via R36 | U10-AW36 |
| TR33_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.28 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R36 | U10-AW36 |
| TR33_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.88 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R36 | U10-AW36 |
| TR33_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.589 | ≥ 0.18V | PASS | LVC8T245 via R36 | U10-AW36 |
| TR33_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.589 | ≥ 0.18V | PASS | LVC8T245 via R36 | U10-AW36 |
| TR33_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.302V, Trough=-0.502V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R36 | U10-AW36 |

## 3.8.28    TR34_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR34_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.609 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AV36 | LVC8T245 via R35 |
| TR34_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.191 | ≤ 0.45V | PASS | U10-AV36 | LVC8T245 via R35 |
| TR34_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 0.28 | ≤ 10ns (JESD8-7A) | PASS | U10-AV36 | LVC8T245 via R35 |
| TR34_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 0.24 | ≤ 10ns (JESD8-7A) | PASS | U10-AV36 | LVC8T245 via R35 |
| TR34_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.341 | ≥ 0.18V | PASS | U10-AV36 | LVC8T245 via R35 |
| TR34_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.386 | ≥ 0.18V | PASS | U10-AV36 | LVC8T245 via R35 |
| TR34_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.609V, Trough=0.191V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AV36 | LVC8T245 via R35 |
| TR34_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.758 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R35 | U10-AV36 |
| TR34_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.042 | ≤ 0.45V | PASS | LVC8T245 via R35 | U10-AV36 |
| TR34_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.29 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R35 | U10-AV36 |
| TR34_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.88 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R35 | U10-AV36 |
| TR34_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.588 | ≥ 0.18V | PASS | LVC8T245 via R35 | U10-AV36 |
| TR34_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.588 | ≥ 0.18V | PASS | LVC8T245 via R35 | U10-AV36 |
| TR34_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.297V, Trough=-0.497V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R35 | U10-AV36 |

## 3.8.29    TR35_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR35_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.609 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AV37 | LVC8T245 via R29 |
| TR35_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.191 | ≤ 0.45V | PASS | U10-AV37 | LVC8T245 via R29 |
| TR35_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 0.28 | ≤ 10ns (JESD8-7A) | PASS | U10-AV37 | LVC8T245 via R29 |
| TR35_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 0.24 | ≤ 10ns (JESD8-7A) | PASS | U10-AV37 | LVC8T245 via R29 |
| TR35_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.341 | ≥ 0.18V | PASS | U10-AV37 | LVC8T245 via R29 |
| TR35_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.386 | ≥ 0.18V | PASS | U10-AV37 | LVC8T245 via R29 |
| TR35_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.609V, Trough=0.191V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AV37 | LVC8T245 via R29 |
| TR35_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.758 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R29 | U10-AV37 |
| TR35_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.042 | ≤ 0.45V | PASS | LVC8T245 via R29 | U10-AV37 |
| TR35_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.29 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R29 | U10-AV37 |
| TR35_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.88 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R29 | U10-AV37 |
| TR35_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.588 | ≥ 0.18V | PASS | LVC8T245 via R29 | U10-AV37 |
| TR35_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.588 | ≥ 0.18V | PASS | LVC8T245 via R29 | U10-AV37 |
| TR35_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.297V, Trough=-0.497V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R29 | U10-AV37 |

## 3.8.30    TR36_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR36_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.605 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AU34 | LVC8T245 via R28 |
| TR36_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.195 | ≤ 0.45V | PASS | U10-AU34 | LVC8T245 via R28 |
| TR36_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 0.28 | ≤ 10ns (JESD8-7A) | PASS | U10-AU34 | LVC8T245 via R28 |
| TR36_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 0.24 | ≤ 10ns (JESD8-7A) | PASS | U10-AU34 | LVC8T245 via R28 |
| TR36_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.337 | ≥ 0.18V | PASS | U10-AU34 | LVC8T245 via R28 |
| TR36_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.382 | ≥ 0.18V | PASS | U10-AU34 | LVC8T245 via R28 |
| TR36_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.605V, Trough=0.195V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AU34 | LVC8T245 via R28 |
| TR36_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.757 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R28 | U10-AU34 |
| TR36_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.043 | ≤ 0.45V | PASS | LVC8T245 via R28 | U10-AU34 |
| TR36_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.29 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R28 | U10-AU34 |
| TR36_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.88 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R28 | U10-AU34 |
| TR36_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.587 | ≥ 0.18V | PASS | LVC8T245 via R28 | U10-AU34 |
| TR36_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.587 | ≥ 0.18V | PASS | LVC8T245 via R28 | U10-AU34 |
| TR36_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.295V, Trough=-0.495V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R28 | U10-AU34 |

## 3.8.31    TR37_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR37_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.609 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AU35 | LVC8T245 via R21 |
| TR37_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.191 | ≤ 0.45V | PASS | U10-AU35 | LVC8T245 via R21 |
| TR37_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 0.28 | ≤ 10ns (JESD8-7A) | PASS | U10-AU35 | LVC8T245 via R21 |
| TR37_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 0.24 | ≤ 10ns (JESD8-7A) | PASS | U10-AU35 | LVC8T245 via R21 |
| TR37_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.341 | ≥ 0.18V | PASS | U10-AU35 | LVC8T245 via R21 |
| TR37_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.386 | ≥ 0.18V | PASS | U10-AU35 | LVC8T245 via R21 |
| TR37_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.609V, Trough=0.191V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AU35 | LVC8T245 via R21 |
| TR37_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.758 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R21 | U10-AU35 |
| TR37_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.042 | ≤ 0.45V | PASS | LVC8T245 via R21 | U10-AU35 |
| TR37_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.29 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R21 | U10-AU35 |
| TR37_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.88 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R21 | U10-AU35 |
| TR37_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.588 | ≥ 0.18V | PASS | LVC8T245 via R21 | U10-AU35 |
| TR37_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.588 | ≥ 0.18V | PASS | LVC8T245 via R21 | U10-AU35 |
| TR37_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.297V, Trough=-0.497V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R21 | U10-AU35 |

## 3.8.32    TR38_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR38_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.609 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AV38 | LVC8T245 via R20 |
| TR38_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.191 | ≤ 0.45V | PASS | U10-AV38 | LVC8T245 via R20 |
| TR38_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 0.28 | ≤ 10ns (JESD8-7A) | PASS | U10-AV38 | LVC8T245 via R20 |
| TR38_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 0.24 | ≤ 10ns (JESD8-7A) | PASS | U10-AV38 | LVC8T245 via R20 |
| TR38_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.341 | ≥ 0.18V | PASS | U10-AV38 | LVC8T245 via R20 |
| TR38_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.386 | ≥ 0.18V | PASS | U10-AV38 | LVC8T245 via R20 |
| TR38_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.609V, Trough=0.191V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AV38 | LVC8T245 via R20 |
| TR38_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.758 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R20 | U10-AV38 |
| TR38_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.042 | ≤ 0.45V | PASS | LVC8T245 via R20 | U10-AV38 |
| TR38_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.29 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R20 | U10-AV38 |
| TR38_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.88 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R20 | U10-AV38 |
| TR38_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.588 | ≥ 0.18V | PASS | LVC8T245 via R20 | U10-AV38 |
| TR38_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.588 | ≥ 0.18V | PASS | LVC8T245 via R20 | U10-AV38 |
| TR38_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.297V, Trough=-0.497V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R20 | U10-AV38 |

## 3.8.33    TR39_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR39_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.609 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AW38 | LVC8T245 via R16 |
| TR39_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.191 | ≤ 0.45V | PASS | U10-AW38 | LVC8T245 via R16 |
| TR39_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 0.28 | ≤ 10ns (JESD8-7A) | PASS | U10-AW38 | LVC8T245 via R16 |
| TR39_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 0.24 | ≤ 10ns (JESD8-7A) | PASS | U10-AW38 | LVC8T245 via R16 |
| TR39_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.341 | ≥ 0.18V | PASS | U10-AW38 | LVC8T245 via R16 |
| TR39_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.386 | ≥ 0.18V | PASS | U10-AW38 | LVC8T245 via R16 |
| TR39_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.609V, Trough=0.191V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AW38 | LVC8T245 via R16 |
| TR39_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.758 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R16 | U10-AW38 |
| TR39_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.042 | ≤ 0.45V | PASS | LVC8T245 via R16 | U10-AW38 |
| TR39_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.29 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R16 | U10-AW38 |
| TR39_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.88 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R16 | U10-AW38 |
| TR39_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.588 | ≥ 0.18V | PASS | LVC8T245 via R16 | U10-AW38 |
| TR39_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.588 | ≥ 0.18V | PASS | LVC8T245 via R16 | U10-AW38 |
| TR39_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.297V, Trough=-0.497V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R16 | U10-AW38 |

## 3.8.34    TR3_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR3_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.741 | ≥ 1.35V (VDD−0.45V) | PASS | U10-E13 | LVC8T245 via R441 |
| TR3_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.059 | ≤ 0.45V | PASS | U10-E13 | LVC8T245 via R441 |
| TR3_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 4.66 | ≤ 10ns (JESD8-7A) | PASS | U10-E13 | LVC8T245 via R441 |
| TR3_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 4.65 | ≤ 10ns (JESD8-7A) | PASS | U10-E13 | LVC8T245 via R441 |
| TR3_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.473 | ≥ 0.18V | PASS | U10-E13 | LVC8T245 via R441 |
| TR3_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.518 | ≥ 0.18V | PASS | U10-E13 | LVC8T245 via R441 |
| TR3_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.741V, Trough=0.059V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-E13 | LVC8T245 via R441 |
| TR3_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.788 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R441 | U10-E13 |
| TR3_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.012 | ≤ 0.45V | PASS | LVC8T245 via R441 | U10-E13 |
| TR3_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.67 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R441 | U10-E13 |
| TR3_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 2.06 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R441 | U10-E13 |
| TR3_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.618 | ≥ 0.18V | PASS | LVC8T245 via R441 | U10-E13 |
| TR3_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.618 | ≥ 0.18V | PASS | LVC8T245 via R441 | U10-E13 |
| TR3_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.928V, Trough=-0.150V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R441 | U10-E13 |

## 3.8.35    TR40_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR40_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.723 | ≥ 1.35V (VDD−0.45V) | PASS | U10-BE38 | LVC8T245 via R251 |
| TR40_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.077 | ≤ 0.45V | PASS | U10-BE38 | LVC8T245 via R251 |
| TR40_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.04 | ≤ 10ns (JESD8-7A) | PASS | U10-BE38 | LVC8T245 via R251 |
| TR40_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.02 | ≤ 10ns (JESD8-7A) | PASS | U10-BE38 | LVC8T245 via R251 |
| TR40_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.456 | ≥ 0.18V | PASS | U10-BE38 | LVC8T245 via R251 |
| TR40_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.501 | ≥ 0.18V | PASS | U10-BE38 | LVC8T245 via R251 |
| TR40_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.723V, Trough=0.077V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-BE38 | LVC8T245 via R251 |
| TR40_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.784 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R251 | U10-BE38 |
| TR40_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.016 | ≤ 0.45V | PASS | LVC8T245 via R251 | U10-BE38 |
| TR40_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.47 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R251 | U10-BE38 |
| TR40_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.87 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R251 | U10-BE38 |
| TR40_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.614 | ≥ 0.18V | PASS | LVC8T245 via R251 | U10-BE38 |
| TR40_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.614 | ≥ 0.18V | PASS | LVC8T245 via R251 | U10-BE38 |
| TR40_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.976V, Trough=-0.261V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R251 | U10-BE38 |

## 3.8.36    TR41_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR41_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.723 | ≥ 1.35V (VDD−0.45V) | PASS | U10-BF38 | LVC8T245 via R250 |
| TR41_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.077 | ≤ 0.45V | PASS | U10-BF38 | LVC8T245 via R250 |
| TR41_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.04 | ≤ 10ns (JESD8-7A) | PASS | U10-BF38 | LVC8T245 via R250 |
| TR41_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.02 | ≤ 10ns (JESD8-7A) | PASS | U10-BF38 | LVC8T245 via R250 |
| TR41_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.456 | ≥ 0.18V | PASS | U10-BF38 | LVC8T245 via R250 |
| TR41_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.501 | ≥ 0.18V | PASS | U10-BF38 | LVC8T245 via R250 |
| TR41_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.723V, Trough=0.077V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-BF38 | LVC8T245 via R250 |
| TR41_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.784 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R250 | U10-BF38 |
| TR41_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.016 | ≤ 0.45V | PASS | LVC8T245 via R250 | U10-BF38 |
| TR41_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.47 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R250 | U10-BF38 |
| TR41_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.87 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R250 | U10-BF38 |
| TR41_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.614 | ≥ 0.18V | PASS | LVC8T245 via R250 | U10-BF38 |
| TR41_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.614 | ≥ 0.18V | PASS | LVC8T245 via R250 | U10-BF38 |
| TR41_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.976V, Trough=-0.261V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R250 | U10-BF38 |

## 3.8.37    TR42_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR42_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.728 | ≥ 1.35V (VDD−0.45V) | PASS | U10-BF42 | LVC8T245 via R249 |
| TR42_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.072 | ≤ 0.45V | PASS | U10-BF42 | LVC8T245 via R249 |
| TR42_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 5.80 | ≤ 10ns (JESD8-7A) | PASS | U10-BF42 | LVC8T245 via R249 |
| TR42_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 5.78 | ≤ 10ns (JESD8-7A) | PASS | U10-BF42 | LVC8T245 via R249 |
| TR42_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.460 | ≥ 0.18V | PASS | U10-BF42 | LVC8T245 via R249 |
| TR42_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.505 | ≥ 0.18V | PASS | U10-BF42 | LVC8T245 via R249 |
| TR42_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.728V, Trough=0.072V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-BF42 | LVC8T245 via R249 |
| TR42_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.785 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R249 | U10-BF42 |
| TR42_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.015 | ≤ 0.45V | PASS | LVC8T245 via R249 | U10-BF42 |
| TR42_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.52 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R249 | U10-BF42 |
| TR42_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.91 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R249 | U10-BF42 |
| TR42_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.615 | ≥ 0.18V | PASS | LVC8T245 via R249 | U10-BF42 |
| TR42_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.615 | ≥ 0.18V | PASS | LVC8T245 via R249 | U10-BF42 |
| TR42_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.954V, Trough=-0.234V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R249 | U10-BF42 |

## 3.8.38    TR43_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR43_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.728 | ≥ 1.35V (VDD−0.45V) | PASS | U10-BF43 | LVC8T245 via R248 |
| TR43_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.072 | ≤ 0.45V | PASS | U10-BF43 | LVC8T245 via R248 |
| TR43_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 5.69 | ≤ 10ns (JESD8-7A) | PASS | U10-BF43 | LVC8T245 via R248 |
| TR43_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 5.67 | ≤ 10ns (JESD8-7A) | PASS | U10-BF43 | LVC8T245 via R248 |
| TR43_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.460 | ≥ 0.18V | PASS | U10-BF43 | LVC8T245 via R248 |
| TR43_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.505 | ≥ 0.18V | PASS | U10-BF43 | LVC8T245 via R248 |
| TR43_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.728V, Trough=0.072V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-BF43 | LVC8T245 via R248 |
| TR43_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.785 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R248 | U10-BF43 |
| TR43_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.015 | ≤ 0.45V | PASS | LVC8T245 via R248 | U10-BF43 |
| TR43_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.52 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R248 | U10-BF43 |
| TR43_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.91 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R248 | U10-BF43 |
| TR43_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.615 | ≥ 0.18V | PASS | LVC8T245 via R248 | U10-BF43 |
| TR43_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.615 | ≥ 0.18V | PASS | LVC8T245 via R248 | U10-BF43 |
| TR43_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.954V, Trough=-0.234V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R248 | U10-BF43 |

## 3.8.39    TR44_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR44_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.728 | ≥ 1.35V (VDD−0.45V) | PASS | U10-BD39 | LVC8T245 via R247 |
| TR44_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.072 | ≤ 0.45V | PASS | U10-BD39 | LVC8T245 via R247 |
| TR44_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 5.79 | ≤ 10ns (JESD8-7A) | PASS | U10-BD39 | LVC8T245 via R247 |
| TR44_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 5.77 | ≤ 10ns (JESD8-7A) | PASS | U10-BD39 | LVC8T245 via R247 |
| TR44_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.461 | ≥ 0.18V | PASS | U10-BD39 | LVC8T245 via R247 |
| TR44_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.506 | ≥ 0.18V | PASS | U10-BD39 | LVC8T245 via R247 |
| TR44_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.728V, Trough=0.072V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-BD39 | LVC8T245 via R247 |
| TR44_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.785 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R247 | U10-BD39 |
| TR44_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.015 | ≤ 0.45V | PASS | LVC8T245 via R247 | U10-BD39 |
| TR44_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.46 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R247 | U10-BD39 |
| TR44_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.86 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R247 | U10-BD39 |
| TR44_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.615 | ≥ 0.18V | PASS | LVC8T245 via R247 | U10-BD39 |
| TR44_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.615 | ≥ 0.18V | PASS | LVC8T245 via R247 | U10-BD39 |
| TR44_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.978V, Trough=-0.263V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R247 | U10-BD39 |

## 3.8.40    TR45_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR45_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.728 | ≥ 1.35V (VDD−0.45V) | PASS | U10-BD40 | LVC8T245 via R246 |
| TR45_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.072 | ≤ 0.45V | PASS | U10-BD40 | LVC8T245 via R246 |
| TR45_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 5.80 | ≤ 10ns (JESD8-7A) | PASS | U10-BD40 | LVC8T245 via R246 |
| TR45_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 5.78 | ≤ 10ns (JESD8-7A) | PASS | U10-BD40 | LVC8T245 via R246 |
| TR45_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.460 | ≥ 0.18V | PASS | U10-BD40 | LVC8T245 via R246 |
| TR45_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.505 | ≥ 0.18V | PASS | U10-BD40 | LVC8T245 via R246 |
| TR45_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.728V, Trough=0.072V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-BD40 | LVC8T245 via R246 |
| TR45_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.785 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R246 | U10-BD40 |
| TR45_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.015 | ≤ 0.45V | PASS | LVC8T245 via R246 | U10-BD40 |
| TR45_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.52 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R246 | U10-BD40 |
| TR45_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.91 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R246 | U10-BD40 |
| TR45_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.615 | ≥ 0.18V | PASS | LVC8T245 via R246 | U10-BD40 |
| TR45_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.615 | ≥ 0.18V | PASS | LVC8T245 via R246 | U10-BD40 |
| TR45_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.954V, Trough=-0.234V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R246 | U10-BD40 |

## 3.8.41    TR46_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR46_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.728 | ≥ 1.35V (VDD−0.45V) | PASS | U10-BC37 | LVC8T245 via R245 |
| TR46_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.072 | ≤ 0.45V | PASS | U10-BC37 | LVC8T245 via R245 |
| TR46_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 5.91 | ≤ 10ns (JESD8-7A) | PASS | U10-BC37 | LVC8T245 via R245 |
| TR46_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 5.89 | ≤ 10ns (JESD8-7A) | PASS | U10-BC37 | LVC8T245 via R245 |
| TR46_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.460 | ≥ 0.18V | PASS | U10-BC37 | LVC8T245 via R245 |
| TR46_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.505 | ≥ 0.18V | PASS | U10-BC37 | LVC8T245 via R245 |
| TR46_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.728V, Trough=0.072V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-BC37 | LVC8T245 via R245 |
| TR46_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.785 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R245 | U10-BC37 |
| TR46_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.015 | ≤ 0.45V | PASS | LVC8T245 via R245 | U10-BC37 |
| TR46_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.46 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R245 | U10-BC37 |
| TR46_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.87 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R245 | U10-BC37 |
| TR46_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.615 | ≥ 0.18V | PASS | LVC8T245 via R245 | U10-BC37 |
| TR46_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.615 | ≥ 0.18V | PASS | LVC8T245 via R245 | U10-BC37 |
| TR46_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.977V, Trough=-0.262V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R245 | U10-BC37 |

## 3.8.42    TR47_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR47_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.728 | ≥ 1.35V (VDD−0.45V) | PASS | U10-BC38 | LVC8T245 via R244 |
| TR47_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.072 | ≤ 0.45V | PASS | U10-BC38 | LVC8T245 via R244 |
| TR47_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 5.79 | ≤ 10ns (JESD8-7A) | PASS | U10-BC38 | LVC8T245 via R244 |
| TR47_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 5.77 | ≤ 10ns (JESD8-7A) | PASS | U10-BC38 | LVC8T245 via R244 |
| TR47_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.461 | ≥ 0.18V | PASS | U10-BC38 | LVC8T245 via R244 |
| TR47_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.506 | ≥ 0.18V | PASS | U10-BC38 | LVC8T245 via R244 |
| TR47_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.728V, Trough=0.072V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-BC38 | LVC8T245 via R244 |
| TR47_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.785 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R244 | U10-BC38 |
| TR47_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.015 | ≤ 0.45V | PASS | LVC8T245 via R244 | U10-BC38 |
| TR47_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.52 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R244 | U10-BC38 |
| TR47_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.91 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R244 | U10-BC38 |
| TR47_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.615 | ≥ 0.18V | PASS | LVC8T245 via R244 | U10-BC38 |
| TR47_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.615 | ≥ 0.18V | PASS | LVC8T245 via R244 | U10-BC38 |
| TR47_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.954V, Trough=-0.234V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R244 | U10-BC38 |

## 3.8.43    TR48_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR48_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.718 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AY11 | LVC8T245 via R489 |
| TR48_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.082 | ≤ 0.45V | PASS | U10-AY11 | LVC8T245 via R489 |
| TR48_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.53 | ≤ 10ns (JESD8-7A) | PASS | U10-AY11 | LVC8T245 via R489 |
| TR48_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.51 | ≤ 10ns (JESD8-7A) | PASS | U10-AY11 | LVC8T245 via R489 |
| TR48_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.450 | ≥ 0.18V | PASS | U10-AY11 | LVC8T245 via R489 |
| TR48_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.495 | ≥ 0.18V | PASS | U10-AY11 | LVC8T245 via R489 |
| TR48_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.718V, Trough=0.082V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AY11 | LVC8T245 via R489 |
| TR48_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.783 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R489 | U10-AY11 |
| TR48_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.017 | ≤ 0.45V | PASS | LVC8T245 via R489 | U10-AY11 |
| TR48_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.42 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R489 | U10-AY11 |
| TR48_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.84 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R489 | U10-AY11 |
| TR48_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.613 | ≥ 0.18V | PASS | LVC8T245 via R489 | U10-AY11 |
| TR48_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.613 | ≥ 0.18V | PASS | LVC8T245 via R489 | U10-AY11 |
| TR48_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.996V, Trough=-0.285V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R489 | U10-AY11 |

## 3.8.44    TR49_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR49_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.718 | ≥ 1.35V (VDD−0.45V) | PASS | U10-BA11 | LVC8T245 via R484 |
| TR49_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.082 | ≤ 0.45V | PASS | U10-BA11 | LVC8T245 via R484 |
| TR49_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.53 | ≤ 10ns (JESD8-7A) | PASS | U10-BA11 | LVC8T245 via R484 |
| TR49_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.51 | ≤ 10ns (JESD8-7A) | PASS | U10-BA11 | LVC8T245 via R484 |
| TR49_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.450 | ≥ 0.18V | PASS | U10-BA11 | LVC8T245 via R484 |
| TR49_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.495 | ≥ 0.18V | PASS | U10-BA11 | LVC8T245 via R484 |
| TR49_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.718V, Trough=0.082V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-BA11 | LVC8T245 via R484 |
| TR49_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.783 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R484 | U10-BA11 |
| TR49_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.017 | ≤ 0.45V | PASS | LVC8T245 via R484 | U10-BA11 |
| TR49_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.42 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R484 | U10-BA11 |
| TR49_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.84 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R484 | U10-BA11 |
| TR49_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.613 | ≥ 0.18V | PASS | LVC8T245 via R484 | U10-BA11 |
| TR49_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.613 | ≥ 0.18V | PASS | LVC8T245 via R484 | U10-BA11 |
| TR49_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.996V, Trough=-0.285V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R484 | U10-BA11 |

## 3.8.45    TR4_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR4_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.736 | ≥ 1.35V (VDD−0.45V) | PASS | U10-G14 | LVC8T245 via R449 |
| TR4_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.064 | ≤ 0.45V | PASS | U10-G14 | LVC8T245 via R449 |
| TR4_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 4.79 | ≤ 10ns (JESD8-7A) | PASS | U10-G14 | LVC8T245 via R449 |
| TR4_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 4.77 | ≤ 10ns (JESD8-7A) | PASS | U10-G14 | LVC8T245 via R449 |
| TR4_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.469 | ≥ 0.18V | PASS | U10-G14 | LVC8T245 via R449 |
| TR4_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.514 | ≥ 0.18V | PASS | U10-G14 | LVC8T245 via R449 |
| TR4_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.736V, Trough=0.064V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-G14 | LVC8T245 via R449 |
| TR4_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.787 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R449 | U10-G14 |
| TR4_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.013 | ≤ 0.45V | PASS | LVC8T245 via R449 | U10-G14 |
| TR4_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.68 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R449 | U10-G14 |
| TR4_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 2.06 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R449 | U10-G14 |
| TR4_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.617 | ≥ 0.18V | PASS | LVC8T245 via R449 | U10-G14 |
| TR4_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.617 | ≥ 0.18V | PASS | LVC8T245 via R449 | U10-G14 |
| TR4_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.927V, Trough=-0.149V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R449 | U10-G14 |

## 3.8.46    TR50_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR50_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.722 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AU14 | LVC8T245 via R483 |
| TR50_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.078 | ≤ 0.45V | PASS | U10-AU14 | LVC8T245 via R483 |
| TR50_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.06 | ≤ 10ns (JESD8-7A) | PASS | U10-AU14 | LVC8T245 via R483 |
| TR50_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.04 | ≤ 10ns (JESD8-7A) | PASS | U10-AU14 | LVC8T245 via R483 |
| TR50_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.455 | ≥ 0.18V | PASS | U10-AU14 | LVC8T245 via R483 |
| TR50_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.500 | ≥ 0.18V | PASS | U10-AU14 | LVC8T245 via R483 |
| TR50_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.722V, Trough=0.078V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AU14 | LVC8T245 via R483 |
| TR50_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.784 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R483 | U10-AU14 |
| TR50_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.016 | ≤ 0.45V | PASS | LVC8T245 via R483 | U10-AU14 |
| TR50_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.47 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R483 | U10-AU14 |
| TR50_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.87 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R483 | U10-AU14 |
| TR50_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.614 | ≥ 0.18V | PASS | LVC8T245 via R483 | U10-AU14 |
| TR50_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.614 | ≥ 0.18V | PASS | LVC8T245 via R483 | U10-AU14 |
| TR50_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.975V, Trough=-0.259V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R483 | U10-AU14 |

## 3.8.47    TR51_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR51_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.718 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AV14 | LVC8T245 via R477 |
| TR51_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.082 | ≤ 0.45V | PASS | U10-AV14 | LVC8T245 via R477 |
| TR51_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.31 | ≤ 10ns (JESD8-7A) | PASS | U10-AV14 | LVC8T245 via R477 |
| TR51_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.29 | ≤ 10ns (JESD8-7A) | PASS | U10-AV14 | LVC8T245 via R477 |
| TR51_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.450 | ≥ 0.18V | PASS | U10-AV14 | LVC8T245 via R477 |
| TR51_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.495 | ≥ 0.18V | PASS | U10-AV14 | LVC8T245 via R477 |
| TR51_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.718V, Trough=0.082V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AV14 | LVC8T245 via R477 |
| TR51_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.783 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R477 | U10-AV14 |
| TR51_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.017 | ≤ 0.45V | PASS | LVC8T245 via R477 | U10-AV14 |
| TR51_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.42 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R477 | U10-AV14 |
| TR51_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.84 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R477 | U10-AV14 |
| TR51_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.613 | ≥ 0.18V | PASS | LVC8T245 via R477 | U10-AV14 |
| TR51_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.613 | ≥ 0.18V | PASS | LVC8T245 via R477 | U10-AV14 |
| TR51_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.996V, Trough=-0.285V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R477 | U10-AV14 |

## 3.8.48    TR52_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR52_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.722 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AW15 | LVC8T245 via R476 |
| TR52_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.078 | ≤ 0.45V | PASS | U10-AW15 | LVC8T245 via R476 |
| TR52_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.06 | ≤ 10ns (JESD8-7A) | PASS | U10-AW15 | LVC8T245 via R476 |
| TR52_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.04 | ≤ 10ns (JESD8-7A) | PASS | U10-AW15 | LVC8T245 via R476 |
| TR52_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.455 | ≥ 0.18V | PASS | U10-AW15 | LVC8T245 via R476 |
| TR52_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.500 | ≥ 0.18V | PASS | U10-AW15 | LVC8T245 via R476 |
| TR52_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.722V, Trough=0.078V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AW15 | LVC8T245 via R476 |
| TR52_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.784 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R476 | U10-AW15 |
| TR52_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.016 | ≤ 0.45V | PASS | LVC8T245 via R476 | U10-AW15 |
| TR52_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.47 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R476 | U10-AW15 |
| TR52_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.87 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R476 | U10-AW15 |
| TR52_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.614 | ≥ 0.18V | PASS | LVC8T245 via R476 | U10-AW15 |
| TR52_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.614 | ≥ 0.18V | PASS | LVC8T245 via R476 | U10-AW15 |
| TR52_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.975V, Trough=-0.259V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R476 | U10-AW15 |

## 3.8.49    TR53_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR53_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.722 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AY15 | LVC8T245 via R470 |
| TR53_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.078 | ≤ 0.45V | PASS | U10-AY15 | LVC8T245 via R470 |
| TR53_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.06 | ≤ 10ns (JESD8-7A) | PASS | U10-AY15 | LVC8T245 via R470 |
| TR53_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.04 | ≤ 10ns (JESD8-7A) | PASS | U10-AY15 | LVC8T245 via R470 |
| TR53_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.455 | ≥ 0.18V | PASS | U10-AY15 | LVC8T245 via R470 |
| TR53_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.500 | ≥ 0.18V | PASS | U10-AY15 | LVC8T245 via R470 |
| TR53_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.722V, Trough=0.078V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AY15 | LVC8T245 via R470 |
| TR53_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.784 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R470 | U10-AY15 |
| TR53_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.016 | ≤ 0.45V | PASS | LVC8T245 via R470 | U10-AY15 |
| TR53_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.47 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R470 | U10-AY15 |
| TR53_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.87 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R470 | U10-AY15 |
| TR53_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.614 | ≥ 0.18V | PASS | LVC8T245 via R470 | U10-AY15 |
| TR53_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.614 | ≥ 0.18V | PASS | LVC8T245 via R470 | U10-AY15 |
| TR53_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.975V, Trough=-0.259V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R470 | U10-AY15 |

## 3.8.50    TR54_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR54_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.723 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AU13 | LVC8T245 via R469 |
| TR54_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.077 | ≤ 0.45V | PASS | U10-AU13 | LVC8T245 via R469 |
| TR54_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.17 | ≤ 10ns (JESD8-7A) | PASS | U10-AU13 | LVC8T245 via R469 |
| TR54_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.14 | ≤ 10ns (JESD8-7A) | PASS | U10-AU13 | LVC8T245 via R469 |
| TR54_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.455 | ≥ 0.18V | PASS | U10-AU13 | LVC8T245 via R469 |
| TR54_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.500 | ≥ 0.18V | PASS | U10-AU13 | LVC8T245 via R469 |
| TR54_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.723V, Trough=0.077V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AU13 | LVC8T245 via R469 |
| TR54_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.784 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R469 | U10-AU13 |
| TR54_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.016 | ≤ 0.45V | PASS | LVC8T245 via R469 | U10-AU13 |
| TR54_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.47 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R469 | U10-AU13 |
| TR54_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.87 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R469 | U10-AU13 |
| TR54_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.614 | ≥ 0.18V | PASS | LVC8T245 via R469 | U10-AU13 |
| TR54_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.614 | ≥ 0.18V | PASS | LVC8T245 via R469 | U10-AU13 |
| TR54_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.975V, Trough=-0.259V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R469 | U10-AU13 |

## 3.8.51    TR55_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR55_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.723 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AV13 | LVC8T245 via R457 |
| TR55_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.077 | ≤ 0.45V | PASS | U10-AV13 | LVC8T245 via R457 |
| TR55_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.17 | ≤ 10ns (JESD8-7A) | PASS | U10-AV13 | LVC8T245 via R457 |
| TR55_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.14 | ≤ 10ns (JESD8-7A) | PASS | U10-AV13 | LVC8T245 via R457 |
| TR55_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.455 | ≥ 0.18V | PASS | U10-AV13 | LVC8T245 via R457 |
| TR55_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.500 | ≥ 0.18V | PASS | U10-AV13 | LVC8T245 via R457 |
| TR55_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.723V, Trough=0.077V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AV13 | LVC8T245 via R457 |
| TR55_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.784 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R457 | U10-AV13 |
| TR55_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.016 | ≤ 0.45V | PASS | LVC8T245 via R457 | U10-AV13 |
| TR55_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.47 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R457 | U10-AV13 |
| TR55_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.87 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R457 | U10-AV13 |
| TR55_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.614 | ≥ 0.18V | PASS | LVC8T245 via R457 | U10-AV13 |
| TR55_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.614 | ≥ 0.18V | PASS | LVC8T245 via R457 | U10-AV13 |
| TR55_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.975V, Trough=-0.259V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R457 | U10-AV13 |

## 3.8.52    TR56_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR56_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.782 | ≥ 1.35V (VDD−0.45V) | PASS | U10-BF10 | LVC8T245 via R383 |
| TR56_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.018 | ≤ 0.45V | PASS | U10-BF10 | LVC8T245 via R383 |
| TR56_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 1.59 | ≤ 10ns (JESD8-7A) | PASS | U10-BF10 | LVC8T245 via R383 |
| TR56_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 1.57 | ≤ 10ns (JESD8-7A) | PASS | U10-BF10 | LVC8T245 via R383 |
| TR56_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.514 | ≥ 0.18V | PASS | U10-BF10 | LVC8T245 via R383 |
| TR56_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.559 | ≥ 0.18V | PASS | U10-BF10 | LVC8T245 via R383 |
| TR56_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.782V, Trough=0.018V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-BF10 | LVC8T245 via R383 |
| TR56_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.796 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R383 | U10-BF10 |
| TR56_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.004 | ≤ 0.45V | PASS | LVC8T245 via R383 | U10-BF10 |
| TR56_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.92 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R383 | U10-BF10 |
| TR56_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 2.38 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R383 | U10-BF10 |
| TR56_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.626 | ≥ 0.18V | PASS | LVC8T245 via R383 | U10-BF10 |
| TR56_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.626 | ≥ 0.18V | PASS | LVC8T245 via R383 | U10-BF10 |
| TR56_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.807V, Trough=-0.009V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R383 | U10-BF10 |

## 3.8.53    TR57_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR57_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.782 | ≥ 1.35V (VDD−0.45V) | PASS | U10-BF9 | LVC8T245 via R384 |
| TR57_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.018 | ≤ 0.45V | PASS | U10-BF9 | LVC8T245 via R384 |
| TR57_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 1.56 | ≤ 10ns (JESD8-7A) | PASS | U10-BF9 | LVC8T245 via R384 |
| TR57_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 1.53 | ≤ 10ns (JESD8-7A) | PASS | U10-BF9 | LVC8T245 via R384 |
| TR57_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.515 | ≥ 0.18V | PASS | U10-BF9 | LVC8T245 via R384 |
| TR57_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.560 | ≥ 0.18V | PASS | U10-BF9 | LVC8T245 via R384 |
| TR57_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.782V, Trough=0.018V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-BF9 | LVC8T245 via R384 |
| TR57_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.797 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R384 | U10-BF9 |
| TR57_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.003 | ≤ 0.45V | PASS | LVC8T245 via R384 | U10-BF9 |
| TR57_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.90 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R384 | U10-BF9 |
| TR57_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 2.36 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R384 | U10-BF9 |
| TR57_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.627 | ≥ 0.18V | PASS | LVC8T245 via R384 | U10-BF9 |
| TR57_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.627 | ≥ 0.18V | PASS | LVC8T245 via R384 | U10-BF9 |
| TR57_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.814V, Trough=-0.018V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R384 | U10-BF9 |

## 3.8.54    TR58_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR58_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.783 | ≥ 1.35V (VDD−0.45V) | PASS | U10-BC7 | LVC8T245 via R385 |
| TR58_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.017 | ≤ 0.45V | PASS | U10-BC7 | LVC8T245 via R385 |
| TR58_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 1.51 | ≤ 10ns (JESD8-7A) | PASS | U10-BC7 | LVC8T245 via R385 |
| TR58_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 1.49 | ≤ 10ns (JESD8-7A) | PASS | U10-BC7 | LVC8T245 via R385 |
| TR58_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.515 | ≥ 0.18V | PASS | U10-BC7 | LVC8T245 via R385 |
| TR58_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.560 | ≥ 0.18V | PASS | U10-BC7 | LVC8T245 via R385 |
| TR58_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.783V, Trough=0.017V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-BC7 | LVC8T245 via R385 |
| TR58_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.797 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R385 | U10-BC7 |
| TR58_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.003 | ≤ 0.45V | PASS | LVC8T245 via R385 | U10-BC7 |
| TR58_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.90 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R385 | U10-BC7 |
| TR58_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 2.36 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R385 | U10-BC7 |
| TR58_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.627 | ≥ 0.18V | PASS | LVC8T245 via R385 | U10-BC7 |
| TR58_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.627 | ≥ 0.18V | PASS | LVC8T245 via R385 | U10-BC7 |
| TR58_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.812V, Trough=-0.016V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R385 | U10-BC7 |

## 3.8.55    TR59_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR59_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.783 | ≥ 1.35V (VDD−0.45V) | PASS | U10-BD7 | LVC8T245 via R386 |
| TR59_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.017 | ≤ 0.45V | PASS | U10-BD7 | LVC8T245 via R386 |
| TR59_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 1.50 | ≤ 10ns (JESD8-7A) | PASS | U10-BD7 | LVC8T245 via R386 |
| TR59_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 1.48 | ≤ 10ns (JESD8-7A) | PASS | U10-BD7 | LVC8T245 via R386 |
| TR59_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.516 | ≥ 0.18V | PASS | U10-BD7 | LVC8T245 via R386 |
| TR59_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.561 | ≥ 0.18V | PASS | U10-BD7 | LVC8T245 via R386 |
| TR59_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.783V, Trough=0.017V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-BD7 | LVC8T245 via R386 |
| TR59_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.797 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R386 | U10-BD7 |
| TR59_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.003 | ≤ 0.45V | PASS | LVC8T245 via R386 | U10-BD7 |
| TR59_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.90 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R386 | U10-BD7 |
| TR59_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 2.36 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R386 | U10-BD7 |
| TR59_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.627 | ≥ 0.18V | PASS | LVC8T245 via R386 | U10-BD7 |
| TR59_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.627 | ≥ 0.18V | PASS | LVC8T245 via R386 | U10-BD7 |
| TR59_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.813V, Trough=-0.016V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R386 | U10-BD7 |

## 3.8.56    TR5_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR5_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.741 | ≥ 1.35V (VDD−0.45V) | PASS | U10-G13 | LVC8T245 via R450 |
| TR5_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.059 | ≤ 0.45V | PASS | U10-G13 | LVC8T245 via R450 |
| TR5_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 4.65 | ≤ 10ns (JESD8-7A) | PASS | U10-G13 | LVC8T245 via R450 |
| TR5_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 4.64 | ≤ 10ns (JESD8-7A) | PASS | U10-G13 | LVC8T245 via R450 |
| TR5_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.473 | ≥ 0.18V | PASS | U10-G13 | LVC8T245 via R450 |
| TR5_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.518 | ≥ 0.18V | PASS | U10-G13 | LVC8T245 via R450 |
| TR5_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.741V, Trough=0.059V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-G13 | LVC8T245 via R450 |
| TR5_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.788 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R450 | U10-G13 |
| TR5_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.012 | ≤ 0.45V | PASS | LVC8T245 via R450 | U10-G13 |
| TR5_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.67 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R450 | U10-G13 |
| TR5_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 2.06 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R450 | U10-G13 |
| TR5_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.618 | ≥ 0.18V | PASS | LVC8T245 via R450 | U10-G13 |
| TR5_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.618 | ≥ 0.18V | PASS | LVC8T245 via R450 | U10-G13 |
| TR5_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.928V, Trough=-0.151V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R450 | U10-G13 |

## 3.8.57    TR60_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR60_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.782 | ≥ 1.35V (VDD−0.45V) | PASS | U10-BD9 | LVC8T245 via R387 |
| TR60_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.018 | ≤ 0.45V | PASS | U10-BD9 | LVC8T245 via R387 |
| TR60_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 1.50 | ≤ 10ns (JESD8-7A) | PASS | U10-BD9 | LVC8T245 via R387 |
| TR60_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 1.48 | ≤ 10ns (JESD8-7A) | PASS | U10-BD9 | LVC8T245 via R387 |
| TR60_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.515 | ≥ 0.18V | PASS | U10-BD9 | LVC8T245 via R387 |
| TR60_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.560 | ≥ 0.18V | PASS | U10-BD9 | LVC8T245 via R387 |
| TR60_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.782V, Trough=0.018V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-BD9 | LVC8T245 via R387 |
| TR60_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.796 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R387 | U10-BD9 |
| TR60_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.004 | ≤ 0.45V | PASS | LVC8T245 via R387 | U10-BD9 |
| TR60_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.91 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R387 | U10-BD9 |
| TR60_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 2.37 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R387 | U10-BD9 |
| TR60_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.626 | ≥ 0.18V | PASS | LVC8T245 via R387 | U10-BD9 |
| TR60_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.626 | ≥ 0.18V | PASS | LVC8T245 via R387 | U10-BD9 |
| TR60_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.812V, Trough=-0.015V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R387 | U10-BD9 |

## 3.8.58    TR61_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR61_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.783 | ≥ 1.35V (VDD−0.45V) | PASS | U10-BD8 | LVC8T245 via R388 |
| TR61_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.017 | ≤ 0.45V | PASS | U10-BD8 | LVC8T245 via R388 |
| TR61_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 1.50 | ≤ 10ns (JESD8-7A) | PASS | U10-BD8 | LVC8T245 via R388 |
| TR61_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 1.48 | ≤ 10ns (JESD8-7A) | PASS | U10-BD8 | LVC8T245 via R388 |
| TR61_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.515 | ≥ 0.18V | PASS | U10-BD8 | LVC8T245 via R388 |
| TR61_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.560 | ≥ 0.18V | PASS | U10-BD8 | LVC8T245 via R388 |
| TR61_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.783V, Trough=0.017V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-BD8 | LVC8T245 via R388 |
| TR61_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.797 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R388 | U10-BD8 |
| TR61_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.003 | ≤ 0.45V | PASS | LVC8T245 via R388 | U10-BD8 |
| TR61_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.90 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R388 | U10-BD8 |
| TR61_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 2.36 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R388 | U10-BD8 |
| TR61_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.627 | ≥ 0.18V | PASS | LVC8T245 via R388 | U10-BD8 |
| TR61_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.627 | ≥ 0.18V | PASS | LVC8T245 via R388 | U10-BD8 |
| TR61_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.812V, Trough=-0.016V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R388 | U10-BD8 |

## 3.8.59    TR62_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR62_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.778 | ≥ 1.35V (VDD−0.45V) | PASS | U10-BA10 | LVC8T245 via R389 |
| TR62_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.022 | ≤ 0.45V | PASS | U10-BA10 | LVC8T245 via R389 |
| TR62_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 1.76 | ≤ 10ns (JESD8-7A) | PASS | U10-BA10 | LVC8T245 via R389 |
| TR62_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 1.73 | ≤ 10ns (JESD8-7A) | PASS | U10-BA10 | LVC8T245 via R389 |
| TR62_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.511 | ≥ 0.18V | PASS | U10-BA10 | LVC8T245 via R389 |
| TR62_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.556 | ≥ 0.18V | PASS | U10-BA10 | LVC8T245 via R389 |
| TR62_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.778V, Trough=0.022V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-BA10 | LVC8T245 via R389 |
| TR62_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.796 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R389 | U10-BA10 |
| TR62_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.004 | ≤ 0.45V | PASS | LVC8T245 via R389 | U10-BA10 |
| TR62_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.88 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R389 | U10-BA10 |
| TR62_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 2.34 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R389 | U10-BA10 |
| TR62_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.626 | ≥ 0.18V | PASS | LVC8T245 via R389 | U10-BA10 |
| TR62_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.626 | ≥ 0.18V | PASS | LVC8T245 via R389 | U10-BA10 |
| TR62_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.825V, Trough=-0.032V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R389 | U10-BA10 |

## 3.8.60    TR63_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR63_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.776 | ≥ 1.35V (VDD−0.45V) | PASS | U10-BA9 | LVC8T245 via R390 |
| TR63_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.024 | ≤ 0.45V | PASS | U10-BA9 | LVC8T245 via R390 |
| TR63_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 1.96 | ≤ 10ns (JESD8-7A) | PASS | U10-BA9 | LVC8T245 via R390 |
| TR63_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 1.94 | ≤ 10ns (JESD8-7A) | PASS | U10-BA9 | LVC8T245 via R390 |
| TR63_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.508 | ≥ 0.18V | PASS | U10-BA9 | LVC8T245 via R390 |
| TR63_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.553 | ≥ 0.18V | PASS | U10-BA9 | LVC8T245 via R390 |
| TR63_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.776V, Trough=0.024V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-BA9 | LVC8T245 via R390 |
| TR63_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.795 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R390 | U10-BA9 |
| TR63_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.005 | ≤ 0.45V | PASS | LVC8T245 via R390 | U10-BA9 |
| TR63_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.86 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R390 | U10-BA9 |
| TR63_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 2.32 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R390 | U10-BA9 |
| TR63_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.625 | ≥ 0.18V | PASS | LVC8T245 via R390 | U10-BA9 |
| TR63_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.625 | ≥ 0.18V | PASS | LVC8T245 via R390 | U10-BA9 |
| TR63_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.836V, Trough=-0.045V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R390 | U10-BA9 |

## 3.8.61    TR6_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR6_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.731 | ≥ 1.35V (VDD−0.45V) | PASS | U10-E16 | LVC8T245 via R462 |
| TR6_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.069 | ≤ 0.45V | PASS | U10-E16 | LVC8T245 via R462 |
| TR6_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 5.25 | ≤ 10ns (JESD8-7A) | PASS | U10-E16 | LVC8T245 via R462 |
| TR6_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 5.23 | ≤ 10ns (JESD8-7A) | PASS | U10-E16 | LVC8T245 via R462 |
| TR6_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.464 | ≥ 0.18V | PASS | U10-E16 | LVC8T245 via R462 |
| TR6_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.509 | ≥ 0.18V | PASS | U10-E16 | LVC8T245 via R462 |
| TR6_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.731V, Trough=0.069V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-E16 | LVC8T245 via R462 |
| TR6_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.786 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R462 | U10-E16 |
| TR6_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.014 | ≤ 0.45V | PASS | LVC8T245 via R462 | U10-E16 |
| TR6_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.57 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R462 | U10-E16 |
| TR6_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.96 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R462 | U10-E16 |
| TR6_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.616 | ≥ 0.18V | PASS | LVC8T245 via R462 | U10-E16 |
| TR6_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.616 | ≥ 0.18V | PASS | LVC8T245 via R462 | U10-E16 |
| TR6_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.950V, Trough=-0.205V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R462 | U10-E16 |

## 3.8.62    TR7_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR7_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.736 | ≥ 1.35V (VDD−0.45V) | PASS | U10-E15 | LVC8T245 via R463 |
| TR7_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.064 | ≤ 0.45V | PASS | U10-E15 | LVC8T245 via R463 |
| TR7_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 5.01 | ≤ 10ns (JESD8-7A) | PASS | U10-E15 | LVC8T245 via R463 |
| TR7_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 4.99 | ≤ 10ns (JESD8-7A) | PASS | U10-E15 | LVC8T245 via R463 |
| TR7_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.469 | ≥ 0.18V | PASS | U10-E15 | LVC8T245 via R463 |
| TR7_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.514 | ≥ 0.18V | PASS | U10-E15 | LVC8T245 via R463 |
| TR7_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.736V, Trough=0.064V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-E15 | LVC8T245 via R463 |
| TR7_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.787 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R463 | U10-E15 |
| TR7_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.013 | ≤ 0.45V | PASS | LVC8T245 via R463 | U10-E15 |
| TR7_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.62 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R463 | U10-E15 |
| TR7_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 2.01 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R463 | U10-E15 |
| TR7_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.617 | ≥ 0.18V | PASS | LVC8T245 via R463 | U10-E15 |
| TR7_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.617 | ≥ 0.18V | PASS | LVC8T245 via R463 | U10-E15 |
| TR7_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.935V, Trough=-0.178V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | LVC8T245 via R463 | U10-E15 |

## 3.8.63    TR8_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR8_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.713 | ≥ 1.35V (VDD−0.45V) | PASS | U10-R16 | LVC8T245 via R269 |
| TR8_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.087 | ≤ 0.45V | PASS | U10-R16 | LVC8T245 via R269 |
| TR8_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.77 | ≤ 10ns (JESD8-7A) | PASS | U10-R16 | LVC8T245 via R269 |
| TR8_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.75 | ≤ 10ns (JESD8-7A) | PASS | U10-R16 | LVC8T245 via R269 |
| TR8_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.446 | ≥ 0.18V | PASS | U10-R16 | LVC8T245 via R269 |
| TR8_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.491 | ≥ 0.18V | PASS | U10-R16 | LVC8T245 via R269 |
| TR8_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.713V, Trough=0.087V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-R16 | LVC8T245 via R269 |
| TR8_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.782 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R269 | U10-R16 |
| TR8_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.018 | ≤ 0.45V | PASS | LVC8T245 via R269 | U10-R16 |
| TR8_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.37 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R269 | U10-R16 |
| TR8_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.83 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R269 | U10-R16 |
| TR8_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.612 | ≥ 0.18V | PASS | LVC8T245 via R269 | U10-R16 |
| TR8_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.612 | ≥ 0.18V | PASS | LVC8T245 via R269 | U10-R16 |
| TR8_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.019V, Trough=-0.312V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R269 | U10-R16 |

## 3.8.64    TR9_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TR9_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.713 | ≥ 1.35V (VDD−0.45V) | PASS | U10-P15 | LVC8T245 via R268 |
| TR9_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.087 | ≤ 0.45V | PASS | U10-P15 | LVC8T245 via R268 |
| TR9_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 6.88 | ≤ 10ns (JESD8-7A) | PASS | U10-P15 | LVC8T245 via R268 |
| TR9_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 6.86 | ≤ 10ns (JESD8-7A) | PASS | U10-P15 | LVC8T245 via R268 |
| TR9_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.446 | ≥ 0.18V | PASS | U10-P15 | LVC8T245 via R268 |
| TR9_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.491 | ≥ 0.18V | PASS | U10-P15 | LVC8T245 via R268 |
| TR9_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.713V, Trough=0.087V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-P15 | LVC8T245 via R268 |
| TR9_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.782 | ≥ 1.35V (VDD−0.45V) | PASS | LVC8T245 via R268 | U10-P15 |
| TR9_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.018 | ≤ 0.45V | PASS | LVC8T245 via R268 | U10-P15 |
| TR9_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 2.37 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R268 | U10-P15 |
| TR9_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 1.83 | ≤ 10ns (JESD8-7A) | PASS | LVC8T245 via R268 | U10-P15 |
| TR9_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.612 | ≥ 0.18V | PASS | LVC8T245 via R268 | U10-P15 |
| TR9_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.612 | ≥ 0.18V | PASS | LVC8T245 via R268 | U10-P15 |
| TR9_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=2.019V, Trough=-0.312V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | LVC8T245 via R268 | U10-P15 |

## 3.8.65    TE_EXT_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| TE_EXT_SYNC_P1V8 (Run A) | VOH (V) | 1.785 | ≥ 1.35V (VDD−0.45V) | PASS | U72-19 | U10-B29 |
| TE_EXT_SYNC_P1V8 (Run A) | VOL (V) | 0.015 | ≤ 0.45V | PASS | U72-19 | U10-B29 |
| TE_EXT_SYNC_P1V8 (Run A) | Rise Time (ns) | 2.57 | ≤ 10ns (JESD8-7A) | PASS | U72-19 | U10-B29 |
| TE_EXT_SYNC_P1V8 (Run A) | Fall Time (ns) | 1.96 | ≤ 10ns (JESD8-7A) | PASS | U72-19 | U10-B29 |
| TE_EXT_SYNC_P1V8 (Run A) | Noise Margin High (V) | 0.615 | ≥ 0.18V | PASS | U72-19 | U10-B29 |
| TE_EXT_SYNC_P1V8 (Run A) | Noise Margin Low (V) | 0.615 | ≥ 0.18V | PASS | U72-19 | U10-B29 |
| TE_EXT_SYNC_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.936V, Trough=-0.205V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U72-19 | U10-B29 |
| TE_EXT_SYNC_P1V8 (Run B) | VOH (V) | 1.728 | ≥ 1.35V (VDD−0.45V) | PASS | U10-B29 | U72-19 |
| TE_EXT_SYNC_P1V8 (Run B) | VOL (V) | 0.072 | ≤ 0.45V | PASS | U10-B29 | U72-19 |
| TE_EXT_SYNC_P1V8 (Run B) | Rise Time (ns) | 5.47 | ≤ 10ns (JESD8-7A) | PASS | U10-B29 | U72-19 |
| TE_EXT_SYNC_P1V8 (Run B) | Fall Time (ns) | 5.45 | ≤ 10ns (JESD8-7A) | PASS | U10-B29 | U72-19 |
| TE_EXT_SYNC_P1V8 (Run B) | Noise Margin High (V) | 0.460 | ≥ 0.18V | PASS | U10-B29 | U72-19 |
| TE_EXT_SYNC_P1V8 (Run B) | Noise Margin Low (V) | 0.505 | ≥ 0.18V | PASS | U10-B29 | U72-19 |
| TE_EXT_SYNC_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.728V, Trough=0.072V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-B29 | U72-19 |

## 3.8.66    SYNC_EN_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| SYNC_EN_P1V8 | VOH (V) | 1.739 | ≥ 1.35V (VDD−0.45V) | PASS | U10-F15 | 8×U83 pins (14-21) |
| SYNC_EN_P1V8 | VOL (V) | 0.061 | ≤ 0.45V | PASS | U10-F15 | 8×U83 pins (14-21) |
| SYNC_EN_P1V8 | Rise Time (ns) | 8.43 | ≤ 10ns (JESD8-7A) | PASS | U10-F15 | 8×U83 pins (14-21) |
| SYNC_EN_P1V8 | Fall Time (ns) | 8.43 | ≤ 10ns (JESD8-7A) | PASS | U10-F15 | 8×U83 pins (14-21) |
| SYNC_EN_P1V8 | Noise Margin High (V) | 0.471 | ≥ 0.18V | PASS | U10-F15 | 8×U83 pins (14-21) |
| SYNC_EN_P1V8 | Noise Margin Low (V) | 0.516 | ≥ 0.18V | PASS | U10-F15 | 8×U83 pins (14-21) |
| SYNC_EN_P1V8 | Overshoot/Undershoot (V) | Peak=1.739V, Trough=0.061V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-F15 | 8×U83 pins (14-21) |

## 3.8.67    DIR_AB_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| DIR_AB_P1V8 | VOH (V) | 1.743 | ≥ 1.35V (VDD−0.45V) | PASS | U10-M14 | 8×U91 pins (14-21) |
| DIR_AB_P1V8 | VOL (V) | 0.057 | ≤ 0.45V | PASS | U10-M14 | 8×U91 pins (14-21) |
| DIR_AB_P1V8 | Rise Time (ns) | 8.00 | ≤ 10ns (JESD8-7A) | PASS | U10-M14 | 8×U91 pins (14-21) |
| DIR_AB_P1V8 | Fall Time (ns) | 8.00 | ≤ 10ns (JESD8-7A) | PASS | U10-M14 | 8×U91 pins (14-21) |
| DIR_AB_P1V8 | Noise Margin High (V) | 0.475 | ≥ 0.18V | PASS | U10-M14 | 8×U91 pins (14-21) |
| DIR_AB_P1V8 | Noise Margin Low (V) | 0.520 | ≥ 0.18V | PASS | U10-M14 | 8×U91 pins (14-21) |
| DIR_AB_P1V8 | Overshoot/Undershoot (V) | Peak=1.743V, Trough=0.057V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-M14 | 8×U91 pins (14-21) |

## 3.8.68    SRS0_UCD_ALERT_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| SRS0_UCD_ALERT_P1V8 | VOH (V) | 1.638 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AV33 | TXS0104 via R55 |
| SRS0_UCD_ALERT_P1V8 | VOL (V) | 0.162 | ≤ 0.45V | PASS | U10-AV33 | TXS0104 via R55 |
| SRS0_UCD_ALERT_P1V8 | Rise Time (ns) | 18.38 | ≤ 10ns (JESD8-7A) | FAIL | U10-AV33 | TXS0104 via R55 |
| SRS0_UCD_ALERT_P1V8 | Fall Time (ns) | 18.35 | ≤ 10ns (JESD8-7A) | FAIL | U10-AV33 | TXS0104 via R55 |
| SRS0_UCD_ALERT_P1V8 | Noise Margin High (V) | 0.468 | ≥ 0.18V | PASS | U10-AV33 | TXS0104 via R55 |
| SRS0_UCD_ALERT_P1V8 | Noise Margin Low (V) | 0.468 | ≥ 0.18V | PASS | U10-AV33 | TXS0104 via R55 |
| SRS0_UCD_ALERT_P1V8 | Overshoot/Undershoot (V) | Peak=1.638V, Trough=0.162V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AV33 | TXS0104 via R55 |

## 3.8.69    SRS0_UCD_CLK_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| SRS0_UCD_CLK_P1V8 | VOH (V) | 1.638 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AV31 | TXS0104 via R48 |
| SRS0_UCD_CLK_P1V8 | VOL (V) | 0.162 | ≤ 0.45V | PASS | U10-AV31 | TXS0104 via R48 |
| SRS0_UCD_CLK_P1V8 | Rise Time (ns) | 18.49 | ≤ 10ns (JESD8-7A) | FAIL | U10-AV31 | TXS0104 via R48 |
| SRS0_UCD_CLK_P1V8 | Fall Time (ns) | 18.46 | ≤ 10ns (JESD8-7A) | FAIL | U10-AV31 | TXS0104 via R48 |
| SRS0_UCD_CLK_P1V8 | Noise Margin High (V) | 0.468 | ≥ 0.18V | PASS | U10-AV31 | TXS0104 via R48 |
| SRS0_UCD_CLK_P1V8 | Noise Margin Low (V) | 0.468 | ≥ 0.18V | PASS | U10-AV31 | TXS0104 via R48 |
| SRS0_UCD_CLK_P1V8 | Overshoot/Undershoot (V) | Peak=1.638V, Trough=0.162V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AV31 | TXS0104 via R48 |

## 3.8.70    SRS0_UCD_DATA_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| SRS0_UCD_DATA_P1V8 | VOH (V) | 1.638 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AW33 | TXS0104 via R52 |
| SRS0_UCD_DATA_P1V8 | VOL (V) | 0.162 | ≤ 0.45V | PASS | U10-AW33 | TXS0104 via R52 |
| SRS0_UCD_DATA_P1V8 | Rise Time (ns) | 18.49 | ≤ 10ns (JESD8-7A) | FAIL | U10-AW33 | TXS0104 via R52 |
| SRS0_UCD_DATA_P1V8 | Fall Time (ns) | 18.46 | ≤ 10ns (JESD8-7A) | FAIL | U10-AW33 | TXS0104 via R52 |
| SRS0_UCD_DATA_P1V8 | Noise Margin High (V) | 0.468 | ≥ 0.18V | PASS | U10-AW33 | TXS0104 via R52 |
| SRS0_UCD_DATA_P1V8 | Noise Margin Low (V) | 0.468 | ≥ 0.18V | PASS | U10-AW33 | TXS0104 via R52 |
| SRS0_UCD_DATA_P1V8 | Overshoot/Undershoot (V) | Peak=1.638V, Trough=0.162V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AW33 | TXS0104 via R52 |

## 3.8.71    SRS1_UCD_ALERT_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| SRS1_UCD_ALERT_P1V8 | VOH (V) | 1.655 | ≥ 1.35V (VDD−0.45V) | PASS | U10-D34 | TXS0104 via R142 |
| SRS1_UCD_ALERT_P1V8 | VOL (V) | 0.145 | ≤ 0.45V | PASS | U10-D34 | TXS0104 via R142 |
| SRS1_UCD_ALERT_P1V8 | Rise Time (ns) | 15.09 | ≤ 10ns (JESD8-7A) | FAIL | U10-D34 | TXS0104 via R142 |
| SRS1_UCD_ALERT_P1V8 | Fall Time (ns) | 15.06 | ≤ 10ns (JESD8-7A) | FAIL | U10-D34 | TXS0104 via R142 |
| SRS1_UCD_ALERT_P1V8 | Noise Margin High (V) | 0.485 | ≥ 0.18V | PASS | U10-D34 | TXS0104 via R142 |
| SRS1_UCD_ALERT_P1V8 | Noise Margin Low (V) | 0.485 | ≥ 0.18V | PASS | U10-D34 | TXS0104 via R142 |
| SRS1_UCD_ALERT_P1V8 | Overshoot/Undershoot (V) | Peak=1.655V, Trough=0.145V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-D34 | TXS0104 via R142 |

## 3.8.72    SRS1_UCD_CLK_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| SRS1_UCD_CLK_P1V8 | VOH (V) | 1.664 | ≥ 1.35V (VDD−0.45V) | PASS | U10-J29 | TXS0104 via R144 |
| SRS1_UCD_CLK_P1V8 | VOL (V) | 0.136 | ≤ 0.45V | PASS | U10-J29 | TXS0104 via R144 |
| SRS1_UCD_CLK_P1V8 | Rise Time (ns) | 13.94 | ≤ 10ns (JESD8-7A) | FAIL | U10-J29 | TXS0104 via R144 |
| SRS1_UCD_CLK_P1V8 | Fall Time (ns) | 13.94 | ≤ 10ns (JESD8-7A) | FAIL | U10-J29 | TXS0104 via R144 |
| SRS1_UCD_CLK_P1V8 | Noise Margin High (V) | 0.494 | ≥ 0.18V | PASS | U10-J29 | TXS0104 via R144 |
| SRS1_UCD_CLK_P1V8 | Noise Margin Low (V) | 0.494 | ≥ 0.18V | PASS | U10-J29 | TXS0104 via R144 |
| SRS1_UCD_CLK_P1V8 | Overshoot/Undershoot (V) | Peak=1.664V, Trough=0.136V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-J29 | TXS0104 via R144 |

## 3.8.73    SRS1_UCD_DATA_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| SRS1_UCD_DATA_P1V8 | VOH (V) | 1.659 | ≥ 1.35V (VDD−0.45V) | PASS | U10-D35 | TXS0104 via R143 |
| SRS1_UCD_DATA_P1V8 | VOL (V) | 0.141 | ≤ 0.45V | PASS | U10-D35 | TXS0104 via R143 |
| SRS1_UCD_DATA_P1V8 | Rise Time (ns) | 14.81 | ≤ 10ns (JESD8-7A) | FAIL | U10-D35 | TXS0104 via R143 |
| SRS1_UCD_DATA_P1V8 | Fall Time (ns) | 14.79 | ≤ 10ns (JESD8-7A) | FAIL | U10-D35 | TXS0104 via R143 |
| SRS1_UCD_DATA_P1V8 | Noise Margin High (V) | 0.489 | ≥ 0.18V | PASS | U10-D35 | TXS0104 via R143 |
| SRS1_UCD_DATA_P1V8 | Noise Margin Low (V) | 0.489 | ≥ 0.18V | PASS | U10-D35 | TXS0104 via R143 |
| SRS1_UCD_DATA_P1V8 | Overshoot/Undershoot (V) | Peak=1.659V, Trough=0.141V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-D35 | TXS0104 via R143 |

## 3.8.74    SRS2_UCD_ALERT_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| SRS2_UCD_ALERT_P1V8 | VOH (V) | 1.751 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AU29 | TXS0104 via R392 |
| SRS2_UCD_ALERT_P1V8 | VOL (V) | 0.049 | ≤ 0.45V | PASS | U10-AU29 | TXS0104 via R392 |
| SRS2_UCD_ALERT_P1V8 | Rise Time (ns) | 3.83 | ≤ 10ns (JESD8-7A) | PASS | U10-AU29 | TXS0104 via R392 |
| SRS2_UCD_ALERT_P1V8 | Fall Time (ns) | 3.82 | ≤ 10ns (JESD8-7A) | PASS | U10-AU29 | TXS0104 via R392 |
| SRS2_UCD_ALERT_P1V8 | Noise Margin High (V) | 0.581 | ≥ 0.18V | PASS | U10-AU29 | TXS0104 via R392 |
| SRS2_UCD_ALERT_P1V8 | Noise Margin Low (V) | 0.581 | ≥ 0.18V | PASS | U10-AU29 | TXS0104 via R392 |
| SRS2_UCD_ALERT_P1V8 | Overshoot/Undershoot (V) | Peak=1.751V, Trough=0.049V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AU29 | TXS0104 via R392 |

## 3.8.75    SRS2_UCD_CLK_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| SRS2_UCD_CLK_P1V8 | VOH (V) | 1.753 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AU27 | TXS0104 via R398 |
| SRS2_UCD_CLK_P1V8 | VOL (V) | 0.047 | ≤ 0.45V | PASS | U10-AU27 | TXS0104 via R398 |
| SRS2_UCD_CLK_P1V8 | Rise Time (ns) | 3.49 | ≤ 10ns (JESD8-7A) | PASS | U10-AU27 | TXS0104 via R398 |
| SRS2_UCD_CLK_P1V8 | Fall Time (ns) | 3.49 | ≤ 10ns (JESD8-7A) | PASS | U10-AU27 | TXS0104 via R398 |
| SRS2_UCD_CLK_P1V8 | Noise Margin High (V) | 0.583 | ≥ 0.18V | PASS | U10-AU27 | TXS0104 via R398 |
| SRS2_UCD_CLK_P1V8 | Noise Margin Low (V) | 0.583 | ≥ 0.18V | PASS | U10-AU27 | TXS0104 via R398 |
| SRS2_UCD_CLK_P1V8 | Overshoot/Undershoot (V) | Peak=1.753V, Trough=0.047V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AU27 | TXS0104 via R398 |

## 3.8.76    SRS2_UCD_DATA_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| SRS2_UCD_DATA_P1V8 | VOH (V) | 1.750 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AU30 | TXS0104 via R397 |
| SRS2_UCD_DATA_P1V8 | VOL (V) | 0.050 | ≤ 0.45V | PASS | U10-AU30 | TXS0104 via R397 |
| SRS2_UCD_DATA_P1V8 | Rise Time (ns) | 3.94 | ≤ 10ns (JESD8-7A) | PASS | U10-AU30 | TXS0104 via R397 |
| SRS2_UCD_DATA_P1V8 | Fall Time (ns) | 3.94 | ≤ 10ns (JESD8-7A) | PASS | U10-AU30 | TXS0104 via R397 |
| SRS2_UCD_DATA_P1V8 | Noise Margin High (V) | 0.580 | ≥ 0.18V | PASS | U10-AU30 | TXS0104 via R397 |
| SRS2_UCD_DATA_P1V8 | Noise Margin Low (V) | 0.580 | ≥ 0.18V | PASS | U10-AU30 | TXS0104 via R397 |
| SRS2_UCD_DATA_P1V8 | Overshoot/Undershoot (V) | Peak=1.750V, Trough=0.050V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AU30 | TXS0104 via R397 |

## 3.8.77    SRS0_UCD_CNTRL_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| SRS0_UCD_CNTRL_P1V8 | VOH (V) | 1.639 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AY31 | U17-2 |
| SRS0_UCD_CNTRL_P1V8 | VOL (V) | 0.161 | ≤ 0.45V | PASS | U10-AY31 | U17-2 |
| SRS0_UCD_CNTRL_P1V8 | Rise Time (ns) | 17.97 | ≤ 10ns (JESD8-7A) | FAIL | U10-AY31 | U17-2 |
| SRS0_UCD_CNTRL_P1V8 | Fall Time (ns) | 17.93 | ≤ 10ns (JESD8-7A) | FAIL | U10-AY31 | U17-2 |
| SRS0_UCD_CNTRL_P1V8 | Noise Margin High (V) | 0.469 | ≥ 0.18V | PASS | U10-AY31 | U17-2 |
| SRS0_UCD_CNTRL_P1V8 | Noise Margin Low (V) | 0.469 | ≥ 0.18V | PASS | U10-AY31 | U17-2 |
| SRS0_UCD_CNTRL_P1V8 | Overshoot/Undershoot (V) | Peak=1.639V, Trough=0.161V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AY31 | U17-2 |

## 3.8.78    SRS1_UCD_CNTRL_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| SRS1_UCD_CNTRL_P1V8 | VOH (V) | 1.668 | ≥ 1.35V (VDD−0.45V) | PASS | U10-K31 | U43-2 |
| SRS1_UCD_CNTRL_P1V8 | VOL (V) | 0.132 | ≤ 0.45V | PASS | U10-K31 | U43-2 |
| SRS1_UCD_CNTRL_P1V8 | Rise Time (ns) | 13.41 | ≤ 10ns (JESD8-7A) | FAIL | U10-K31 | U43-2 |
| SRS1_UCD_CNTRL_P1V8 | Fall Time (ns) | 13.44 | ≤ 10ns (JESD8-7A) | FAIL | U10-K31 | U43-2 |
| SRS1_UCD_CNTRL_P1V8 | Noise Margin High (V) | 0.498 | ≥ 0.18V | PASS | U10-K31 | U43-2 |
| SRS1_UCD_CNTRL_P1V8 | Noise Margin Low (V) | 0.498 | ≥ 0.18V | PASS | U10-K31 | U43-2 |
| SRS1_UCD_CNTRL_P1V8 | Overshoot/Undershoot (V) | Peak=1.668V, Trough=0.132V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-K31 | U43-2 |

## 3.8.79    SRS2_UCD_CNTRL_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| SRS2_UCD_CNTRL_P1V8 | VOH (V) | 1.753 | ≥ 1.35V (VDD−0.45V) | PASS | U10-AV29 | U110-2 |
| SRS2_UCD_CNTRL_P1V8 | VOL (V) | 0.047 | ≤ 0.45V | PASS | U10-AV29 | U110-2 |
| SRS2_UCD_CNTRL_P1V8 | Rise Time (ns) | 3.66 | ≤ 10ns (JESD8-7A) | PASS | U10-AV29 | U110-2 |
| SRS2_UCD_CNTRL_P1V8 | Fall Time (ns) | 3.67 | ≤ 10ns (JESD8-7A) | PASS | U10-AV29 | U110-2 |
| SRS2_UCD_CNTRL_P1V8 | Noise Margin High (V) | 0.583 | ≥ 0.18V | PASS | U10-AV29 | U110-2 |
| SRS2_UCD_CNTRL_P1V8 | Noise Margin Low (V) | 0.583 | ≥ 0.18V | PASS | U10-AV29 | U110-2 |
| SRS2_UCD_CNTRL_P1V8 | Overshoot/Undershoot (V) | Peak=1.753V, Trough=0.047V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-AV29 | U110-2 |

## 3.8.80    SRS0_PLL_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| SRS0_PLL_SYNC_P1V8 | VOH (V) | 1.691 | ≥ 1.35V (VDD−0.45V) | PASS | U10-P28 | U67-3 |
| SRS0_PLL_SYNC_P1V8 | VOL (V) | 0.109 | ≤ 0.45V | PASS | U10-P28 | U67-3 |
| SRS0_PLL_SYNC_P1V8 | Rise Time (ns) | 8.66 | ≤ 10ns (JESD8-7A) | PASS | U10-P28 | U67-3 |
| SRS0_PLL_SYNC_P1V8 | Fall Time (ns) | 8.63 | ≤ 10ns (JESD8-7A) | PASS | U10-P28 | U67-3 |
| SRS0_PLL_SYNC_P1V8 | Noise Margin High (V) | 0.521 | ≥ 0.18V | PASS | U10-P28 | U67-3 |
| SRS0_PLL_SYNC_P1V8 | Noise Margin Low (V) | 0.521 | ≥ 0.18V | PASS | U10-P28 | U67-3 |
| SRS0_PLL_SYNC_P1V8 | Overshoot/Undershoot (V) | Peak=1.691V, Trough=0.109V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-P28 | U67-3 |

## 3.8.81    SRS1_PLL_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| SRS1_PLL_SYNC_P1V8 | VOH (V) | 1.714 | ≥ 1.35V (VDD−0.45V) | PASS | U10-A29 | U67-4 |
| SRS1_PLL_SYNC_P1V8 | VOL (V) | 0.086 | ≤ 0.45V | PASS | U10-A29 | U67-4 |
| SRS1_PLL_SYNC_P1V8 | Rise Time (ns) | 6.74 | ≤ 10ns (JESD8-7A) | PASS | U10-A29 | U67-4 |
| SRS1_PLL_SYNC_P1V8 | Fall Time (ns) | 6.70 | ≤ 10ns (JESD8-7A) | PASS | U10-A29 | U67-4 |
| SRS1_PLL_SYNC_P1V8 | Noise Margin High (V) | 0.544 | ≥ 0.18V | PASS | U10-A29 | U67-4 |
| SRS1_PLL_SYNC_P1V8 | Noise Margin Low (V) | 0.544 | ≥ 0.18V | PASS | U10-A29 | U67-4 |
| SRS1_PLL_SYNC_P1V8 | Overshoot/Undershoot (V) | Peak=1.714V, Trough=0.086V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-A29 | U67-4 |

## 3.8.82    SRS2_PLL_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| SRS2_PLL_SYNC_P1V8 | VOH (V) | 1.714 | ≥ 1.35V (VDD−0.45V) | PASS | U10-A30 | U67-5 |
| SRS2_PLL_SYNC_P1V8 | VOL (V) | 0.086 | ≤ 0.45V | PASS | U10-A30 | U67-5 |
| SRS2_PLL_SYNC_P1V8 | Rise Time (ns) | 6.74 | ≤ 10ns (JESD8-7A) | PASS | U10-A30 | U67-5 |
| SRS2_PLL_SYNC_P1V8 | Fall Time (ns) | 6.70 | ≤ 10ns (JESD8-7A) | PASS | U10-A30 | U67-5 |
| SRS2_PLL_SYNC_P1V8 | Noise Margin High (V) | 0.544 | ≥ 0.18V | PASS | U10-A30 | U67-5 |
| SRS2_PLL_SYNC_P1V8 | Noise Margin Low (V) | 0.544 | ≥ 0.18V | PASS | U10-A30 | U67-5 |
| SRS2_PLL_SYNC_P1V8 | Overshoot/Undershoot (V) | Peak=1.714V, Trough=0.086V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-A30 | U67-5 |

## 3.8.83    PRS_UCD_ALERT_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| PRS_UCD_ALERT_P1V8 | VOH (V) | 1.713 | ≥ 1.35V (VDD−0.45V) | PASS | U10-D19 | TXS0104 via R324 |
| PRS_UCD_ALERT_P1V8 | VOL (V) | 0.087 | ≤ 0.45V | PASS | U10-D19 | TXS0104 via R324 |
| PRS_UCD_ALERT_P1V8 | Rise Time (ns) | 6.63 | ≤ 10ns (JESD8-7A) | PASS | U10-D19 | TXS0104 via R324 |
| PRS_UCD_ALERT_P1V8 | Fall Time (ns) | 6.60 | ≤ 10ns (JESD8-7A) | PASS | U10-D19 | TXS0104 via R324 |
| PRS_UCD_ALERT_P1V8 | Noise Margin High (V) | 0.543 | ≥ 0.18V | PASS | U10-D19 | TXS0104 via R324 |
| PRS_UCD_ALERT_P1V8 | Noise Margin Low (V) | 0.543 | ≥ 0.18V | PASS | U10-D19 | TXS0104 via R324 |
| PRS_UCD_ALERT_P1V8 | Overshoot/Undershoot (V) | Peak=1.713V, Trough=0.087V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-D19 | TXS0104 via R324 |

## 3.8.84    PRS_UCD_CLK_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| PRS_UCD_CLK_P1V8 | VOH (V) | 1.713 | ≥ 1.35V (VDD−0.45V) | PASS | U10-F17 | TXS0104 via R322 |
| PRS_UCD_CLK_P1V8 | VOL (V) | 0.087 | ≤ 0.45V | PASS | U10-F17 | TXS0104 via R322 |
| PRS_UCD_CLK_P1V8 | Rise Time (ns) | 6.74 | ≤ 10ns (JESD8-7A) | PASS | U10-F17 | TXS0104 via R322 |
| PRS_UCD_CLK_P1V8 | Fall Time (ns) | 6.71 | ≤ 10ns (JESD8-7A) | PASS | U10-F17 | TXS0104 via R322 |
| PRS_UCD_CLK_P1V8 | Noise Margin High (V) | 0.543 | ≥ 0.18V | PASS | U10-F17 | TXS0104 via R322 |
| PRS_UCD_CLK_P1V8 | Noise Margin Low (V) | 0.543 | ≥ 0.18V | PASS | U10-F17 | TXS0104 via R322 |
| PRS_UCD_CLK_P1V8 | Overshoot/Undershoot (V) | Peak=1.713V, Trough=0.087V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-F17 | TXS0104 via R322 |

## 3.8.85    PRS_UCD_DATA_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| PRS_UCD_DATA_P1V8 | VOH (V) | 1.714 | ≥ 1.35V (VDD−0.45V) | PASS | U10-F18 | TXS0104 via R323 |
| PRS_UCD_DATA_P1V8 | VOL (V) | 0.086 | ≤ 0.45V | PASS | U10-F18 | TXS0104 via R323 |
| PRS_UCD_DATA_P1V8 | Rise Time (ns) | 6.74 | ≤ 10ns (JESD8-7A) | PASS | U10-F18 | TXS0104 via R323 |
| PRS_UCD_DATA_P1V8 | Fall Time (ns) | 6.70 | ≤ 10ns (JESD8-7A) | PASS | U10-F18 | TXS0104 via R323 |
| PRS_UCD_DATA_P1V8 | Noise Margin High (V) | 0.544 | ≥ 0.18V | PASS | U10-F18 | TXS0104 via R323 |
| PRS_UCD_DATA_P1V8 | Noise Margin Low (V) | 0.544 | ≥ 0.18V | PASS | U10-F18 | TXS0104 via R323 |
| PRS_UCD_DATA_P1V8 | Overshoot/Undershoot (V) | Peak=1.714V, Trough=0.086V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-F18 | TXS0104 via R323 |

## 3.8.86    PRS_UCD_CNTRL_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| PRS_UCD_CNTRL_P1V8 | VOH (V) | 1.719 | ≥ 1.35V (VDD−0.45V) | PASS | U10-D20 | U86-2 |
| PRS_UCD_CNTRL_P1V8 | VOL (V) | 0.081 | ≤ 0.45V | PASS | U10-D20 | U86-2 |
| PRS_UCD_CNTRL_P1V8 | Rise Time (ns) | 6.49 | ≤ 10ns (JESD8-7A) | PASS | U10-D20 | U86-2 |
| PRS_UCD_CNTRL_P1V8 | Fall Time (ns) | 6.46 | ≤ 10ns (JESD8-7A) | PASS | U10-D20 | U86-2 |
| PRS_UCD_CNTRL_P1V8 | Noise Margin High (V) | 0.549 | ≥ 0.18V | PASS | U10-D20 | U86-2 |
| PRS_UCD_CNTRL_P1V8 | Noise Margin Low (V) | 0.549 | ≥ 0.18V | PASS | U10-D20 | U86-2 |
| PRS_UCD_CNTRL_P1V8 | Overshoot/Undershoot (V) | Peak=1.719V, Trough=0.081V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-D20 | U86-2 |

## 3.8.87    PRS_SPI0_VX_CS_N_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| PRS_SPI0_VX_CS_N_P1V8 | VOH (V) | 1.773 | ≥ 1.35V (VDD−0.45V) | PASS | U10-C18 | Zynq via R354 |
| PRS_SPI0_VX_CS_N_P1V8 | VOL (V) | 0.027 | ≤ 0.45V | PASS | U10-C18 | Zynq via R354 |
| PRS_SPI0_VX_CS_N_P1V8 | Rise Time (ns) | 1.94 | ≤ 10ns (JESD8-7A) | PASS | U10-C18 | Zynq via R354 |
| PRS_SPI0_VX_CS_N_P1V8 | Fall Time (ns) | 1.90 | ≤ 10ns (JESD8-7A) | PASS | U10-C18 | Zynq via R354 |
| PRS_SPI0_VX_CS_N_P1V8 | Noise Margin High (V) | 0.603 | ≥ 0.18V | PASS | U10-C18 | Zynq via R354 |
| PRS_SPI0_VX_CS_N_P1V8 | Noise Margin Low (V) | 0.603 | ≥ 0.18V | PASS | U10-C18 | Zynq via R354 |
| PRS_SPI0_VX_CS_N_P1V8 | Overshoot/Undershoot (V) | Peak=1.773V, Trough=0.027V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-C18 | Zynq via R354 |

## 3.8.88    PRS_SPI0_PS_CS_N_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| PRS_SPI0_PS_CS_N_P1V8 | VOH (V) | 1.617 | ≥ 1.35V (VDD−0.45V) | PASS | U4-AC21 | FPGA via R353 |
| PRS_SPI0_PS_CS_N_P1V8 | VOL (V) | 0.183 | ≤ 0.45V | PASS | U4-AC21 | FPGA via R353 |
| PRS_SPI0_PS_CS_N_P1V8 | Rise Time (ns) | 0.28 | ≤ 10ns (JESD8-7A) | PASS | U4-AC21 | FPGA via R353 |
| PRS_SPI0_PS_CS_N_P1V8 | Fall Time (ns) | 0.24 | ≤ 10ns (JESD8-7A) | PASS | U4-AC21 | FPGA via R353 |
| PRS_SPI0_PS_CS_N_P1V8 | Noise Margin High (V) | 0.447 | ≥ 0.18V | PASS | U4-AC21 | FPGA via R353 |
| PRS_SPI0_PS_CS_N_P1V8 | Noise Margin Low (V) | 0.447 | ≥ 0.18V | PASS | U4-AC21 | FPGA via R353 |
| PRS_SPI0_PS_CS_N_P1V8 | Overshoot/Undershoot (V) | Peak=1.617V, Trough=0.183V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U4-AC21 | FPGA via R353 |

## 3.8.89    PRS_SPI0_SCLK_P1V8_VX

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| PRS_SPI0_SCLK_P1V8_VX | VOH (V) | 1.776 | ≥ 1.35V (VDD−0.45V) | PASS | U10-C21 | Zynq via R355 |
| PRS_SPI0_SCLK_P1V8_VX | VOL (V) | 0.024 | ≤ 0.45V | PASS | U10-C21 | Zynq via R355 |
| PRS_SPI0_SCLK_P1V8_VX | Rise Time (ns) | 1.86 | ≤ 10ns (JESD8-7A) | PASS | U10-C21 | Zynq via R355 |
| PRS_SPI0_SCLK_P1V8_VX | Fall Time (ns) | 1.84 | ≤ 10ns (JESD8-7A) | PASS | U10-C21 | Zynq via R355 |
| PRS_SPI0_SCLK_P1V8_VX | Noise Margin High (V) | 0.606 | ≥ 0.18V | PASS | U10-C21 | Zynq via R355 |
| PRS_SPI0_SCLK_P1V8_VX | Noise Margin Low (V) | 0.606 | ≥ 0.18V | PASS | U10-C21 | Zynq via R355 |
| PRS_SPI0_SCLK_P1V8_VX | Overshoot/Undershoot (V) | Peak=1.776V, Trough=0.024V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-C21 | Zynq via R355 |

## 3.8.90    PRS_SPI0_SCLK_P1V8_PS

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| PRS_SPI0_SCLK_P1V8_PS | VOH (V) | 1.647 | ≥ 1.35V (VDD−0.45V) | PASS | U4-AB20 | FPGA via R526 |
| PRS_SPI0_SCLK_P1V8_PS | VOL (V) | 0.153 | ≤ 0.45V | PASS | U4-AB20 | FPGA via R526 |
| PRS_SPI0_SCLK_P1V8_PS | Rise Time (ns) | 16.28 | ≤ 10ns (JESD8-7A) | FAIL | U4-AB20 | FPGA via R526 |
| PRS_SPI0_SCLK_P1V8_PS | Fall Time (ns) | 16.26 | ≤ 10ns (JESD8-7A) | FAIL | U4-AB20 | FPGA via R526 |
| PRS_SPI0_SCLK_P1V8_PS | Noise Margin High (V) | 0.477 | ≥ 0.18V | PASS | U4-AB20 | FPGA via R526 |
| PRS_SPI0_SCLK_P1V8_PS | Noise Margin Low (V) | 0.477 | ≥ 0.18V | PASS | U4-AB20 | FPGA via R526 |
| PRS_SPI0_SCLK_P1V8_PS | Overshoot/Undershoot (V) | Peak=1.647V, Trough=0.153V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U4-AB20 | FPGA via R526 |

## 3.8.91    PRS_SPI0_MOSI_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| PRS_SPI0_MOSI_P1V8 (Run A) | VOH (V) | 1.604 | ≥ 1.35V (VDD−0.45V) | PASS | U10-C19 | U4-AB18 |
| PRS_SPI0_MOSI_P1V8 (Run A) | VOL (V) | 0.196 | ≤ 0.45V | PASS | U10-C19 | U4-AB18 |
| PRS_SPI0_MOSI_P1V8 (Run A) | Rise Time (ns) | 0.28 | ≤ 10ns (JESD8-7A) | PASS | U10-C19 | U4-AB18 |
| PRS_SPI0_MOSI_P1V8 (Run A) | Fall Time (ns) | 0.24 | ≤ 10ns (JESD8-7A) | PASS | U10-C19 | U4-AB18 |
| PRS_SPI0_MOSI_P1V8 (Run A) | Noise Margin High (V) | 0.434 | ≥ 0.18V | PASS | U10-C19 | U4-AB18 |
| PRS_SPI0_MOSI_P1V8 (Run A) | Noise Margin Low (V) | 0.434 | ≥ 0.18V | PASS | U10-C19 | U4-AB18 |
| PRS_SPI0_MOSI_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.604V, Trough=0.196V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-C19 | U4-AB18 |
| PRS_SPI0_MOSI_P1V8 (Run B) | VOH (V) | 1.604 | ≥ 1.35V (VDD−0.45V) | PASS | U4-AB18 | U10-C19 |
| PRS_SPI0_MOSI_P1V8 (Run B) | VOL (V) | 0.196 | ≤ 0.45V | PASS | U4-AB18 | U10-C19 |
| PRS_SPI0_MOSI_P1V8 (Run B) | Rise Time (ns) | 0.28 | ≤ 10ns (JESD8-7A) | PASS | U4-AB18 | U10-C19 |
| PRS_SPI0_MOSI_P1V8 (Run B) | Fall Time (ns) | 0.24 | ≤ 10ns (JESD8-7A) | PASS | U4-AB18 | U10-C19 |
| PRS_SPI0_MOSI_P1V8 (Run B) | Noise Margin High (V) | 0.434 | ≥ 0.18V | PASS | U4-AB18 | U10-C19 |
| PRS_SPI0_MOSI_P1V8 (Run B) | Noise Margin Low (V) | 0.434 | ≥ 0.18V | PASS | U4-AB18 | U10-C19 |
| PRS_SPI0_MOSI_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.604V, Trough=0.196V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U4-AB18 | U10-C19 |

## 3.8.92    PRS_SPI0_MISO_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| PRS_SPI0_MISO_P1V8 (Run A) | VOH (V) | 1.750 | ≥ 1.35V (VDD−0.45V) | PASS | U81-16 | U10-B21 |
| PRS_SPI0_MISO_P1V8 (Run A) | VOL (V) | 0.050 | ≤ 0.45V | PASS | U81-16 | U10-B21 |
| PRS_SPI0_MISO_P1V8 (Run A) | Rise Time (ns) | 2.31 | ≤ 10ns (JESD8-7A) | PASS | U81-16 | U10-B21 |
| PRS_SPI0_MISO_P1V8 (Run A) | Fall Time (ns) | 1.90 | ≤ 10ns (JESD8-7A) | PASS | U81-16 | U10-B21 |
| PRS_SPI0_MISO_P1V8 (Run A) | Noise Margin High (V) | 0.580 | ≥ 0.18V | PASS | U81-16 | U10-B21 |
| PRS_SPI0_MISO_P1V8 (Run A) | Noise Margin Low (V) | 0.580 | ≥ 0.18V | PASS | U81-16 | U10-B21 |
| PRS_SPI0_MISO_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=2.277V, Trough=-0.477V | Peak ≤ 2.1V, Trough ≥ −0.3V | FAIL | U81-16 | U10-B21 |
| PRS_SPI0_MISO_P1V8 (Run B) | VOH (V) | 1.577 | ≥ 1.35V (VDD−0.45V) | PASS | U10-B21 | U81-16 |
| PRS_SPI0_MISO_P1V8 (Run B) | VOL (V) | 0.223 | ≤ 0.45V | PASS | U10-B21 | U81-16 |
| PRS_SPI0_MISO_P1V8 (Run B) | Rise Time (ns) | 0.28 | ≤ 10ns (JESD8-7A) | PASS | U10-B21 | U81-16 |
| PRS_SPI0_MISO_P1V8 (Run B) | Fall Time (ns) | 0.24 | ≤ 10ns (JESD8-7A) | PASS | U10-B21 | U81-16 |
| PRS_SPI0_MISO_P1V8 (Run B) | Noise Margin High (V) | 0.309 | ≥ 0.18V | PASS | U10-B21 | U81-16 |
| PRS_SPI0_MISO_P1V8 (Run B) | Noise Margin Low (V) | 0.354 | ≥ 0.18V | PASS | U10-B21 | U81-16 |
| PRS_SPI0_MISO_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.577V, Trough=0.223V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-B21 | U81-16 |

## 3.8.93    PRS_PS_PCIE_RESET_N_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| PRS_PS_PCIE_RESET_N_P1V8 | VOH (V) | 1.682 | ≥ 1.35V (VDD−0.45V) | PASS | U4-G16 | FPGA via R306 |
| PRS_PS_PCIE_RESET_N_P1V8 | VOL (V) | 0.118 | ≤ 0.45V | PASS | U4-G16 | FPGA via R306 |
| PRS_PS_PCIE_RESET_N_P1V8 | Rise Time (ns) | 9.45 | ≤ 10ns (JESD8-7A) | PASS | U4-G16 | FPGA via R306 |
| PRS_PS_PCIE_RESET_N_P1V8 | Fall Time (ns) | 9.43 | ≤ 10ns (JESD8-7A) | PASS | U4-G16 | FPGA via R306 |
| PRS_PS_PCIE_RESET_N_P1V8 | Noise Margin High (V) | 0.512 | ≥ 0.18V | PASS | U4-G16 | FPGA via R306 |
| PRS_PS_PCIE_RESET_N_P1V8 | Noise Margin Low (V) | 0.512 | ≥ 0.18V | PASS | U4-G16 | FPGA via R306 |
| PRS_PS_PCIE_RESET_N_P1V8 | Overshoot/Undershoot (V) | Peak=1.682V, Trough=0.118V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U4-G16 | FPGA via R306 |

## 3.8.94    PRS_PLL_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| PRS_PLL_SYNC_P1V8 | VOH (V) | 1.691 | ≥ 1.35V (VDD−0.45V) | PASS | U10-R28 | U67-2 |
| PRS_PLL_SYNC_P1V8 | VOL (V) | 0.109 | ≤ 0.45V | PASS | U10-R28 | U67-2 |
| PRS_PLL_SYNC_P1V8 | Rise Time (ns) | 8.55 | ≤ 10ns (JESD8-7A) | PASS | U10-R28 | U67-2 |
| PRS_PLL_SYNC_P1V8 | Fall Time (ns) | 8.52 | ≤ 10ns (JESD8-7A) | PASS | U10-R28 | U67-2 |
| PRS_PLL_SYNC_P1V8 | Noise Margin High (V) | 0.521 | ≥ 0.18V | PASS | U10-R28 | U67-2 |
| PRS_PLL_SYNC_P1V8 | Noise Margin Low (V) | 0.521 | ≥ 0.18V | PASS | U10-R28 | U67-2 |
| PRS_PLL_SYNC_P1V8 | Overshoot/Undershoot (V) | Peak=1.691V, Trough=0.109V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-R28 | U67-2 |

## 3.8.95    LMK_STATUS_LD1_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| LMK_STATUS_LD1_P1V8 | VOH (V) | 1.775 | ≥ 1.35V (VDD−0.45V) | PASS | U10-D28 | LMK via R349 |
| LMK_STATUS_LD1_P1V8 | VOL (V) | 0.025 | ≤ 0.45V | PASS | U10-D28 | LMK via R349 |
| LMK_STATUS_LD1_P1V8 | Rise Time (ns) | 2.03 | ≤ 10ns (JESD8-7A) | PASS | U10-D28 | LMK via R349 |
| LMK_STATUS_LD1_P1V8 | Fall Time (ns) | 2.01 | ≤ 10ns (JESD8-7A) | PASS | U10-D28 | LMK via R349 |
| LMK_STATUS_LD1_P1V8 | Noise Margin High (V) | 0.507 | ≥ 0.18V | PASS | U10-D28 | LMK via R349 |
| LMK_STATUS_LD1_P1V8 | Noise Margin Low (V) | 0.552 | ≥ 0.18V | PASS | U10-D28 | LMK via R349 |
| LMK_STATUS_LD1_P1V8 | Overshoot/Undershoot (V) | Peak=1.775V, Trough=0.025V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-D28 | LMK via R349 |

## 3.8.96    LMK_STATUS_LD2_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| LMK_STATUS_LD2_P1V8 | VOH (V) | 1.775 | ≥ 1.35V (VDD−0.45V) | PASS | U10-E28 | LMK via R350 |
| LMK_STATUS_LD2_P1V8 | VOL (V) | 0.025 | ≤ 0.45V | PASS | U10-E28 | LMK via R350 |
| LMK_STATUS_LD2_P1V8 | Rise Time (ns) | 2.03 | ≤ 10ns (JESD8-7A) | PASS | U10-E28 | LMK via R350 |
| LMK_STATUS_LD2_P1V8 | Fall Time (ns) | 2.01 | ≤ 10ns (JESD8-7A) | PASS | U10-E28 | LMK via R350 |
| LMK_STATUS_LD2_P1V8 | Noise Margin High (V) | 0.508 | ≥ 0.18V | PASS | U10-E28 | LMK via R350 |
| LMK_STATUS_LD2_P1V8 | Noise Margin Low (V) | 0.553 | ≥ 0.18V | PASS | U10-E28 | LMK via R350 |
| LMK_STATUS_LD2_P1V8 | Overshoot/Undershoot (V) | Peak=1.775V, Trough=0.025V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-E28 | LMK via R350 |

## 3.8.97    LMK_RST_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| LMK_RST_P1V8 | VOH (V) | 1.787 | ≥ 1.35V (VDD−0.45V) | PASS | U81-21 | U10-G22 |
| LMK_RST_P1V8 | VOL (V) | 0.013 | ≤ 0.45V | PASS | U81-21 | U10-G22 |
| LMK_RST_P1V8 | Rise Time (ns) | 2.62 | ≤ 10ns (JESD8-7A) | PASS | U81-21 | U10-G22 |
| LMK_RST_P1V8 | Fall Time (ns) | 2.01 | ≤ 10ns (JESD8-7A) | PASS | U81-21 | U10-G22 |
| LMK_RST_P1V8 | Noise Margin High (V) | 0.617 | ≥ 0.18V | PASS | U81-21 | U10-G22 |
| LMK_RST_P1V8 | Noise Margin Low (V) | 0.617 | ≥ 0.18V | PASS | U81-21 | U10-G22 |
| LMK_RST_P1V8 | Overshoot/Undershoot (V) | Peak=1.943V, Trough=-0.178V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U81-21 | U10-G22 |

## 3.8.98    LMK_PLL_SYNC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| LMK_PLL_SYNC_P1V8 | VOH (V) | 1.785 | ≥ 1.35V (VDD−0.45V) | PASS | U72-14 | U10-C27 |
| LMK_PLL_SYNC_P1V8 | VOL (V) | 0.015 | ≤ 0.45V | PASS | U72-14 | U10-C27 |
| LMK_PLL_SYNC_P1V8 | Rise Time (ns) | 2.52 | ≤ 10ns (JESD8-7A) | PASS | U72-14 | U10-C27 |
| LMK_PLL_SYNC_P1V8 | Fall Time (ns) | 1.91 | ≤ 10ns (JESD8-7A) | PASS | U72-14 | U10-C27 |
| LMK_PLL_SYNC_P1V8 | Noise Margin High (V) | 0.615 | ≥ 0.18V | PASS | U72-14 | U10-C27 |
| LMK_PLL_SYNC_P1V8 | Noise Margin Low (V) | 0.615 | ≥ 0.18V | PASS | U72-14 | U10-C27 |
| LMK_PLL_SYNC_P1V8 | Overshoot/Undershoot (V) | Peak=1.953V, Trough=-0.232V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U72-14 | U10-C27 |

## 3.8.99    GE1_MDIO_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| GE1_MDIO_P1V8 (Run A) | VOH (V) | 1.755 | ≥ 1.35V (VDD−0.45V) | PASS | U874-41 | U78-5 |
| GE1_MDIO_P1V8 (Run A) | VOL (V) | 0.045 | ≤ 0.45V | PASS | U874-41 | U78-5 |
| GE1_MDIO_P1V8 (Run A) | Rise Time (ns) | 0.84 | ≤ 10ns (JESD8-7A) | PASS | U874-41 | U78-5 |
| GE1_MDIO_P1V8 (Run A) | Fall Time (ns) | 0.84 | ≤ 10ns (JESD8-7A) | PASS | U874-41 | U78-5 |
| GE1_MDIO_P1V8 (Run A) | Noise Margin High (V) | 0.585 | ≥ 0.18V | PASS | U874-41 | U78-5 |
| GE1_MDIO_P1V8 (Run A) | Noise Margin Low (V) | 0.585 | ≥ 0.18V | PASS | U874-41 | U78-5 |
| GE1_MDIO_P1V8 (Run A) | Overshoot/Undershoot (V) | Peak=1.755V, Trough=0.045V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U874-41 | U78-5 |
| GE1_MDIO_P1V8 (Run B) | VOH (V) | 1.755 | ≥ 1.35V (VDD−0.45V) | PASS | TXS0104 via R1713 | U874-41 |
| GE1_MDIO_P1V8 (Run B) | VOL (V) | 0.045 | ≤ 0.45V | PASS | TXS0104 via R1713 | U874-41 |
| GE1_MDIO_P1V8 (Run B) | Rise Time (ns) | 0.46 | ≤ 10ns (JESD8-7A) | PASS | TXS0104 via R1713 | U874-41 |
| GE1_MDIO_P1V8 (Run B) | Fall Time (ns) | 0.46 | ≤ 10ns (JESD8-7A) | PASS | TXS0104 via R1713 | U874-41 |
| GE1_MDIO_P1V8 (Run B) | Noise Margin High (V) | 0.551 | ≥ 0.18V | PASS | TXS0104 via R1713 | U874-41 |
| GE1_MDIO_P1V8 (Run B) | Noise Margin Low (V) | 0.299 | ≥ 0.18V | PASS | TXS0104 via R1713 | U874-41 |
| GE1_MDIO_P1V8 (Run B) | Overshoot/Undershoot (V) | Peak=1.755V, Trough=0.045V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | TXS0104 via R1713 | U874-41 |

## 3.8.100    GE1_MDC_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| GE1_MDC_P1V8 | VOH (V) | 1.758 | ≥ 1.35V (VDD−0.45V) | PASS | U874-42 | FPGA via R307 |
| GE1_MDC_P1V8 | VOL (V) | 0.042 | ≤ 0.45V | PASS | U874-42 | FPGA via R307 |
| GE1_MDC_P1V8 | Rise Time (ns) | 0.63 | ≤ 10ns (JESD8-7A) | PASS | U874-42 | FPGA via R307 |
| GE1_MDC_P1V8 | Fall Time (ns) | 0.63 | ≤ 10ns (JESD8-7A) | PASS | U874-42 | FPGA via R307 |
| GE1_MDC_P1V8 | Noise Margin High (V) | 0.588 | ≥ 0.18V | PASS | U874-42 | FPGA via R307 |
| GE1_MDC_P1V8 | Noise Margin Low (V) | 0.588 | ≥ 0.18V | PASS | U874-42 | FPGA via R307 |
| GE1_MDC_P1V8 | Overshoot/Undershoot (V) | Peak=1.758V, Trough=0.042V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U874-42 | FPGA via R307 |

## 3.8.101    FAN_EN_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| FAN_EN_P1V8 | VOH (V) | 1.723 | ≥ 1.35V (VDD−0.45V) | PASS | U10-C28 | U72-20 |
| FAN_EN_P1V8 | VOL (V) | 0.077 | ≤ 0.45V | PASS | U10-C28 | U72-20 |
| FAN_EN_P1V8 | Rise Time (ns) | 6.17 | ≤ 10ns (JESD8-7A) | PASS | U10-C28 | U72-20 |
| FAN_EN_P1V8 | Fall Time (ns) | 6.14 | ≤ 10ns (JESD8-7A) | PASS | U10-C28 | U72-20 |
| FAN_EN_P1V8 | Noise Margin High (V) | 0.456 | ≥ 0.18V | PASS | U10-C28 | U72-20 |
| FAN_EN_P1V8 | Noise Margin Low (V) | 0.501 | ≥ 0.18V | PASS | U10-C28 | U72-20 |
| FAN_EN_P1V8 | Overshoot/Undershoot (V) | Peak=1.723V, Trough=0.077V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-C28 | U72-20 |

## 3.8.102    RUN_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| RUN_P1V8 | VOH (V) | 1.799 | ≥ 1.35V (VDD−0.45V) | PASS | U73-1 | U75-2 |
| RUN_P1V8 | VOL (V) | 0.001 | ≤ 0.45V | PASS | U73-1 | U75-2 |
| RUN_P1V8 | Rise Time (ns) | 0.52 | ≤ 10ns (JESD8-7A) | PASS | U73-1 | U75-2 |
| RUN_P1V8 | Fall Time (ns) | 0.49 | ≤ 10ns (JESD8-7A) | PASS | U73-1 | U75-2 |
| RUN_P1V8 | Noise Margin High (V) | 0.629 | ≥ 0.18V | PASS | U73-1 | U75-2 |
| RUN_P1V8 | Noise Margin Low (V) | 0.629 | ≥ 0.18V | PASS | U73-1 | U75-2 |
| RUN_P1V8 | Overshoot/Undershoot (V) | Peak=1.799V, Trough=0.001V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U73-1 | U75-2 |

## 3.8.103    P48V_EN_P1V8

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| P48V_EN_P1V8 | VOH (V) | 1.727 | ≥ 1.35V (VDD−0.45V) | PASS | U10-C29 | U72-21 |
| P48V_EN_P1V8 | VOL (V) | 0.073 | ≤ 0.45V | PASS | U10-C29 | U72-21 |
| P48V_EN_P1V8 | Rise Time (ns) | 5.48 | ≤ 10ns (JESD8-7A) | PASS | U10-C29 | U72-21 |
| P48V_EN_P1V8 | Fall Time (ns) | 5.46 | ≤ 10ns (JESD8-7A) | PASS | U10-C29 | U72-21 |
| P48V_EN_P1V8 | Noise Margin High (V) | 0.460 | ≥ 0.18V | PASS | U10-C29 | U72-21 |
| P48V_EN_P1V8 | Noise Margin Low (V) | 0.505 | ≥ 0.18V | PASS | U10-C29 | U72-21 |
| P48V_EN_P1V8 | Overshoot/Undershoot (V) | Peak=1.727V, Trough=0.073V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | U10-C29 | U72-21 |

## 3.8.104    P3V3_P1V8_MGTV_RUN

| Signal | Parameter | Measured Value | Specification | Pass/Fail | Driver Pin | Receiver Pin |
| --- | --- | --- | --- | --- | --- | --- |
| P3V3_P1V8_MGTV_RUN | VOH (V) | 1.668 | ≥ 1.35V (VDD−0.45V) | PASS | TPS3808/ext | 4×LTC7151+TPS3808 |
| P3V3_P1V8_MGTV_RUN | VOL (V) | 0.132 | ≤ 0.45V | PASS | TPS3808/ext | 4×LTC7151+TPS3808 |
| P3V3_P1V8_MGTV_RUN | Rise Time (ns) | 13.63 | ≤ 10ns (JESD8-7A) | FAIL | TPS3808/ext | 4×LTC7151+TPS3808 |
| P3V3_P1V8_MGTV_RUN | Fall Time (ns) | 13.68 | ≤ 10ns (JESD8-7A) | FAIL | TPS3808/ext | 4×LTC7151+TPS3808 |
| P3V3_P1V8_MGTV_RUN | Noise Margin High (V) | 0.498 | ≥ 0.18V | PASS | TPS3808/ext | 4×LTC7151+TPS3808 |
| P3V3_P1V8_MGTV_RUN | Noise Margin Low (V) | 0.498 | ≥ 0.18V | PASS | TPS3808/ext | 4×LTC7151+TPS3808 |
| P3V3_P1V8_MGTV_RUN | Overshoot/Undershoot (V) | Peak=1.668V, Trough=0.132V | Peak ≤ 2.1V, Trough ≥ −0.3V | PASS | TPS3808/ext | 4×LTC7151+TPS3808 |